3GPP TSG-RAN WG3 Meeting #107-bis-e	R3-201796

E-meeting, Apr 20th – Apr 30th, 2020

Agenda item:	15.2.3

Source:	Qualcomm Incorporated

Title:	(TP for NR_Mob_enh BL CR for TS 38.413) Handover Success procedure for NGAP based DAPS handover

WID/SID:	NR_Mob_enh-Core – Release 16

Document for:	Discussion and Decision

# Introduction

For HO Success:

Introduce two new class 2 procedures to support the HO Success procedure similar to the X2/Xn interface.

It was agreed in the previous RAN3 #107-e meeting [1] that a Handover Success procedure would be supported for S1/NG based DAPS handover. It was previously agreed that such a procedure would be supported for X2/Xn based DAPS handover. The agreement for S1/NG based DAPS handover is as follows:

In this contribution we discuss the signalling related to support of the Handover Success procedure over NG-AP.

# Handover Success procedure for NG-AP based DAPS handover

As in the case of Xn/X2 based DAPS handover, for NG-AP based DAPS handover, a Handover Success procedure can be introduced to transmit a Handover Success message from the target NG-RAN node to the source NG-RAN node in order to indicate to the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

Similar to Xn/X2 based DAPS handover, upon receiving the message, the source NG-RAN node shall initiate late data forwarding for the UE towards the target NG-RAN node, if configured.

Figure 1: Handover Success procedure over NG-AP

Proposal 1. The Handover Success procedure over NG-AP consists of the following two messages:

- Upon UE successfully accessing the target NG-RAN node, the HANDOVER SUCCESS INDICATION message is transmitted from target NG-RAN node to the AMF;

- Upon reception of the HANDOVER SUCCESS INDICATION message, the AMF initiates the transmission of the HANDOVER SUCCESS message to the source NG-RAN node.

Proposal 2. Upon receiving the HANDOVER SUCCESS message, the source NG-RAN node shall initiate late data forwarding for the UE towards the target NG-RAN node, if configured.

Proposal 3. The HANDOVER SUCCESS INDICATION and HANDOVER SUCCESS messages contain the target cell ID.

The target cell ID information is used by the source NG-RAN node to initiate late data forwarding towards the target NG-RAN node using the corresponding tunnel address information that was provided to it during the Handover Preparation procedure.

# Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following proposals:

Proposal 1. The Handover Success procedure over NG-AP consists of the following two messages:

- Upon UE successfully accessing the target NG-RAN node, the HANDOVER SUCCESS INDICATION message is transmitted from target NG-RAN node to the AMF;

- Upon reception of the HANDOVER SUCCESS INDICATION message, the AMF initiates the transmission of the HANDOVER SUCCESS message to the source NG-RAN node.

Proposal 2. Upon receiving the HANDOVER SUCCESS message, the source NG-RAN node shall initiate late data forwarding for the UE towards the target NG-RAN node, if configured.

Proposal 3. The HANDOVER SUCCESS INDICATION and HANDOVER SUCCESS messages contain the target cell ID.

In the annex below, we highlight the changes required in the NG-AP specification, TS 38.423.

# Annex: TP for draftCR for TS 38.413

### ------------------------------ Change #1, Section 8.4.x, TS 38.413 --------------------------------------------------------------

### 8.4.x	Handover Success

#### 8.4.x.1	General

The Handover Success procedure is used during a DAPS Handover, to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: Handover Success, successful operation

The AMF initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

The HANDOVER SUCCESS INDICATON message is used to inform the AMF that the UE has successfully attached to the target node.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

------------------------------ End of change #1, Section 8.4.x, TS 38.413 --------------------------------------------------------

------------------------------ Change #2, Section 9.2.3, TS 38.413 ----------------------------------------------------------------

#### 9.2.3.y	HANDOVER SUCCESS

This message is sent by the AMF to the source NG-RAN node, to indicate the successful access of the UE toward the target NG-RAN node.

Direction: AMF NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | YES | reject |

#### 9.2.3.z	HANDOVER SUCCESS INDICATION

This message is sent by the target NG-RAN node to the AMF, to indicate the successful access of the UE toward the target NG-RAN node.

Direction: NG-RAN node AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | YES | reject |

------------------------------ End of change #2, Section 9.2.3, TS 38.413 --------------------------------------------------------

# References

[1] Chairman’s notes, RAN3 #107-e.