3GPP TSG-RAN3 Meeting #107bis-e	R3-201933

20 – 30 April 2020 E-Meeting

Title: 	(TP for SON BL CR for TS 38.423) Addition of RAReport and ConnEstFailReport transfer

Source: 	CATT

Agenda item:	10.2.1.1

Document Type:	other

# 1. Introduction

This contribution is the TP for XNAP BL CR R3-201614 with Addition of inter-node transfer of RAReport and ConnEstFailReport.

# 2. Discussion

Addition of RAReport and ConnEstFailReport in FAILURE INDICATION message.

# 3. Annex – TP

Start of the first change

8.4.x	Failure Indication

8.4.x.1	General

The purpose of the Failure Indication procedure is to transfer information regarding RRC re-establishment attempts, or received RLF Reports, RA Report，ConnEstFail Report between NG-RAN nodes. The signalling takes place from the NG-RAN node at which a re-establishment attempt is made, or an RLF Report, RA Report，ConnEstFail Report is received, to an NG-RAN node to which the UE concerned may have previously been attached prior to the connection failure. This may aid the detection of radio link failure, handover failure cases.

The procedure uses non UE-associated signalling.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: Failure Indication, successful operation

NG-RAN node2 initiates the procedure by sending the FAILURE INDICATION message to NG-RAN node1, following a re-establishment attempt or an RLF Report, RA Report，ConnEstFail Report reception from a UE at NG-RAN node2, when NG-RAN node2 considers that the UE may have previously suffered a connection failure at a cell controlled by NG-RAN node1.

If the UE RLF Report Container IE，UE RA Report Container IE，UE ConnEstFail Report Container  IE is included in the FAILURE INDICATION message, NG-RAN node1 shall use it to derive failure case information.

8.4.x.3	Unsuccessful Operation

Not applicable.

8.4.x.4	Abnormal Conditions

Void.

9.1.3.x	FAILURE INDICATION

This message is sent by the NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node 1.

FFS: To include UE RA Report Container IE, the name of FAILURE INDICATION may be changed.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.X |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  |  |  |
| > RRC Reestab |  |  |  |  |  |  |
| >>Failure cell PCI | C- ifUERLFReportContainerAbsent |  | 9.2.2.10 | Physical Cell Identifier | YES | ignore |
| >>Re-establishment cell CGI | C- ifUERLFReportContainerAbsent |  | NG-RAN CGI 9.2.2.x |  | YES | ignore |
| >>C-RNTI | C- ifUERLFReportContainerAbsent |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRCRe-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>ShortMAC-I | C- ifUERLFReportContainerAbsent |  | BIT STRING (SIZE (16)) | ShortMAC-I contained in the RRC Re-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>UE RLF Report Container | O |  | 9.2.2.z | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10] or TS 36.331 [14]) | YES | ignore |
| > RRC Setup |  |  |  |  |  |  |
| >>UE RLF Report Container | O |  | 9.2.2.z | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10] or TS 36.331 [14]) | YES | ignore |
| UE RA Report Container | O |  | OCTET STRING | RA-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [18]. | YES | ignore |
| UE ConnEstFail Report Container | O |  | OCTET STRING | ConnEstFailReport-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [18]. | YES | ignore |

9.3.5	Information Element definitions

#### -- I

InitiatingCondition-FailureIndication ::= CHOICE {

rRCReestab					RRCReestab-initiated,

rRCSetup					RRCSetup-initiated,

uERAReportContainer        OCTET STRING			OPTIONAL,

uECEFReportContainer       OCTET STRING			OPTIONAL,

choice-extension				ProtocolIE-Single-Container { {InitiatingCondition-FailureIndication-ExtIEs} }

}

End of the last change