3GPP TSG-RAN3 Meeting #107-bis-e	R3-202053

E-meeting, 20 -30 April 2020

Title: 	Discussion on spatial relationship of DL and UL resources

Source: 	Huawei

Agenda item:	19.2.2

Document for:	Discussion

# 1. Introduction

Last RAN3 meeting has discussed the NRPPa procedure and open issues in the email discussion are summarized in [1]. There is another email discussion about the exchange of DL PRS configuration over Xn [2].

This paper discusses the issues of exchanging DL PRS configuration.

# 2. Discussion

In [2], it is proposed that:

“For Rel-15, it can be noted that gNB is already identifying the spatial relationships. The spatial relationship is alignment between UL and DL Resources. gNB may have more information than LMF about neighbour NG-RAN nodes to perform such task. Thus, gNB is best suited to determine the spatial relations.

The spatial relation also depends upon NRPPa/Xn exchanges. Thus, RAN3 should discuss and decide on those aspects.”

It can be seen that PRS configurations of the neighbouring cells are proposed to be used for the serving gNB to decide the spatial relationship and QCLs when configuring UE sensing UL SRS. There are at least two options regarding this issue:

- Exchange the PRS configurations over Xn.

- LMF can send the PRS beam information when requesting SRS configurations from serving gNB

Since the LMF would collect PRS anyway from multiple TRP/gNBs, it seems straightforward for the LMF to send the PRS configurations to the serving gNB. The current procedures would fully support LMF based information exchange. The LMF can send the PRS information to the gNB and gNB decide the spatial relationships and QCLs. Exchanging PRS over Xn looks having no extra benefits but incurring extra work.

Observation 1: The current positioning procedures can easily support PRS configuration based on LMF, while exchanging PRS over Xn looks having no extra benefits but incurring extra work.

It looks RAN2 has been discussing this issue already, and based on the latest status from RAN2, LMF would recommend the spatial relation and pathloss reference for SRS in the message of requesting SRS configuration from the serving gNB, i.e., POSITIONING INFORMATION REQUEST.

The information to be carried in POSITIONING INFORMATION REQUEST message currently contains the following IEs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.x |  | YES | ignore |

We believe that the PRS or SSB information to assist the serving gNB to configure SRS should be included in the Request SRS Transmission Characteristics.

To be more specific, the IE Request SRS Transmission Characteristics should contain the following options for each SRS as an enabler of RRC configuration between serving gNB and the UE of the SRS spatial relation and pathloss reference.

DL-PRS-Info-r16  ::=     SEQUENCE {

trp-Id-r16						 	INTEGER (0..255),

dl-PRS-ResourceSetId-r16 		 	INTEGER (0..7),

dl-PRS-ResourceId-r16 		 	 	INTEGER (0..63) 		OPTIONAL		–- Cond Pathloss

}

- PRS related information

  - TRP-ID: A local TRP ID as defined in LPP, not the same as the TRP ID defined in RAN3.

  - PRS resource set ID

  - PRS resource ID

- Note: No detailed PRS configuration is needed as UE would anyway know the PRS configuration via LPPSSB related information

  - Needs to be addressed after RAN2 finalizes the design.

Proposal 1: LMF should send the PRS ID related information as a recommendation to the serving gNB to assist SRS configuration.

# 3. Conclusion

Base on the discussion above, the following proposal is proposed.

Observation 1: The current positioning procedures can easily support PRS configuration based on LMF, while exchanging PRS over Xn looks having no extra benefits but incurring extra work.

Proposal 1: LMF should send the PRS ID related information as a recommendation to the serving gNB to assist SRS configuration.

# 4. Reference

- R3-201188 Email057-Pos_NRPPa_ext: Summary of offline.

- R3-201189 Summary of offline discussion on exchange of DL PRS configuration over Xn.