3GPP TSG-RAN3 Meeting #107-bis-e	R3-202056

E-meeting, 20 -30 April 2020

Title: 	(TP for BL CR for TS 38.470): F1 support for NR positioning

Source: 	Huawei

Agenda item:	19.5

Document Type:	other

# 1. Introduction

The RAN3#106 meeting discussed the support of NR positioning over F1AP, and agreed the BL CR in [1] with the following editor’s note.

- Editor’s Note: to be updated when F1 AP will be agreed

In this document, the TP will be provided for the BL CR update.

# 2. Discussion

- Based on our companion paper discussing the F1AP issues [2], the TP for 38.470 has been provided.

- Proposal 1: Agree the TP.

# 3. Conclusion

Based on the discussion in this paper, we propose:

Proposal 1: Agree the TP.

# 4. Reference

- R3-200062 Positioning support over F1AP

- R3-xxxxxx (TP for BL CR for 38.473) F1 support for positioning

# Annex A – TP for TS 38.470

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 5.2.X	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU.  With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows to transfer the positioning assistance data from gNB-CU to gNB-DU. gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

Editor’s Note: the above text needs futher check and revision

<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>

### 6.1.x	Positioning procedures

The F1 Positioning procedures are listed below:

- Positioning Measurement procedure

- TRP information exchange procedure

- UL Positioning Information Exchange procedure

- TRP Information update procedure

- Assistance Information Control procedure

- Assistance Information Feedback procedure

- Positioning Measurement Report procedure

- Positioning Measurement Modification procedure

- Positioning Measurement Termination procedure

- Positioning Measurement Failure Indication Procedure

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>