3GPP TSG-RAN3 Meeting #107-bis-e	R3-202057

E-meeting, 20 -30 April 2020

Title: 	(TP for BL CR for TS 38.473): F1 support for NR positioning

Source: 	Huawei

Agenda item:	19.5

Document Type:	other

# 1. Introduction

Last RAN3#106 meeting discussed the support of NR positioning over F1AP, and agreed the BL CR in [1] with the following editor’s notes.

- Editor’s Note: whether the SRS Transmission request is sent via this procedure, or via a positioning specific procedure, is FFS.

- Editor’s Note: It is FFS whether a single procedure is used for UE associated measurements and non-UE associated information exchange, or whether these are separated into two procedures. As currently the procedure describes UE measurements, and is UE-associated,  if it stays as such, then it should be renamed to Positioning Measurement procedure.

In this document, the positioning support procedure at F1AP will be discussed.

# 2. Discussion

The NRPPa procedures now comprise Positioining information exchange procedure, measurement procedure, TRP information exchange procedure.

- The postioning inforamtion exchange procedure is UE-associated and used for indicating RAN node to configure UE transmiting SRS and retrieve SRS configurations.

- The measurement procedure is used for LMF to request measurement from RAN nodes and is non-UE associated.

- The TRP information exchange procedure is used for LMF to retrieve the TRP configurations and the PRS configurations.

In F1AP, the smiliar procedures should be defined.

Proposal 1: Define similar procedures to NRPPa including SRS related Positioning information exchange procedure, Measurement procedure, TRP information exchange procedure.

2.1 SRS transmission request – UL positioning related procedure

In the UTDOA positioning method in LTE, the UTDOA information exchange procedure is applied to:

- Indicate to the eNB the need to configure the UE to transmit periodic SRS signals

- Retrieve the SRS configuration from the eNB.

In NR, the procedures related to UL positioning methods including UL-TODA, UL-AOA, Multi-RTT also need to realize the two function mentioned above: request to configure UE sending SRS and retrieve SRS configurations. In the current BL CR [1], a single IE “Requested SRS Transmission Characteristics” was included in the “UE CONTEXT MODIFICATION REQUEST” message. It can only realize the request for configuring UE sending SRS but cannot retrieve the SRS configurations for LMF from NG-RAN nodes.

Observation 1: The “Requested SRS Transmission Characteristics” IE included in the “UE CONTEXT MODIFICATION REQUEST” message can only realize the request of UE sending SRS but cannot retrieve SRS configurations for LMF from the serving NG-RAN.

LMF need a UE associated class 1 procedure to request the RAN node to configure UE sending SRS and retrieve SRS configurations. With the SRS configurations, LMF can then request UL measurements from multiple RAN nodes. On NRPPa, the positioning information exchange procedure has this function.

Proposal 2: Define UL positioning information exchange procedure to request gNB-DU to configure UE sensing SRS and retrieve SRS configurations.

2.2 positioning measurement procedure

In current BL CR, the positioning information exchange procedure is confusing becauseing the procedure is main used for measurement exchange. The name of postioning information exchange is used on NPRRa but is used for LMF to request RAN node to configure UE sensing SRS and retrieve SRS configurations. It is proposed to change the name to Postioning measuremnt procedure.

Observation 2: The positioning information exchange procedure is confusing in current BL CR as the procedure is mainly used for measurement exchange, while the same name is used for SRS retrieve on NRPPa.

Proposal 3: Change the name of Positioning Information Exchange procedure to Positioning Measurement procedure.

2.3 TRP/PRS information exchange

In current BL CR, there is no procedure to exchange TRP/PRS information. Similar to NRPPa, a new class 1 procedure is proposed to be defined.

Proposal 4: Define TRP information exchange procedure at F1AP to exchange TRP/PRS information.

Proposal 5: Agree the TP. ASN.1 could be provided in revision if the tabular is agreed.

# 3. Conclusion

Based on the discussion in this paper, we propose:

Proposal 1: Define similar procedures to NRPPa including SRS related Positioning information exchange procedure, Measurement procedure, TRP information exchange procedure.

Observation 1: The “Requested SRS Transmission Characteristics” IE included in the “UE CONTEXT MODIFICATION REQUEST” message can only realize the request of UE sending SRS but cannot retrieve SRS configurations for LMF from the serving NG-RAN.

Proposal 2: Define UL positioning information exchange procedure to request gNB-DU to configure UE sensing SRS and retrieve SRS configurations.

Observation 2: The positioning information exchange procedure is confusing in current BL CR as the procedure is mainly used for measurement exchange, while the same name is used for SRS retrieve on NRPPa.

Proposal 3: Change the name of Positioning Information Exchange procedure to Positioning Measurement procedure.

Proposal 4: Define TRP information exchange procedure at F1AP to exchange TRP/PRS information.

Proposal 5: Agree the TP. ASN.1 could be provided in revision if the tabular is agreed.

# 4. Reference

- R3-200061 was R3-197692 Positioning support over F1AP.

# Annex A – TP for TS 38.473

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

[Editor’s Notes: procedures and associated functions require further checking]

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| GNB-DU RESOURCE COORDINATION | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| Positioning Measurement Exchange | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| UL Positioning Information Exchange | UL POSITIONING INFORMATION REQUEST | UL POSITIONING INFORMATION RESPONSE | UL POSITIONING INFORMATION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Termination | POSITIONING MEASUREMENT TERMINATION COMMAND |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Modification | POSITIONING MEASUREMENT MODIFICATION |

>>>> NEXT CHANGE <<<<

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

>>> skip unchanged text <<<

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

>>>> NEXT CHANGE <<<<

### 8.x.3	Positioning Measurement Exchange

#### 8.x.3.1	General

The purpose of the Positioning Measurement procedure is to request the gNB-DU to perform and report positioning measurements for the purpose of UL-TDOA, UL-AOA and multi-RTT positioning..

#### 8.x.3.2	Successful Operation

Figure 8.x.3.2-1: Positioning Measurement procedure: successful operation

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT REQUEST message. If the requested positioning measurement has been successful, the gNB-DU shall reply with the POSITIONING MEASUREMENT RESPONSE message.

If the Measurement Quantities IE is included in the POSITIONING MEASUREMENT REQUEST message:

-	For each Report Characteristics IE set to "OnDemand", the gNB-DU shall return the corresponding measurement results in the Measurement Result List IE in the POSITIONING MEASUREMENT RESPONSE message, and the gNB-CU shall consider that this measurement has been terminated by the gNB-DU.

-	For each Report Characteristics IE set to "Periodic", the gNB-DU shall initiate the corresponding measurement, and it shall reply with the POSITIONING MEASURMENT RESPONSE message without including this measurement in the message. The gNB-DU shall then periodically initiate the Positioning Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

-	If the gNB-DU is not able to initiate one or more requested measurements, it may include the Measurement Failure List IE in the POSITIONING MEASUREMENT RESPONSE message with the corresponding LMF UE Measurement ID(s).

Editor’s note: further details are FFS pending RAN1 and RAN2 progress

#### 8.x.3.3	Unsuccessful Operation

Figure 8.x.3.3-1: Positioning Measurement procedure: unsuccessful operation

If the gNB-DU is unable to provide the requested positioning information, it shall respond with a POSITIONING MEASUREMENT FAILURE message.

#### 8.x.3.4	Abnormal Conditions

If the gNB-DU receives a POSITIONING MEASUREMENT REQUEST message containing an LMF UE Measurement ID corresponding to an ongoing positioning measurement, it shall consider the procedure as failed and initiate local error handling.

### 8.x.7	Positioning Measurement Modification [FFS]

#### 8.x.7.1	General

The purpose of the Positioning Measurement Modification procedure is to modify one or more periodic positioning measurements performed by the gNB-DU.

[Editor’s note: It is FFS if this procedure is required vs e.g. stop/start]

#### 8.x.7.2	Successful Operation

Figure 8.x.7.2-1: Positioning Measurement Modification procedure: successful operation

The gNB-CU initiates the procedure by generating a POSITIONING MEASUREMENT MODIFICATION message. Upon receiving the message, the gNB-DU shall overwrite the previously received measurement configuration for the corresponding measurements.

#### 8.x.7.3	Unsuccessful Operation

Not applicable.

#### 8.x.7.4	Abnormal Conditions

If the gNB-DU cannot identify the given positioning measurements, it shall regard the procedure as failed and initiate local error handling.

>>>> NEXT CHANGE <<<<

### 8.x.8	TRP Information Exchange

#### 8.x.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the gNB-CU to request the gNB-DU to provide detailed information for TRPs hosted by the gNB-DU.

#### 8.x.8.2	Successful Operation

Figure 8.x.8.2-1: TRP Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a TRP INFORMATION REQUEST message. The gNB-DU responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information. The NG-RAN node includes information for all TRPs hosted by the gNB-DU in the TRP INFORMATION RESPONSE message

#### 8.x.8.3	Unsuccessful Operation

Figure 8.x.8.3-1: TRP Information Exchange procedure, unsuccessful operation

If the gNB-DU cannot provide any of the requested information, the gNB-DU shall respond with a TRP INFORMATION FAILURE message.

### 8.x.9	UL Positioning Informaition Porcedure

#### 8.x.9.1	General

The Uplink Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the eNB.

#### 8.x.9.2	Successful Operation

Figure 8.x.9.2-1: Uplink Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a UPLINK POSITIONING INFORMATION REQUEST message to the gNB-DU. This message may contain the SRS configuration and number of SRS transmissions desired. If the gNB-CU requests a number of SRS transmissions, the gNB-DU may take this information into account when configuring SRS transmissions for the UE. The gNB-DU shall reply with the UPLINK POSITIONING INFORMATION RESPONSE message.

#### 8.x.9.3	Unsuccessful Operation

Figure 8.x.9.3-1: Uplink Positioning Information Exchange procedure, unsuccessful operation

If the gNB-DU is unable to configure any SRS transmissions for the UE, the gNB-DU shall respond with a UPLINK POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the eNB shall send a UPLINK POSITIONING INFORMATION FAILURE message with an appropriate cause value.

>>>> NEXT CHANGE <<<<

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the RRCConnectionReconfiguration message as defined in TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >> UL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >> RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | YES | reject |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

>>>> NEXT CHANGE <<<<

#### 9.2.x.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request positioning information.

Direction: gNB-CU  gNB-DU.

Editor’s note: message contents are FFS pending RAN1 and RAN2 progress; TRP-specific IEs can be added to the message. It is FFS if the Measurement Quantities IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Quantities |  | 0..1 |  |  | YES | reject |
| >Measurement Quantities Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |
| >>Measurement Type | M |  | ENUMERATED (Cell-ID, UL RTOA, UL AoA, gNB RX-TX,…) [FFS] |  |  | - |
| >>Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | - |
| >>Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) [FFS] |  | YES | - |
| >>UL SRS Measurement Configuration | C-ifULRTOA |  | [FFS] |  | YES | - |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

#### 9.2.x.4	POSITIONING MEASUREMENT RESPONSE

This message is sent by the gNB-DU to signal the requested positioning information.

Direction: gNB-DU  gNB-CU.

Editor’s note: message contents are FFS pending RAN1 and RAN2 progress; TRP-specific IEs can be added to the message. . It is FFS if the Measurement Result List IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Result List |  | 0..1 |  |  | YES | reject |
| >Measurement Result List Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | - | - |
| >>Measurement Result | M |  | 9.3.1.a |  | - | - |
| Measurement Failure List |  | 0..1 |  |  | YES | ignore |
| >Measurement Failure List Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |
| >>Cause | M |  | 9.3.1.2 |  | YES | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

#### 9.2.x.5	POSITIONING MEASUREMNET FAILURE [FFS]

This message is sent by the gNB-DU to indicate that the requested positioning information cannot be signaled.

Direction: gNB-DU  gNB-CU.

Editor’s note: Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measurements that can be reported with one message. Value is 64. |

NEXT CHANGE

#### 9.1.1.e	TRP INFORMATION REQUEST

This message is sent by a gNB-CU to request information for TRPs hosted by a gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >TRP Information Type Item | M |  | ENUMERATED (arfcn, FFS, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is FFS. |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.1.f	TRP INFORMATION RESPONSE

This message is sent by a gNB-DU to convey TRP information to a gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.3.1.23 |  | - |  |
| TRP Information List | M | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >TRP Information | M |  | 9.3.1.h |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 16384. |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.1.g	TRP INFORMATION FAILURE

This message is sent by a gNB-DU node to indicate that the requested TRP information cannot be provided to a gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | - |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

NEXT CHANGE

#### 9.2.x.9	UPLINK POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.d |  | YES | ignore |

#### 9.2.x.10	UPLINK POSITIONING INFORMATION RESPONSE

This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UL Configuration | M |  | 9.3.1.g |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.11	UPLINK POSITIONING INFORMATION FAILURE

This message is sent by the gNB-DU to indicate that no SRS transmissions could be configured for the UE for uplink positioning measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.12			UPLINK POSITIONING INFORMATION UPDATE

This message is sent by the gNB-DU to indicate that the SRS configuration for the UE, for one or more cells, has changed.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UL Configuration | O |  | 9.3.1.g |  | YES | ignore |

NEXT CHANGE

### 9.3.1.d	UL SRS Measurement Configuration

The purpose of the UL Measurement Configuration IE is to convey the UL measurement parameters to the gNB-DU.

Editor’s Note: Details are FFS pending RAN1 progress.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Search Window Parameters | O |  | FFS |  |
| Number of Transmissions | M |  | INTEGER (FFS) | The number of periodic SRS transmissions. The value of ‘0’ represents an infinite number of SRS transmissions. |
| SRS Configuration | M | <1.. maxServCell> |  | Configuration of SRS for corresponding serving cells. |
| >NR PCI | M |  | INTEGER (0.. 1007) |  |
| >UL NR-ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >UL-bandwidth | M |  | ENUMERATED (FFS) | Cell transmission bandwidth configuration in uplink |
| >srs-BandwidthConfig | M |  | ENUMERATED (FFS) | Cell-specific SRS bandwidth configuration |

| Range bound | Explanation |
| --- | --- |
| maxServCell | Maximum number of serving cells with SRS configuration. Value is FFS. |

### 9.3.1.g	UL Configuration

The purpose of the Uplink Configuration information element is to inform the gNB-CU of the uplink configuration parameters.

Editor’s Note: Details are FFS pending RAN1 progress.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Configuration | M | 1 ..<maxServCell> |  | Configuration of SRS for corresponding serving cells. |
| >NR PCI | M |  | INTEGER (0.. 1007) |  |
| >UL NR-ARFCN | M |  | INTEGER (0.. 3279165) | The uplink NR-UTRA carrier frequency |
| >Number of Transmissions | M |  | INTEGER (FFS) | The number of periodic SRS transmissions. The value of ‘0’ represents an infinite number of SRS transmissions. |
| >UL-bandwidth | M |  | ENUMERATED(FFS) | Cell transmission bandwidth configuration in uplink |
| >srs-BandwidthConfig | M |  | ENUMERATED (FFS) |  |

| Range bound | Explanation |
| --- | --- |
| maxServCell | Maximum number of serving cells with SRS configuration. Value is FFS. |

### 9.3.1.h	TRP Information

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | INTEGER(1..16384) |  |
| Cell Information | M |  | NG-RAN CGI 9.3.1.12 |  |
| PRS Configurations | O |  | 9.3.1.x |  |
| NG-RAN Access Point Position | M |  | 9.3.1.y | The configured estimated geographical position of the antenna of the cell/TRP. |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  |
| >CHOICE TRP Information Item | M |  |  |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is FFS. |

### 9.1.3.x	PRS Configurations

The PRS Configurations IE contains configurations of PRS associated with a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SFN Initialisation Time | M |  | BIT STRING (64) |  |

### 9.3.1.y	NG-RAN Access Point Position

The NG-RAN Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude "h" expressed in metres is derived from the "uncertainty code" k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER (0..100) | In percentage |