3GPP TSG-RAN3 Meeting #107-bis-e	R3-202247

E-Meeting, 20 - 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signalling UE capability Identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-RAN |  |  |  |  |  | Date: |  |  | 2020-4-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE Radio Capability ID IE and associated signalling are agreed to support RACS in S1AP. There is a need to include the new function and new procedure in the stage2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new UE Radio Capability ID Mapping Function and UE Radio Capability ID Mapping procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS new function and new procedure are not reflected in stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE BEGINS

### 19.2.1	S1 Interface Functions

#### 19.2.1.0	General

The S1 interface provides the following functions:

-	E-RAB Service Management function:

-	Setup, Modify, Release.

-	Mobility Functions for UEs in ECM-CONNECTED:

-	Intra-LTE Handover;

-	Inter-3GPP-RAT Handover.

-	S1 Paging function:

-	NAS Signalling Transport function;

-	LPPa Signalling Transport function;

-	S1-interface management functions:

-	Error indication;

-	Reset.

-	Network Sharing Function;

-	Roaming and Access Restriction Support function;

-	NAS Node Selection Function;

-	Initial Context Setup Function;

-	UE Context Modification Function;

-	UE Context Resume Function;

-	MME Load balancing Function;

-	Location Reporting Function;

-	PWS (which includes ETWS and CMAS) Message Transmission Function;

-	Overload function;

-	RAN Information Management Function;

-	Configuration Transfer Function;

-	S1 CDMA2000 Tunnelling function;

-	Trace function;

-	UE Radio Capability Match;

-	Retrieve UE Information Function;

-	UE Information Transfer Function;

-	Report of Secondary RAT data volumes Function;

-	UE Radio Capability ID Mapping Function.

NEXT CHANGE

#### 19.2.1.a	UE Radio Capability ID Mapping Function

The UE Radio Capability ID Mapping function supports the eNB to request the MME to provide the UE Radio Capability information that maps to a specific UE Radio Capability ID.

NEXT CHANGE

#### 19.2.2.b	UE Radio Capability ID Mapping procedure

The UE Radio Capability ID Mapping procedure is initiated by the eNB to request the UE Radio Capability information that maps to a specific capability ID from the MME. This procedure could be only initiated by the eNB which support the capability ID. Upon receiving a UE Radio Capability ID IE, if the UE radio capability information associated to this ID has not been obtained by the eNB, the UE Radio Capability ID Mapping procedure shall be initiated.

# Figure 19.2.2.b-1: UE radio capability ID mapping procedure

The UE Radio Capability ID Mapping procedure comprises the following steps:

-	The eNB initiates the UE Radio Capability ID Mapping procedure by sending UE RADIO CAPABILITY ID MAPPING REQUEST to the MME.

-	The MME responds with UE RADIO CAPABILITY ID MAPPING RESPONSE with including the UE radio capability information.

CHANGE ENDs