3GPP TSG-RAN3 Meeting #107bis-e	R3-202352

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.300): Support of RAN sharing with NPN

Source: 	Huawei

Agenda item:	16.8

Document for:	Discussion

Annex – TP for TS 38.300 (on the top of BL CR R3-201590)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 4.6	Radio Access Network Sharing

Each physical cell of the NG-RAN node can support one or multiple cell identities which constitute different logical cells.

In this version of the specification, a cell identity (i.e. a logical cell) can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]. The maximum supported number of PLMNs, PNI-NPNs and SNPNs in a physical cell is up to 12.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

Annex E:
NG-RAN Architecture for Radio Access Network Sharing with multiple cell ID broadcast (informative)

Each NG-RAN node serving a cell identified by a Cell Identity associated with a subset of PLMNs, or a subset of SNPNs, or a subset of PNI-NPNs is connected to another NG-RAN node via a single Xn-C interface instance.

Each Xn-C interface instance is setup and removed individually.

Xn-C interface instances terminating at NG-RAN nodes which share the same physical radio resources may share the same signalling transport resources. If this option is applied:

-	Non-UE associated signalling is associated to an Xn-C interface instance by including an Interface Instance Indication in the XnAP message;

-	Node related, non-UE associated Xn-C interface signalling may provide information destined for multiple logical nodes in a single XnAP procedure instance once the Xn-C interface instance is setup;

NOTE 1:	If the Interface Instance Indication corresponds to more than one interface instance, the respective XnAP message carries information destined for multiple logical nodes.

-	A UE associated signalling connection is associated to an Xn-C interface instance by allocating values for the corresponding NG-RAN node UE XnAP IDs so that they can be mapped to that Xn-C interface instance.

NOTE 2:	One possible implementation is to partition the value ranges of the NG-RAN node UE XnAP IDs and associate each value range with an Xn-C interface instance.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>