3GPP TSG-RAN WG3 Meeting #107bis-e 	R3-202431

20 - 30 April 2020, E-Meeting

Agenda item:	13.2.1.2.

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR-IAB BL CR for TS 38.473) Configure Donor-DU BAP Address in the Donor-DU

WID/SID:	NR_IAB-Core - Release 16

Document for:	Discussion and Decision

# Introduction

Last RAN3 meeting agreed “Donor-CU configures Donor-DU with the BAP address during the F1 Setup procedure between the Donor-DU and Donor-CU. FFS on whether include multiple BAP address. ” This contribution discuss the Stage-3 detail.

# Discussion

For the UL, the IAB node includes the Donor-DU’s BAP address as part of the Routing ID. When the Donor-DU receives the UL packet, the Donor-DU will check the BAP address of the Routing ID. It is enough to have one BAP address for a Donor-DU. There is no need to assign multiple BAP address to a Donor-DU.

Proposal 1: Add the Donor-DU BAP Address in the F1 SETUP RESPONSE message.

# Conclusion

In this contribution, we discussed the Stage-3 detail to configure the Donor-DU BAP address in the Donor-DU. Our proposals are:

Proposal 1: Add the Donor-DU BAP Address in the F1 SETUP RESPONSE message.

The proposed TP to TS38.473 BL CR can be found at the Annex section.

# References

[1]	R3-201621, BL CR to 38.473: Support for IAB

Proposed TP for TS38.473 BL CR:

-------------------------------------------Start of Change -------------------------------------------

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

If the F1 SETUP RESPONSE message contains the Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the F1 SETUP RESPONSE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the F1 SETUP REQUEST message is received from an IAB-donor-DU, the gNB-CU shall, if supported, include the BAP Address IE in the F1 SETUP RESPONSE message. If the F1 SETUP RESPONSE message contains the BAP Address IE, the IAB-donor-DU shall store the received BAP address and take it into account for IAB operation.

-------------------------------------------Next Change-------------------------------------------

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information associated to an F1-C interface instance.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.p | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| gNB-CU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.n | Provides the bearer mapping configuration information for the corresponding BH RLC channel, used for mapping of uplink non-UP traffic. | YES | reject |
| BAP Address | O |  | 9.3.1.v | This IE indicates the BAP address to be used by IAB-donor-DU. | YES | ignore |

-------------------------------------------Next Change-------------------------------------------

### 9.4.4	PDU Definitions

Unaffected parts skipped

-- **************************************************************

--

##### -- F1 Setup Response

--

-- **************************************************************

F1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupResponseIEs} },

...

}

F1SetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-Name						CRITICALITY ignore	TYPE GNB-CU-Name						PRESENCE optional	}|

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE Cells-to-be-Activated-List			PRESENCE optional	}|

{ ID id-GNB-CU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping	CRITICALITY reject	TYPE UL-BH-Non-UP-Traffic-Mapping	PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	},

...

}

-------------------------------------------End of changes ------------------------------------------