3GPP TSG-RAN3 Meeting #107bis-e	R3-202681

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.300): Paging for NPN

Source: 	Huawei

Agenda item:	16.5

Document for:	Discussion

Annex – TP for TS 38.300 (on the top of BL CR R3-201590)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 16.x	Stand-Alone NPN

<Unchanged Text Omitted>

### 16.x.x3	Paging

When receiving a paging message over an NG interface instance corresponding to an SNPN, the NG-RAN node may do some paging optimization by avoiding to page in cells which are not compatible with this SNPN.

For UEs in RRC_INACTIVE state, the NG-RAN node may page a neighbour NG-RAN node including the serving SNPN ID so that the neighbour NG-RAN node can do some paging optimization by avoiding to page in cells which are not compatible with the received SNPN ID.

Editor’s note: The section may be updated or even removed if no strong motivation is found at next phase.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>