3GPP TSG-RAN WG3 Meeting #107bis-e	R3-202844

# Online, April 20th – 30th 2020

Title:	RRC Message Design for IAB IP Address Allocation

Response to:	-

Release:	Rel-16

Study Item:	IAB_NR

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		 R3-202843

1. Overall Description:

RAN3 has further discussed the IP allocation for IAB-nodes and concluded that the RRC signalling for IAB IP address allocation should enable the following:

- For IAB-donor-based IP address allocation:

  - An IAB-node can request from the IAB-donor-CU via UL RRC message up to 8 full IPv4 addresses and/or and up to 8 full IPv6 addresses and/or one 64-bit IPv6 address prefix per specific usage.

  - Possible specific IP address usages are: F1-C traffic, F1-U traffic and non-F1 traffic.

  - The IAB-donor-CU indicates to the IAB-node via DL RRC message the full IPv4/IPv6 addresses and the IPv6 address prefixes and the usage of each allocated full IPv4/IPv6 address and IPv6 prefix.

- For OAM-based IP address allocation:

  - The IAB-node indicates to the IAB-donor-CU via UL RRC message the full IPv4/IPv6 addresses and IPv6 prefixes allocated by the OAM and their specific usage.

  - The same maximum number of allocated addresses/prefixes as for the IAB-donor-based IP address allocation applies.

  - Possible specific IP address usages are: F1-C traffic, F1-U traffic and non-F1 traffic.

  - The purpose of indicating the OAM-allocated IP addresses to the IAB-donor-CU is to enable the IAB-donor-CU to configure the IAB-donor-DU with the mapping between the IP addresses allocated to the IAB-node and the corresponding DL BAP Routing IDs.

- The IAB-node should be able to send the abovementioned UL RRC messages at any time after network integration.

  - The IAB-MT may need to first obtain OAM configuration (including the IP addresses) via PDU session or PDN connection.

  - For EN-DC, OAM connectivity may be obtained via LTE or via NR.

- IP addresses are updated via DL RRC signalling, where the updated address replaces the old one.

2. Actions:

To RAN2 group:

RAN3 respectfully asks RAN2 to include the above information into RRC messages for IAB IP address allocation.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#108	2–12 June 2020	Online