3GPP TSG-RAN3 Meeting #107-bis-e	R3-202729

E-meeting, 20 -30 April 2020

Title: 	(TP for WWC BL CR for TS 29.413) Support for interfacing wireline 5G access networks to the 5GC

Source: 	Huawei, BT, Broadcom, Telecom Italia

Agenda item:	21.2

Document for:	Other

Annex – TP

Change Begins

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

/**Skip the unrevelant**/

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	MobilityRestriction List IE

-	Index to RAT/Frequency Selection Priority IE

UPLINK NAS TRANSPORT message:

-	W-AGF Identity Information IE: the information given within this IE between the W-AGF and the AMF contains a list of identifiers of NG-U terminations at W-AGF as specified in TS 23.316 [x].

-	TNGF Identity Information IE: the information given within this IE between the TNGF and the AMF contains a list of identifiers of N3 terminations at TNGF as specified in TS 23.502 [4].

-	TWIF Identity Information IE: the information given within this IE between the TWIF and the AMF contains a list of identifiers of N3 terminations at TWIF as specified in TS 23.502 [4].

NG SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

/**Skip the unrevelant**/

End Change