3GPP TSG-RAN WG3 Meeting #107bis-e	R3-202758

# Online, April 20th – 30th 2020

Agenda Item:	13.1

Source:	Ericsson

Title:	(TP for NR-IAB BL draftCR for TS 38.300): Correction

Document for: 	Agreement

# Annex: TP for NR-IAB BL draftCR for TS 38.300

-------------------------------------------Change 1-------------------------------------------

### 4.x.y	Mapping of Uplink Traffic to Backhaul RLC Channels

The IAB-donor-CU configures the IAB-node with mappings between upstream F1- and non-F1-traffic originated at the IAB-node, and the appropriate BAP routing ID, next-hop BAP address and Backhaul RLC channel. A specific mapping is configured:

-  for each F1-U GTP-U tunnel,

-  for non-UE associated F1AP messages,

-  for UE-associated F1AP messages.

-  for non-F1 traffic.

Multiple mappings can contain the same Backhaul RLC channel and/or next-hop BAP address and/or BAP routing ID.

-------------------------------------------End of changes ------------------------------------------