3GPP TSG-RAN WG3 #108-e	R3-203012

# E-Meeting 1-11 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.410 | CR | 0020 | rev | 6 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of CP CIoT 5GS Optimisation for NB-IoT and MTC connected to 5GC (Stage 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core, LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following procedures need to be introduced to support CP CIoT 5GS Optimisation for NB-IoT and MTC connected to 5GC: - Connection Establishment Indication procedure, - AMF CP Relocation Indication procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following functions and procedures are added to Stage 2: - Connection Establishment Indication function and procedure, - AMF CP Relocation Indication function and procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CP CIoT 5GS optimisation for NB-IoT and MT connected to 5GC cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X, 5.Y, 6.X, 6.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4: rebased on 38.410 16.0.0 Rev5: rebased on 38.410 16.1.0 |  |  |  |  |  |  |  |  |

---------------- Start of the First Change ---------------

5	Functions of the NG interface

5.1	General

The following clauses describe the functions supported over the NG interface.

[Unchanged text skipped]

5.20	Report of Secondary RAT data volumes Function

The Report of Secondary RAT data volumes Function enables the NG-RAN node to report Secondary RAT usage data information in case of MR-DC, either with a dedicated procedure or by including Secondary RAT usage data information in other messages.

5.21	RIM Information Transfer function

The RIM Information Transfer function is a generic mechanism that allows the transfer of Remote Interference Management (RIM) information between two RAN nodes via the core network.

5.X	Connection Establishment Indication Function

The connection establishment indication function enables the AMF to complete the establishment of the UE-associated logical NG-connection.

5.Y	AMF CP Relocation Indication Function

The AMF CP relocation indication function enables the AMF to inform the previously serving NG-RAN that the UE's connection is to be relocated to a new NG-RAN.

---------------- Start of the First Change ---------------

-------------- Start of the Second Change --------------

6	Signalling procedures of the NG interface

6.1	PDU Session Management Procedures

The following list of PDU Session management procedures are used to establish, release or modify NG-RAN resources for a PDU session:

[Unchanged text skipped]

6.15	Secondary RAT Data Usage Report procedure

The following procedure is used to provide information on the used resources of the secondary RAT .

-	Secondary RAT Data Usage Report procedure.

6.16	RIM Information Transfer procedures

The following procedures are used by the AMF to transfer the Remote Interference Management (RIM) information.

-	Downlink RIM Information Transfer procedure;

-	Uplink RIM Information Transfer procedure.

6.X	Connection Establishment Indication procedure

The following connection establishment indication procedure is used to enable the AMF to provide information to the NG-RAN to complete the establishment of the UE-associated logical NG-connection and/or to trigger the NG-RAN to obtain and report UE Radio Capability, for UEs using CIoT 5GS Optimization.

-	Connection Establishment Indication.

6.Y	AMF CP Relocation Indication procedure

The following AMF CP relocation indication procedure is used to inform the previously serving NG-RAN that the UE's connection is to be relocated to a new NG-RAN, for UEs using Control Plane CIoT 5GS optimizations.

-	AMF CP Relocation Indication.

--------------- End of the Second Change --------------