3GPP TSG-RAN3 Meeting #108-e	R3-203026

E-meeting, 01 - 11 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0280 | rev | 6 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MDT feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of Minimizaion of Drive Test function on NGAP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the MDT configuration and user consent information in related procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Signaling based MDT in NG-RAN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.3.1, 8.4.2, 8.11.1, 8.11.4, 9.2.2.1, 9.2.3.4, 9.3.1.14, 9.3.1.xx1, 9.3.1.xx3, 9.3.1.xx4, 9.3.1.xx5, 9.3.1.x1, 9.3.1.x3, 9.3.1.x4, 9.3.1.x5, 9.3.1.x6, 9.3.1.x7, 9.3.1.x8, 9.3.1.x9, 9.3.1.x10, 9.3.1.x11, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR 0492 TS 38.423 CR 1405 TS 38.401 CR 0099 TS 38.463 CR 0477 TS 36.413 CR 1747 TS 36.423 CR 1440 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 5, R3-201613, update based on latest spec v16.1.0. Rev 4: R3-201478, merged TP from R3-201407. Rev 3: add ASN.1 on top of R3-197819; rebuild based on rel-16 spec. Rev 2: includes the agreed R3-197767. Rev 1: revision of R3-196271, resubmission. Rev 0: creation of the document as per agreements at RAN3 #105bis meeting. |  |  |  |  |  |  |  |  |

Start Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[11]	3GPP TS 32.422: "Trace control and configuration management".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC inactive state".

[13]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[14]	3GPP TS 38.414: "NG-RAN; NG data transport".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 38.331: "NG-RAN; Radio Resource Control (RRC) Protocol Specification".

[19]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[20]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[25]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[26]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[30]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[32]	3GPP TS 37.340: " Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[x]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

Next Change

## 8.3	UE Context Management Procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Index to RAT/Frequency Selection Priority IE, if available in the AMF.

If the NAS-PDU IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall pass it transparently towards the UE.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Radio Capability in the UE context;

-	store the received Index to RAT/Frequency Selection Priority in the UE context and use it as defined in TS 23.501 [9];

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context and, if the NG-RAN node is required to activate security for the UE, take this security key into use.

-	if supported, store the received SRVCC Operation Possible in the UE context and use it as defined in TS 23.216 [31].

-	store the received Management Based MDT PLMN List information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

If the PDU Session Resource Setup Request List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The NG-RAN node shall report to the AMF in the INITIAL CONTEXT SETUP RESPONSE message the result for each PDU session resource requested to be setup as defined in the PDU Session Resource Setup procedure.

Upon reception of the INITIAL CONTEXT SETUP RESPONSE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Response Transfer IE or PDU Session Resource Setup Unsuccessful Transfer IE to the SMF associated with the concerned PDU session. In case the splitting PDU session is not used by the NG-RAN node, the SMF should remove the Additional Transport Layer Information, if any.

The NG-RAN node shall use the information in the Mobility Restriction List IE if present in the INITIAL CONTEXT SETUP REQUEST message to

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation;

-	assign proper RNA(s) for the UE when moving the UE to RRC_INACTIVE state.

If the Mobility Restriction List IE is not contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the QoS flows includes a particular ARP value (TS 23.501 [9]).

If the Trace Activation IE is included in the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN node shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, set to “Immediate MDT Only”, “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN shall ignore Interfaces To Trace IE, and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the MDT Configuration IE and if the NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

If the UE Security Capabilities IE included in the INITIAL CONTEXT SETUP REQUEST message only contains the EIA0 or NIA0 algorithm as defined in TS 33.501 [13] and if the EIA0 or NIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.501 [13]), the NG-RAN node shall take it into use and ignore the keys received in the Security Key IE.

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for e.g. the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

If the CN Assisted RAN Parameters Tuning IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node may use it as described in TS 23.501 [9].

If the RRC Inactive Transition Report Request IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context.

If the Emergency Fallback Indicator IE is included in the INITIAL CONTEXT SETUP REQUEST message, it indicates that the UE context to be set up is subject to emergency service fallback as described in TS 23.501 [9] and the NG-RAN node may, if supported, take the appropriate mobility actions.

If the Old AMF IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall consider that this UE-associated logical NG-connection was redirected to this AMF from another AMF identified by the Old AMF IE.

If the Redirection for Voice EPS Fallback IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store it and use it in a subsequent decision of EPS fallback for voice as specified in TS 23.502 [10].

If the Location Reporting Request Type IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node should perform the requested location reporting functionality for the UE as described in subclause 8.12.

If the Management Based MDT PLMN List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN shall use it, if supported, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11].

Interactions with Initial UE Message procedure:

The NG-RAN node shall use the AMF UE NGAP ID IE and RAN UE NGAP ID IE received in the INITIAL CONTEXT SETUP REQUEST message as identification of the logical connection even if the RAN UE NGAP ID IE had been allocated in an INITIAL UE MESSAGE message sent over a different NG interface instance.

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the INITIAL CONTEXT SETUP REQUEST message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send the RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

Next Change

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration and associated security;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Context in the UE context and take it into use as defined in TS 33.501 [13].

Upon reception of the UE History Information IE, which is included within the Source to Target Transparent Container IE of the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The target NG-RAN node shall report to the AMF in the HANDOVER REQUEST ACKNOWLEDGE message the result for each PDU session resource requested to be setup. In particular, for each PDU session resource successfully setup, it shall include the Handover Request Acknowledge Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flow Setup Response List IE.

-	The Data Forwarding Accepted IE if the data forwarding for the QoS flow is accepted.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flow Failed to Setup List IE.

-	The UP transport layer information to be used for the PDU session.

-	The security result associated to the PDU session.

For each PDU session resource which failed to be setup, the Handover Resource Allocation Unsuccessful Transfer IE shall be included in the HANDOVER REQUEST ACKNOWLEDGE message containing a cause value that should be precise enough to enable the SMF to know the reason for the unsuccessful establishment.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the Handover Request Acknowledge Transfer IE or Handover Resource Allocation Unsuccessful Transfer IE to the SMF associated with the concerned PDU session.

If the HANDOVER REQUEST message contains the Data Forwarding Not Possible IE associated with a given PDU session within the Handover Request Transfer IE set to "data forwarding not possible", the target NG-RAN node may not include the DL Forwarding UP TNL Information IE and for intra-system handover the Data Forwarding Response DRB List IE within the Handover Request Acknowledge Transfer IE in the HANDOVER REQUEST ACKNOWLEDGE message for that PDU session.

In case of intra-system handover, if the target NG-RAN node accepts the downlink data forwarding for at least one QoS flow for which the DL Forwarding IE is set to "DL forwarding proposed", it may include the DL Forwarding UP TNL Information IE in the Handover Request Acknowledge Transfer IE as forwarding tunnel for the QoS flows listed in the QoS Flow Setup Response List IE of the HANDOVER REQUEST ACKNOWLEDGE message.

In case of intra-system handover, if the target NG-RAN node accepts the uplink data forwarding for at least one QoS flow for which the UL Forwarding IE is set to "UL forwarding proposed", it may include the UL Forwarding UP TNL Information IE in the Handover Request Acknowledge Transfer IE for the PDU session within the PDU Session Resource Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message.

In case of intra-system handover, for each PDU session for which the Additional DL UP TNL Information for HO List IE is included in the Handover Request Acknowledge Transfer IE of the HANDOVER REQUEST ACKNOWLEDGE message, the SMF shall consider the included Additional DL NG-U UP TNL Information IE as the downlink termination point for the associated flows indicated in the Additional QoS Flow Setup Response List IE for this PDU session split in different tunnels and shall consider the Additional DL Forwarding UP TNL Information IE, if included, as the forwarding tunnel associated to these QoS flows.

In case of intra-system handover, for each PDU session for which the Additional UL Forwarding UP TNL Information IE is included in the Handover Request Acknowledge Transfer IE of the HANDOVER REQUEST ACKNOWLEDGE message, the SMF shall consider it as the termination points for the uplink forwarding tunnels for this PDU session split in different tunnels.

In case of intra-system handover, if the target NG-RAN node accepts the data forwarding for a successfully configured DRB, the target NG-RAN node may include the DL Forwarding UP TNL Information IE for the DRB within the Data Forwarding Response DRB List IE within Handover Request Acknowledge Transfer IE of the HANDOVER REQUEST ACKNOWLEDGE message.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding UP TNL Information IE for a given DRB in the Data Forwarding Response DRB List IE within the Handover Request Acknowledge Transfer IE, it indicates the target NG-RAN node has requested the forwarding of uplink data for the DRB.

In case of inter-system handover from E-UTRAN, if the PDU Session Resource Setup Request Transfer IE contains the Direct Forwarding Path Availability IE set to "direct path available", the target NG-RAN node shall, if supported, and if it accepts downlink data forwarding for the QoS flows mapped to an E-RAB of an admitted PDU session, include the DL Forwarding UP TNL Information IE in the Data Forwarding Response E-RAB List IE in the Handover Request Acknowledge Transfer IE in the HANDOVER REQUEST ACKNOWLEDGE message for that mapped E-RAB.

In case of inter-system handover from E-UTRAN, the target NG-RAN node includes the Data Forwarding Accepted IE for each QoS flow that the DL Forwarding IE is set to "DL forwarding proposed" for the corresponding E-RAB in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE and that the target NG-RAN node has admitted the proposed forwarding of downlink data for the QoS flow. If indirect data forwarding is applied for inter-system handover, if the target NG-RAN node accepts the downlink data forwarding for at least one QoS flow of an admitted PDU session it shall include the DL Forwarding UP TNL Information IE in the PDU Session Resource Setup Response Transfer IE for that PDU session within the PDU Session Resources Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message.

In case of inter-system handover from E-UTRAN with direct forwarding, if the target NG-RAN node receives the SgNB UE X2AP ID IE in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE, it may use it for internal forwarding as described in TS 37.340 [32].

The target NG-RAN node shall use the information in the Mobility Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the target NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation;

-	assign proper RNA(s) for the UE when moving the UE to RRC_INACTIVE state.

If the Mobility Restriction List IE is not contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The target NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the QoS flows includes a particular ARP value (TS 23.501 [9]).

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, set to “Immediate MDT Only”, “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the target NG-RAN node shall ignore Interfaces To Trace IE, and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [x].

-	if the Trace Activation IE includes the MDT Configuration IE and if the NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

If the Location Reporting Request Type IE is included in the HANDOVER REQUEST message, the target NG-RAN node should perform the requested location reporting functionality for the UE as described in subclause 8.12.

If the Core Network Assistance Information for RRC INACTIVE IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for e.g. the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

If the CN Assisted RAN Parameters Tuning IE is included in the HANDOVER REQUEST message, the NG-RAN node may use it as described in TS 23.501 [9].

If the New Security Context Indicator IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall use the information as specified in TS 33.501 [13].

If the NASC IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall use it towards the UE as specified in TS 33.501 [13].

If the RRC Inactive Transition Report Request IE is included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context.

If the Redirection for Voice EPS Fallback IE is included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, store it and use it in a subsequent decision of EPS fallback for voice as specified in TS 23.502 [10].

If the SRVCC Operation Possible IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the received SRVCC Operation Possible IE in the UE context and use it as defined in TS 23.216 [31].

After all necessary resources for the admitted PDU session resources have been allocated, the target NG-RAN node shall generate the HANDOVER REQUEST ACKNOWLEDGE message.

If the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target NG-RAN shall, if supported, store the received information in the UE context, and use this information to allow subsequent selections of the UE for management based MDT defined in TS 32.422 [11].

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the HANDOVER REQUEST message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send the RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

Next Change

## 8.11	Trace Procedures

### 8.11.1	Trace Start

#### 8.11.1.1	General

The purpose of the Trace Start procedure is to allow the AMF to request the NG-RAN node to initiate a trace session for a UE. The procedure uses UE-associated signalling. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established as part of the procedure.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Trace start

The AMF initiates the procedure by sending a TRACE START message. Upon reception of the TRACE START message, the NG-RAN node shall initiate the requested trace session as described in TS 32.422 [11].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT and Trace”, the NG-RAN shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [11].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT Only”, “Logged MDT only”, the NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN node shall ignore Interfaces To Trace IE, and Trace Depth IE.

If the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, the NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT Only”, “Logged MDT only” and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [x].

If the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [x].

If the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, the NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [x].

If the Trace Activation IE includes the Sensor Measurement Configuration IE, within the MDT Configuration IE, the NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [x].

If the Trace Activation IE includes the MDT Configuration IE and if the NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

Interactions with other procedures:

If the NG-RAN node is not able to initiate the trace session due to ongoing handover of the UE to another NG-RAN node, the NG-RAN node shall initiate a Trace Failure Indication procedure with the appropriate cause value.

#### 8.11.1.3	Abnormal Conditions

Void.

Next Change

### 8.11.4	Cell Traffic Trace

#### 8.11.4.1	General

The purpose of the Cell Traffic Trace procedure is to send the allocated Trace Recording Session Reference and the Trace Reference to the AMF. The procedure uses UE-associated signalling.

#### 8.11.4.2	Successful Operation

Figure 8.11.4.2-1: Cell traffic trace

The NG-RAN node initiates the procedure by sending a CELL TRAFFIC TRACE message.

If the Privacy Indicator IE is included in the message, the AMF shall take the information into account for anonymization of MDT data as described in TS 32.422 [11].

#### 8.11.4.3	Abnormal Conditions

Void.

Next Change

### 9.2.2	UE Context Management Messages

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | C-ifPDUsessionResourceSetup |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| PDU Session Resource Setup Request List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Setup Request Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session NAS-PDU | O |  | NAS-PDU 9.3.3.4 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>PDU Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network | YES | reject |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Key | M |  | 9.3.1.87 |  | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 |  | YES | ignore |
| Emergency Fallback Indicator | O |  | 9.3.1.26 |  | YES | reject |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | 9.3.1.xx3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifPDUsessionResourceSetup | This IE shall be present if the PDU Session Resource Setup List IE is present. |

Next Change

#### 9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.22 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| NASC | O |  | NAS-PDU 9.3.3.4 | Refers to either the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container”, the details of the IE definition and the encoding arespecified in TS 24.501 [26]. | YES | reject |
| PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>Handover Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| Source to Target Transparent Container | M |  | 9.3.1.20 |  | YES | reject |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | 9.3.1.xx3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

Next Change

#### 9.3.1.14	Trace Activation

This IE defines parameters related to a trace session activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [11] (leftmost 6 octets, with PLMN information encoded as in 9.3.3.1), and Trace Recording Session Reference defined in TS 32.422 [11] (last 2 octets). |
| Interfaces to Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an NG-RAN node interface: first bit = NG-C, second bit = Xn-C, third bit = Uu, fourth bit = F1-C, fifth bit = E1: other bits reserved for future use. Value '1' indicates 'should be traced'. Value '0' indicates 'should not be traced'. |
| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, minimumWithoutVendorSpecificExtension, mediumWithoutVendorSpecificExtension, maximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [11]. |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.4 | Defined in TS 32.422 [11] |
| MDT Configuration | O |  | 9.3.1.xx1 |  |

Next Change

#### 9.3.1.xx1	MDT Configuration

The IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Configuration-NR | O |  | 9.3.1.xx4 |  | - | - |
| MDT Configuration-EUTRA | O |  | 9.3.1.xx5 |  | - | - |

#### 9.3.1.xx3	MDT PLMN List

The purpose of the MDT PLMN List IE is to provide the list of PLMN allowed for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT PLMN List |  | 1..<maxnoofMDTPLMNs> |  |  |
| >PLMN Identity | M |  | 9.3.3.5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |

#### 9.3.1.xx4	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…) |  | - | - |
| CHOICE Area Scope of MDT | M |  | FFS |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  |  |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [x]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration. Value “1” indicates “activate” and value “0” indicates “do not activate”. |  |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.x1 |  |  |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.x4 |  |  |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.x5 |  |  |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.x6 |  |  |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.x7 |  |  |  |
| >>M8 Configuration | O |  | 9.3.1.x8 |  |  |  |
| >>M9 Configuration | O |  | 9.3.1.x9 |  |  |  |
| >>MDT Location Information | O |  | 9.3.1.x10 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x11 (FFS: pending to RAN2) |  |  |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (1.28, 2.56, 5.12, 10.24, 20.48, 30.72, 40.96 and 61.44) | This IE is defined in TS 38.331 [18]. Unit: [second] |  |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120) | This IE is defined in TS 38.331 [18]. Unit: [minute]. |  |  |
| >>CHOICE Report Type | M |  |  |  |  |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>> Logged Event Trigger Config | M |  | 9.3.1.x14 |  |  |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.x11 |  |  |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.x12 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x13 |  |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.3.1.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

Editor note: The definition of Area Scope of MDT  IE may need to be updated and pending to RAN2 discussion.

Editor note: The measurement configurattions for M1 ~M9 in above tabular for immediate MDT are FFS.

#### 9.3.1.xx5	MDT Configuration- EUTRA

The IE defines the MDT configuration parameters of EUTRA.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - | - |
| CHOICE Area Scope of MDT | M |  | FFS |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>> E-UTRA CGI | M |  | 9.3.1.9 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  |  |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  | 9.3.3.11 |  | - | - |
| MDT Mode | M |  | OCTET STRING | MDTMode IE defined in TS 36.413 [16]. | - | - |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.3.1.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

Editor note: The definition of Area Scope of MDT  IE may need to be updated and pending to RAN2 discussion.

Editor note: The measurement configurattions for M1 ~M9 in above tabular for immediate MDT are FFS.

#### 9.3.1.x1	M1 Configuration

This IE defines the parameters for M1 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, A2event-triggered periodic, …) |  | - | - |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1. | - | - |
| >CHOICE Threshold | M |  |  |  | - | - |
| >>RSRP |  |  |  |  |  | - |
| >>>Threshold RSRP | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. | - | - |
| >>RSRQ |  |  |  |  |  | - |
| >>>Threshold RSRQ | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. | - | - |
| >>SINR |  |  |  |  |  |  |
| >>>Threshold SINR | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. | - | - |
| M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1. | - | - |
| >Report interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | This IE is defined in TS 38.331 [18]. | - | - |
| >Report amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. | - | - |

#### 9.3.1.x4	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M4 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.3.1.x5	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M5 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.3.1.x6	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, …) |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |
| M6 Delay Threshold | C-ifUL |  | ENUMERATED (ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, …) |  |

| Condition | Explanation |
| --- | --- |
| ifUL | This IE can be present only if the M6 Links to log IE is set to “uplink” or to “both-uplink-and-downlink”. |

#### 9.3.1.x7	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) | Unit: minutes |
| M7 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.3.1.x8	M8 Configuration

This IE defines the parameters for Bluetooth measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | 9.3.1.x2 |  |

#### 9.3.1.x9	M9 Configuration

This IE defines the parameters for WLAN measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | 9.3.1.x3 |  |

#### 9.3.1.x10	MDT Location Information

This IE defines the MDT Location Information.

Editor note: the definition of this IE may need to be updated and pending to RAN2 discussion.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [x]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value “1” indicates “activate” and value “0” indicates “do not activate”. |

9.3.1.x11	Bluetooth Measurement Configuration

This IE defines the parameters for Bluetooth measurement collection.

Editor note: the definition of this IE may need to be updated and pending to RAN2 discussion.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Bluetooth Measurement Configuration Name List |  | 0..1 |  |  |
| >Bluetooth Measurement Configuration Name Item IEs |  | 1 .. <maxnoofBluetoothName> |  |  |
| >>Bluetooth Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..248)) |  |
| BT RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 measurement as defined in 37.320 [x]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBluetoothName | Maximum no. of Bluetooth local name used for Bluetooth measurement collection. Value is 4. |

9.2.1.x12	WLAN Measurement Configuration

This IE defines the parameters for WLAN measurement collection.

Editor note: the definition of this IE may need to be updated and pending to RAN2 discussion.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| WLAN Measurement Configuration Name List |  | 0..1 |  |  |
| >WLAN Measurement Configuration Name Item IEs |  | 1 .. <maxnoofWLANName> |  |  |
| >>WLAN Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..32)) |  |
| WLAN RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 as defined in 37.320 [x]. |
| WLAN RTT | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M9 as defined in 37.320 [x]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofWLANName | Maximum no. of WLAN SSID used for WLAN measurement collection. Value is 4. |

9.3.1.x13		Sensor Measurement Configuration

This IE defines the parameters for Sensor measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Sensor Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Sensor Measurement Configuration Name |  | 0..1 |  |  |
| >Sensor Measurement Configuration Name Item IEs |  | 1 .. <maxnoofSensorName> |  |  |
| >>Choice Sesor Name | M |  | FFS: Pending to RAN2 |  |
| >>>Uncompensated Barometeric |  |  |  |  |
| >>>>Uncompensated Barometeric Configuration | M |  | ENUMERATED (True, …) |  |
| >>>UE speed |  |  |  |  |
| >>>>UE Speed Configuration | M |  | ENUMERATED (True, …) |  |
| >>>UE orientation |  |  |  |  |
| >>>>UE eorientation Configuration | M |  | ENUMERATED (True, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSensorName | Maximum no. of Sensor local name used for Sensor measurement collection. Value is 3 |

#### 9.3.1.x14	Event trigger Logged MDT Configuration

The IE defines the event trigger logged MDT configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Event trigger type | M |  |  |  |
| >Out-of-coverage |  |  |  |  |
| >>Out-of-Coverage Configuration |  | ENUMERATED {true, ...} |  |  |
| >A2 Event |  |  |  |  |
| >>CHOICE A2 Event Threshold | M |  |  |  |
| >>>RSRP |  |  |  |  |
| >>>>Threshold RSRP | M | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |  |
| >>>RSRQ |  |  |  |  |
| >>>>Threshold RSRQ | M | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |  |
| >>Hysteresis |  |  | INTEGER (0..30) | This parameter is used within the entry and leave condition of an event triggered reporting condition. |
| >>Time to trigger |  |  | ENUMERATED {ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120} | Time during which specific criteria for the event needs to be met in order to trigger a measurement report. |

Next Change

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFStatusIndication,

CellTrafficTrace,

DeactivateTrace,

DownlinkNASTransport,

DownlinkNonUEAssociatedNRPPaTransport,

DownlinkRANConfigurationTransfer,

DownlinkRANStatusTransfer,

DownlinkUEAssociatedNRPPaTransport,

ErrorIndication,

HandoverCancel,

HandoverCancelAcknowledge,

HandoverCommand,

HandoverFailure,

HandoverNotify,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

HandoverRequired,

InitialContextSetupFailure,

InitialContextSetupRequest,

InitialContextSetupResponse,

InitialUEMessage,

LocationReport,

LocationReportingControl,

LocationReportingFailureIndication,

NASNonDeliveryIndication,

NGReset,

NGResetAcknowledge,

NGSetupFailure,

NGSetupRequest,

NGSetupResponse,

OverloadStart,

OverloadStop,

Paging,

PathSwitchRequest,

PathSwitchRequestAcknowledge,

PathSwitchRequestFailure,

PDUSessionResourceModifyConfirm,

PDUSessionResourceModifyIndication,

PDUSessionResourceModifyRequest,

PDUSessionResourceModifyResponse,

PDUSessionResourceNotify,

PDUSessionResourceReleaseCommand,

PDUSessionResourceReleaseResponse,

PDUSessionResourceSetupRequest,

PDUSessionResourceSetupResponse,

PrivateMessage,

PWSCancelRequest,

PWSCancelResponse,

PWSFailureIndication,

PWSRestartIndication,

RANConfigurationUpdate,

RANConfigurationUpdateAcknowledge,

RANConfigurationUpdateFailure,

RerouteNASRequest,

RRCInactiveTransitionReport,

SecondaryRATDataUsageReport,

TraceFailureIndication,

TraceStart,

UEContextModificationFailure,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextReleaseRequest,

UERadioCapabilityCheckRequest,

UERadioCapabilityCheckResponse,

UERadioCapabilityInfoIndication,

UETNLABindingReleaseRequest,

UplinkNASTransport,

UplinkNonUEAssociatedNRPPaTransport,

UplinkRANConfigurationTransfer,

UplinkRANStatusTransfer,

UplinkUEAssociatedNRPPaTransport,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

UplinkRIMInformationTransfer,

DownlinkRIMInformationTransfer

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFStatusIndication,

id-CellTrafficTrace,

id-DeactivateTrace,

id-DownlinkNASTransport,

id-DownlinkNonUEAssociatedNRPPaTransport,

id-DownlinkRANConfigurationTransfer,

id-DownlinkRANStatusTransfer,

id-DownlinkUEAssociatedNRPPaTransport,

id-ErrorIndication,

id-HandoverCancel,

id-HandoverNotification,

id-HandoverPreparation,

id-HandoverResourceAllocation,

id-InitialContextSetup,

id-InitialUEMessage,

id-LocationReport,

id-LocationReportingControl,

id-LocationReportingFailureIndication,

id-NASNonDeliveryIndication,

id-NGReset,

id-NGSetup,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

id-PDUSessionResourceModify,

id-PDUSessionResourceModifyIndication,

id-PDUSessionResourceNotify,

id-PDUSessionResourceRelease,

id-PDUSessionResourceSetup,

id-PrivateMessage,

id-PWSCancel,

id-PWSFailureIndication,

id-PWSRestartIndication,

id-RANConfigurationUpdate,

id-RerouteNASRequest,

id-RRCInactiveTransitionReport,

id-SecondaryRATDataUsageReport,

id-TraceFailureIndication,

id-TraceStart,

id-UEContextModification,

id-UEContextRelease,

id-UEContextReleaseRequest,

id-UERadioCapabilityCheck,

id-UERadioCapabilityInfoIndication,

id-UETNLABindingRelease,

id-UplinkNASTransport,

id-UplinkNonUEAssociatedNRPPaTransport,

id-UplinkRANConfigurationTransfer,

id-UplinkRANStatusTransfer,

id-UplinkUEAssociatedNRPPaTransport,

id-WriteReplaceWarning,

id-UplinkRIMInformationTransfer,

id-DownlinkRIMInformationTransfer

FROM NGAP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

NGAP-PDU ::= CHOICE {

initiatingMessage			InitiatingMessage,

successfulOutcome			SuccessfulOutcome,

unsuccessfulOutcome			UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

uEContextModification			|

uEContextRelease				|

uERadioCapabilityCheck			|

writeReplaceWarning

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-2 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFStatusIndication				|

cellTrafficTrace				|

deactivateTrace					|

downlinkNASTransport			|

downlinkNonUEAssociatedNRPPaTransport	|

downlinkRANConfigurationTransfer	|

downlinkRANStatusTransfer		|

downlinkUEAssociatedNRPPaTransport	|

errorIndication					|

handoverNotification			|

initialUEMessage				|

locationReport					|

locationReportingControl		|

locationReportingFailureIndication	|

nASNonDeliveryIndication		|

overloadStart					|

overloadStop					|

paging							|

pDUSessionResourceNotify		|

privateMessage					|

pWSFailureIndication						|

pWSRestartIndication			|

rerouteNASRequest				|

rRCInactiveTransitionReport					|

secondaryRATDataUsageReport		|

traceFailureIndication			|

traceStart						|

uEContextReleaseRequest			|

uERadioCapabilityInfoIndication	|

uETNLABindingRelease			|

uplinkNASTransport				|

uplinkNonUEAssociatedNRPPaTransport	|

uplinkRANConfigurationTransfer	|

uplinkRANStatusTransfer			|

uplinkUEAssociatedNRPPaTransport				|

uplinkRIMInformationTransfer					|

downlinkRIMInformationTransfer

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

aMFConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFConfigurationUpdate

SUCCESSFUL OUTCOME		AMFConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	AMFConfigurationUpdateFailure

PROCEDURE CODE			id-AMFConfigurationUpdate

CRITICALITY				reject

}

aMFStatusIndication NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		AMFStatusIndication

PROCEDURE CODE			id-AMFStatusIndication

CRITICALITY				ignore

}

cellTrafficTrace NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-CellTrafficTrace

CRITICALITY				ignore

}

deactivateTrace NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

downlinkNASTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkNASTransport

PROCEDURE CODE			id-DownlinkNASTransport

CRITICALITY				ignore

}

downlinkNonUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkNonUEAssociatedNRPPaTransport

PROCEDURE CODE			id-DownlinkNonUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

downlinkRANConfigurationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRANConfigurationTransfer

PROCEDURE CODE			id-DownlinkRANConfigurationTransfer

CRITICALITY				ignore

}

downlinkRANStatusTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRANStatusTransfer

PROCEDURE CODE			id-DownlinkRANStatusTransfer

CRITICALITY				ignore

}

downlinkUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkUEAssociatedNRPPaTransport

PROCEDURE CODE			id-DownlinkUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

errorIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

handoverCancel NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

SUCCESSFUL OUTCOME		HandoverCancelAcknowledge

PROCEDURE CODE			id-HandoverCancel

CRITICALITY				reject

}

handoverNotification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverNotify

PROCEDURE CODE			id-HandoverNotification

CRITICALITY				ignore

}

handoverPreparation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequired

SUCCESSFUL OUTCOME		HandoverCommand

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-HandoverPreparation

CRITICALITY				reject

}

handoverResourceAllocation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverFailure

PROCEDURE CODE			id-HandoverResourceAllocation

CRITICALITY				reject

}

initialContextSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialContextSetupRequest

SUCCESSFUL OUTCOME		InitialContextSetupResponse

UNSUCCESSFUL OUTCOME	InitialContextSetupFailure

PROCEDURE CODE			id-InitialContextSetup

CRITICALITY				reject

}

initialUEMessage NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialUEMessage

PROCEDURE CODE			id-InitialUEMessage

CRITICALITY				ignore

}

locationReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReport

PROCEDURE CODE			id-LocationReport

CRITICALITY				ignore

}

locationReportingControl NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReportingControl

PROCEDURE CODE			id-LocationReportingControl

CRITICALITY				ignore

}

locationReportingFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReportingFailureIndication

PROCEDURE CODE			id-LocationReportingFailureIndication

CRITICALITY				ignore

}

nASNonDeliveryIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NASNonDeliveryIndication

PROCEDURE CODE			id-NASNonDeliveryIndication

CRITICALITY				ignore

}

nGReset NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGReset

SUCCESSFUL OUTCOME		NGResetAcknowledge

PROCEDURE CODE			id-NGReset

CRITICALITY				reject

}

nGSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGSetupRequest

SUCCESSFUL OUTCOME		NGSetupResponse

UNSUCCESSFUL OUTCOME	NGSetupFailure

PROCEDURE CODE			id-NGSetup

CRITICALITY				reject

}

overloadStart NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStart

PROCEDURE CODE			id-OverloadStart

CRITICALITY				ignore

}

overloadStop NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStop

PROCEDURE CODE			id-OverloadStop

CRITICALITY				reject

}

paging NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

pathSwitchRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PathSwitchRequest

SUCCESSFUL OUTCOME		PathSwitchRequestAcknowledge

UNSUCCESSFUL OUTCOME	PathSwitchRequestFailure

PROCEDURE CODE			id-PathSwitchRequest

CRITICALITY				reject

}

pDUSessionResourceModify NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceModifyRequest

SUCCESSFUL OUTCOME		PDUSessionResourceModifyResponse

PROCEDURE CODE			id-PDUSessionResourceModify

CRITICALITY				reject

}

pDUSessionResourceModifyIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceModifyIndication

SUCCESSFUL OUTCOME		PDUSessionResourceModifyConfirm

PROCEDURE CODE			id-PDUSessionResourceModifyIndication

CRITICALITY				reject

}

pDUSessionResourceNotify NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceNotify

PROCEDURE CODE			id-PDUSessionResourceNotify

CRITICALITY				ignore

}

pDUSessionResourceRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceReleaseCommand

SUCCESSFUL OUTCOME		PDUSessionResourceReleaseResponse

PROCEDURE CODE			id-PDUSessionResourceRelease

CRITICALITY				reject

}

pDUSessionResourceSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceSetupRequest

SUCCESSFUL OUTCOME		PDUSessionResourceSetupResponse

PROCEDURE CODE			id-PDUSessionResourceSetup

CRITICALITY				reject

}

privateMessage NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-PrivateMessage

CRITICALITY				ignore

}

pWSCancel NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

pWSFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

pWSRestartIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

rANConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANConfigurationUpdate

SUCCESSFUL OUTCOME		RANConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	RANConfigurationUpdateFailure

PROCEDURE CODE			id-RANConfigurationUpdate

CRITICALITY				reject

}

rerouteNASRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RerouteNASRequest

PROCEDURE CODE			id-RerouteNASRequest

CRITICALITY				reject

}

rRCInactiveTransitionReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCInactiveTransitionReport

PROCEDURE CODE			id-RRCInactiveTransitionReport

CRITICALITY				ignore

}

secondaryRATDataUsageReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-SecondaryRATDataUsageReport

CRITICALITY				ignore

}

traceFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceFailureIndication

PROCEDURE CODE			id-TraceFailureIndication

CRITICALITY				ignore

}

traceStart NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

uEContextModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextReleaseRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

uERadioCapabilityCheck NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityCheckRequest

SUCCESSFUL OUTCOME		UERadioCapabilityCheckResponse

PROCEDURE CODE			id-UERadioCapabilityCheck

CRITICALITY				reject

}

uERadioCapabilityInfoIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityInfoIndication

PROCEDURE CODE			id-UERadioCapabilityInfoIndication

CRITICALITY				ignore

}

uETNLABindingRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UETNLABindingReleaseRequest

PROCEDURE CODE			id-UETNLABindingRelease

CRITICALITY				ignore

}

uplinkNASTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkNASTransport

PROCEDURE CODE			id-UplinkNASTransport

CRITICALITY				ignore

}

uplinkNonUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkNonUEAssociatedNRPPaTransport

PROCEDURE CODE			id-UplinkNonUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

uplinkRANConfigurationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRANConfigurationTransfer

PROCEDURE CODE			id-UplinkRANConfigurationTransfer

CRITICALITY				ignore

}

uplinkRANStatusTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRANStatusTransfer

PROCEDURE CODE			id-UplinkRANStatusTransfer

CRITICALITY				ignore

}

uplinkUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkUEAssociatedNRPPaTransport

PROCEDURE CODE			id-UplinkUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

writeReplaceWarning NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

uplinkRIMInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRIMInformationTransfer

PROCEDURE CODE			id-UplinkRIMInformationTransfer

CRITICALITY				ignore

}

downlinkRIMInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRIMInformationTransfer

PROCEDURE CODE			id-DownlinkRIMInformationTransfer

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

CNAssistedRANTuning,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformationForInactive,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EN-DCSONConfigurationTransfer,

EUTRA-CGI,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverFlag,

HandoverType,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

LAI,

LocationReportingRequestType,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NASSecurityParametersFromNGRAN,

NewSecurityContextInd,

NGRAN-CGI,

NGRAN-TNLAssociationToRemoveList,

NGRANTraceID,

NR-CGI,

NRPPa-PDU,

NumberOfBroadcastsRequested,

OverloadResponse,

OverloadStartNSSAIList,

PagingDRX,

PagingOrigin,

PagingPriority,

PDUSessionAggregateMaximumBitRate,

PDUSessionResourceAdmittedList,

PDUSessionResourceFailedToModifyListModCfm,

PDUSessionResourceFailedToModifyListModRes,

PDUSessionResourceFailedToSetupListCxtFail,

PDUSessionResourceFailedToSetupListCxtRes,

PDUSessionResourceFailedToSetupListHOAck,

PDUSessionResourceFailedToSetupListPSReq,

PDUSessionResourceFailedToSetupListSURes,

PDUSessionResourceHandoverList,

PDUSessionResourceListCxtRelCpl,

PDUSessionResourceListCxtRelReq,

PDUSessionResourceListHORqd,

PDUSessionResourceModifyListModCfm,

PDUSessionResourceModifyListModInd,

PDUSessionResourceModifyListModReq,

PDUSessionResourceModifyListModRes,

PDUSessionResourceNotifyList,

PDUSessionResourceReleasedListNot,

PDUSessionResourceReleasedListPSAck,

PDUSessionResourceReleasedListPSFail,

PDUSessionResourceReleasedListRelRes,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupListCxtReq,

PDUSessionResourceSetupListCxtRes,

PDUSessionResourceSetupListHOReq,

PDUSessionResourceSetupListSUReq,

PDUSessionResourceSetupListSURes,

PDUSessionResourceSwitchedList,

PDUSessionResourceToBeSwitchedDLList,

PDUSessionResourceToReleaseListHOCmd,

PDUSessionResourceToReleaseListRelCmd,

PLMNSupportList,

PWSFailedCellIDList,

RANNodeName,

RANPagingPriority,

RANStatusTransfer-TransparentContainer,

RAN-UE-NGAP-ID,

RedirectionVoiceFallback,

RelativeAMFCapacity,

RepetitionPeriod,

ResetType,

RoutingID,

RRCEstablishmentCause,

RRCInactiveTransitionReportRequest,

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SourceToTarget-AMFInformationReroute,

SRVCCOperationPossible,

SupportedTAList,

TAIListForPaging,

TAIListForRestart,

TargetID,

TargetToSource-TransparentContainer,

TimeToWait,

TNLAssociationList,

TraceActivation,

TrafficLoadReductionIndication,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-connectionList,

UEContextRequest,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UERetentionInformation,

UESecurityCapabilities,

UnavailableGUAMIList,

UserLocationInformation,

WarningAreaCoordinates,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType,

RIMInformationTransfer,

MDTPLMNList

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-CNAssistedRANTuning,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformationForInactive,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-ENDC-SONConfigurationTransferDL,

id-ENDC-SONConfigurationTransferUL,

id-EUTRA-CGI,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverFlag,

id-HandoverType,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-LocationReportingRequestType,

id-MaskedIMEISV,

id-MessageIdentifier,

id-MobilityRestrictionList,

id-NAS-PDU,

id-NASC,

id-NASSecurityParametersFromNGRAN,

id-NewAMF-UE-NGAP-ID,

id-NewGUAMI,

id-NewSecurityContextInd,

id-NGAP-Message,

id-NGRAN-CGI,

id-NGRAN-TNLAssociationToRemoveList,

id-NGRANTraceID,

id-NR-CGI,

id-NRPPa-PDU,

id-NumberOfBroadcastsRequested,

id-OldAMF,

id-OverloadStartNSSAIList,

id-PagingDRX,

id-PagingOrigin,

id-PagingPriority,

id-PDUSessionResourceAdmittedList,

id-PDUSessionResourceFailedToModifyListModCfm,

id-PDUSessionResourceFailedToModifyListModRes,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceFailedToSetupListCxtRes,

id-PDUSessionResourceFailedToSetupListHOAck,

id-PDUSessionResourceFailedToSetupListPSReq,

id-PDUSessionResourceFailedToSetupListSURes,

id-PDUSessionResourceHandoverList,

id-PDUSessionResourceListCxtRelCpl,

id-PDUSessionResourceListCxtRelReq,

id-PDUSessionResourceListHORqd,

id-PDUSessionResourceModifyListModCfm,

id-PDUSessionResourceModifyListModInd,

id-PDUSessionResourceModifyListModReq,

id-PDUSessionResourceModifyListModRes,

id-PDUSessionResourceNotifyList,

id-PDUSessionResourceReleasedListNot,

id-PDUSessionResourceReleasedListPSAck,

id-PDUSessionResourceReleasedListPSFail,

id-PDUSessionResourceReleasedListRelRes,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourceSetupListCxtReq,

id-PDUSessionResourceSetupListCxtRes,

id-PDUSessionResourceSetupListHOReq,

id-PDUSessionResourceSetupListSUReq,

id-PDUSessionResourceSetupListSURes,

id-PDUSessionResourceSwitchedList,

id-PDUSessionResourceToBeSwitchedDLList,

id-PDUSessionResourceToReleaseListHOCmd,

id-PDUSessionResourceToReleaseListRelCmd,

id-PLMNSupportList,

id-PWSFailedCellIDList,

id-RANNodeName,

id-RANPagingPriority,

id-RANStatusTransfer-TransparentContainer,

id-RAN-UE-NGAP-ID,

id-RedirectionVoiceFallback,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

id-ResetType,

id-RoutingID,

id-RRCEstablishmentCause,

id-RRCInactiveTransitionReportRequest,

id-RRCState,

id-SecurityContext,

id-SecurityKey,

id-SerialNumber,

id-ServedGUAMIList,

id-SliceSupportList,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-TransparentContainer,

id-SourceToTarget-AMFInformationReroute,

id-SRVCCOperationPossible,

id-SupportedTAList,

id-TAIListForPaging,

id-TAIListForRestart,

id-TargetID,

id-TargetToSource-TransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-connectionList,

id-UEContextRequest,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UERetentionInformation,

id-UESecurityCapabilities,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-RIMInformationTransfer,

id-ManagementBasedMDTPLMNList

FROM NGAP-Constants;

-- **************************************************************

--

#### -- PDU SESSION MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU Session Resource Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE SETUP REQUEST

--

-- **************************************************************

PDUSessionResourceSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceSetupRequestIEs} },

...

}

PDUSessionResourceSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority							PRESENCE optional		}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU									PRESENCE optional		}|

{ ID id-PDUSessionResourceSetupListSUReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListSUReq		PRESENCE mandatory	}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY ignore	TYPE UEAggregateMaximumBitRate				PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE SETUP RESPONSE

--

-- **************************************************************

PDUSessionResourceSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceSetupResponseIEs} },

...

}

PDUSessionResourceSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListSURes			CRITICALITY ignore	TYPE PDUSessionResourceSetupListSURes				PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToSetupListSURes	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListSURes		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE RELEASE COMMAND

--

-- **************************************************************

PDUSessionResourceReleaseCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceReleaseCommandIEs} },

...

}

PDUSessionResourceReleaseCommandIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RANPagingPriority							CRITICALITY ignore	TYPE RANPagingPriority								PRESENCE optional		}|

{ ID id-NAS-PDU										CRITICALITY ignore	TYPE NAS-PDU											PRESENCE optional		}|

{ ID id-PDUSessionResourceToReleaseListRelCmd		CRITICALITY reject	TYPE PDUSessionResourceToReleaseListRelCmd		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE RELEASE RESPONSE

--

-- **************************************************************

PDUSessionResourceReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceReleaseResponseIEs} },

...

}

PDUSessionResourceReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListRelRes			CRITICALITY ignore	TYPE PDUSessionResourceReleasedListRelRes		PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation						PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Modify Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY REQUEST

--

-- **************************************************************

PDUSessionResourceModifyRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyRequestIEs} },

...

}

PDUSessionResourceModifyRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority							PRESENCE optional		}|

{ ID id-PDUSessionResourceModifyListModReq		CRITICALITY reject	TYPE PDUSessionResourceModifyListModReq	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY RESPONSE

--

-- **************************************************************

PDUSessionResourceModifyResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyResponseIEs} },

...

}

PDUSessionResourceModifyResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID									CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID									CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-PDUSessionResourceModifyListModRes				CRITICALITY ignore	TYPE PDUSessionResourceModifyListModRes			PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToModifyListModRes		CRITICALITY ignore	TYPE PDUSessionResourceFailedToModifyListModRes		PRESENCE optional	}|

{ ID id-UserLocationInformation							CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Notify Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

#### -- PDU SESSION RESOURCE NOTIFY

--

-- **************************************************************

PDUSessionResourceNotify ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceNotifyIEs} },

...

}

PDUSessionResourceNotifyIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceNotifyList			CRITICALITY reject	TYPE PDUSessionResourceNotifyList				PRESENCE optional		}|

{ ID id-PDUSessionResourceReleasedListNot		CRITICALITY ignore	TYPE PDUSessionResourceReleasedListNot		PRESENCE optional		}|

{ ID id-UserLocationInformation					CRITICALITY ignore	TYPE UserLocationInformation						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Modify Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY INDICATION

--

-- **************************************************************

PDUSessionResourceModifyIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyIndicationIEs} },

...

}

PDUSessionResourceModifyIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceModifyListModInd		CRITICALITY reject	TYPE PDUSessionResourceModifyListModInd		PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY ignore	TYPE UserLocationInformation						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY CONFIRM

--

-- **************************************************************

PDUSessionResourceModifyConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyConfirmIEs} },

...

}

PDUSessionResourceModifyConfirmIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceModifyListModCfm			CRITICALITY ignore	TYPE PDUSessionResourceModifyListModCfm		PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToModifyListModCfm	CRITICALITY ignore	TYPE PDUSessionResourceFailedToModifyListModCfm		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE CONTEXT MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Initial Context Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL CONTEXT SETUP REQUEST

--

-- **************************************************************

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-OldAMF									CRITICALITY reject	TYPE AMFName											PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY reject	TYPE UEAggregateMaximumBitRate						PRESENCE conditional	}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-GUAMI									CRITICALITY reject	TYPE GUAMI												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListCxtReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListCxtReq			PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI										PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities							PRESENCE mandatory	}|

{ ID id-SecurityKey								CRITICALITY reject	TYPE SecurityKey										PRESENCE mandatory	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional		}|

{ ID id-MobilityRestrictionList					CRITICALITY ignore	TYPE MobilityRestrictionList							PRESENCE optional		}|

{ ID id-UERadioCapability						CRITICALITY ignore	TYPE UERadioCapability									PRESENCE optional		}|

{ ID id-IndexToRFSP								CRITICALITY ignore	TYPE IndexToRFSP										PRESENCE optional		}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional		}|

{ ID id-NAS-PDU									CRITICALITY ignore	TYPE NAS-PDU											PRESENCE optional		}|

{ ID id-EmergencyFallbackIndicator				CRITICALITY reject	TYPE EmergencyFallbackIndicator						PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest			PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging				CRITICALITY ignore	TYPE UERadioCapabilityForPaging						PRESENCE optional		}|

{ ID id-RedirectionVoiceFallback				CRITICALITY ignore	TYPE RedirectionVoiceFallback						PRESENCE optional		}|

{ ID id-LocationReportingRequestType			CRITICALITY ignore	TYPE LocationReportingRequestType					PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning								PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible							PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList										PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- INITIAL CONTEXT SETUP RESPONSE

--

-- **************************************************************

InitialContextSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupResponseIEs} },

...

}

InitialContextSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListCxtRes			CRITICALITY ignore	TYPE PDUSessionResourceSetupListCxtRes					PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToSetupListCxtRes	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListCxtRes		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- INITIAL CONTEXT SETUP FAILURE

--

-- **************************************************************

InitialContextSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupFailureIEs} },

...

}

InitialContextSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToSetupListCxtFail	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListCxtFail		PRESENCE optional		}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause														PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE Context Release Request Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT RELEASE REQUEST

--

-- **************************************************************

UEContextReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseRequest-IEs} },

...

}

UEContextReleaseRequest-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID						CRITICALITY reject	TYPE RAN-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-PDUSessionResourceListCxtRelReq		CRITICALITY reject	TYPE PDUSessionResourceListCxtRelReq		PRESENCE optional		}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UE Context Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMMAND

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseCommand-IEs} },

...

}

UEContextReleaseCommand-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-UE-NGAP-IDs				CRITICALITY reject	TYPE UE-NGAP-IDs			PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseComplete-IEs} },

...

}

UEContextReleaseComplete-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging	PRESENCE optional		}|

{ ID id-PDUSessionResourceListCxtRelCpl				CRITICALITY	reject	TYPE PDUSessionResourceListCxtRelCpl					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE Context Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

#### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority								PRESENCE optional		}|

{ ID id-SecurityKey								CRITICALITY reject	TYPE SecurityKey									PRESENCE optional		}|

{ ID id-IndexToRFSP								CRITICALITY ignore	TYPE IndexToRFSP									PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY ignore	TYPE UEAggregateMaximumBitRate					PRESENCE optional		}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities						PRESENCE optional		}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive			PRESENCE optional		}|

{ ID id-EmergencyFallbackIndicator				CRITICALITY reject	TYPE EmergencyFallbackIndicator					PRESENCE optional		}|

{ ID id-NewAMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest		PRESENCE optional		}|

{ ID id-NewGUAMI								CRITICALITY reject	TYPE GUAMI											PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning							PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RRCState					CRITICALITY ignore	TYPE RRCState					PRESENCE optional		}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation	PRESENCE optional 	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

UEContextModificationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationFailureIEs} },

...

}

UEContextModificationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- RRC INACTIVE TRANSITION REPORT

--

-- **************************************************************

RRCInactiveTransitionReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RRCInactiveTransitionReportIEs} },

...

}

RRCInactiveTransitionReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RRCState					CRITICALITY ignore	TYPE RRCState						PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE MOBILITY MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Handover Preparation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- HANDOVER REQUIRED

--

-- **************************************************************

HandoverRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequiredIEs} },

...

}

HandoverRequiredIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	}|

{ ID id-TargetID								CRITICALITY reject	TYPE TargetID										PRESENCE mandatory	}|

{ ID id-DirectForwardingPathAvailability		CRITICALITY ignore	TYPE DirectForwardingPathAvailability			PRESENCE optional		}|

{ ID id-PDUSessionResourceListHORqd				CRITICALITY reject	TYPE PDUSessionResourceListHORqd				PRESENCE mandatory	}|

{ ID id-SourceToTarget-TransparentContainer		CRITICALITY reject	TYPE SourceToTarget-TransparentContainer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- HANDOVER COMMAND

--

-- **************************************************************

HandoverCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverCommandIEs} },

...

}

HandoverCommandIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-HandoverType								CRITICALITY reject	TYPE HandoverType											PRESENCE mandatory	}|

{ ID id-NASSecurityParametersFromNGRAN				CRITICALITY reject	TYPE NASSecurityParametersFromNGRAN					PRESENCE conditional	}|

-- This IE shall be present if HandoverType IE is set to value "5GStoEPPS" or “5GStoUTRAN” --

{ ID id-PDUSessionResourceHandoverList				CRITICALITY ignore	TYPE PDUSessionResourceHandoverList					PRESENCE optional	}|

{ ID id-PDUSessionResourceToReleaseListHOCmd		CRITICALITY ignore	TYPE PDUSessionResourceToReleaseListHOCmd			PRESENCE optional		}|

{ ID id-TargetToSource-TransparentContainer			CRITICALITY reject	TYPE TargetToSource-TransparentContainer			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverPreparationFailureIEs} },

...

}

HandoverPreparationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- Handover Resource Allocation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequestIEs} },

...

}

HandoverRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory	}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive			PRESENCE optional		}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities						PRESENCE mandatory	}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext								PRESENCE mandatory	}|

{ ID id-NewSecurityContextInd					CRITICALITY reject	TYPE NewSecurityContextInd						PRESENCE optional		}|

{ ID id-NASC									CRITICALITY reject	TYPE NAS-PDU										PRESENCE optional		}|

{ ID id-PDUSessionResourceSetupListHOReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListHOReq			PRESENCE mandatory	}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE mandatory	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional		}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV									PRESENCE optional		}|

{ ID id-SourceToTarget-TransparentContainer		CRITICALITY reject	TYPE SourceToTarget-TransparentContainer		PRESENCE mandatory	}|

{ ID id-MobilityRestrictionList					CRITICALITY ignore	TYPE MobilityRestrictionList						PRESENCE optional		}|

{ ID id-LocationReportingRequestType			CRITICALITY ignore	TYPE LocationReportingRequestType				PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest		PRESENCE optional		}|

{ ID id-GUAMI									CRITICALITY reject	TYPE GUAMI											PRESENCE mandatory	}|

{ ID id-RedirectionVoiceFallback				CRITICALITY ignore	TYPE RedirectionVoiceFallback					PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning							PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible						PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList									PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequestAcknowledgeIEs} },

...

}

HandoverRequestAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-PDUSessionResourceAdmittedList				CRITICALITY ignore	TYPE PDUSessionResourceAdmittedList					PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToSetupListHOAck	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListHOAck		PRESENCE optional		}|

{ ID id-TargetToSource-TransparentContainer			CRITICALITY reject	TYPE TargetToSource-TransparentContainer			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- HANDOVER FAILURE

--

-- **************************************************************

HandoverFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverFailureIEs} },

...

}

HandoverFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- Handover Notification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- HANDOVER NOTIFY

--

-- **************************************************************

HandoverNotify ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverNotifyIEs} },

...

}

HandoverNotifyIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation	PRESENCE mandatory		},

...

}

-- **************************************************************

--

#### -- Path Switch Request Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PATH SWITCH REQUEST

--

-- **************************************************************

PathSwitchRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestIEs} },

...

}

PathSwitchRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-SourceAMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities						CRITICALITY ignore	TYPE UESecurityCapabilities								PRESENCE mandatory	}|

{ ID id-PDUSessionResourceToBeSwitchedDLList		CRITICALITY reject	TYPE PDUSessionResourceToBeSwitchedDLList			PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToSetupListPSReq	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListPSReq		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PATH SWITCH REQUEST ACKNOWLEDGE

--

-- **************************************************************

PathSwitchRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestAcknowledgeIEs} },

...

}

PathSwitchRequestAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities						CRITICALITY reject	TYPE UESecurityCapabilities							PRESENCE optional		}|

{ ID id-SecurityContext								CRITICALITY reject	TYPE SecurityContext									PRESENCE mandatory	}|

{ ID id-NewSecurityContextInd						CRITICALITY reject	TYPE NewSecurityContextInd							PRESENCE optional		}|

{ ID id-PDUSessionResourceSwitchedList				CRITICALITY ignore	TYPE PDUSessionResourceSwitchedList				PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListPSAck			CRITICALITY ignore	TYPE PDUSessionResourceReleasedListPSAck		PRESENCE optional		}|

{ ID id-AllowedNSSAI								CRITICALITY reject	TYPE AllowedNSSAI										PRESENCE mandatory	}|

{ ID id-CoreNetworkAssistanceInformationForInactive			CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive			PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest			CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest			PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		}|

{ ID id-RedirectionVoiceFallback					CRITICALITY ignore	TYPE RedirectionVoiceFallback						PRESENCE optional		}|

{ ID id-CNAssistedRANTuning							CRITICALITY ignore	TYPE CNAssistedRANTuning								PRESENCE optional		}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible							PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PATH SWITCH REQUEST FAILURE

--

-- **************************************************************

PathSwitchRequestFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestFailureIEs} },

...

}

PathSwitchRequestFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY ignore	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY ignore	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListPSFail	CRITICALITY ignore	TYPE PDUSessionResourceReleasedListPSFail	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- Handover Cancellation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverCancelIEs} },

...

}

HandoverCancelIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause				PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- HANDOVER CANCEL ACKNOWLEDGE

--

-- **************************************************************

HandoverCancelAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverCancelAcknowledgeIEs} },

...

}

HandoverCancelAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Uplink RAN Status Transfer Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UPLINK RAN STATUS TRANSFER

--

-- **************************************************************

UplinkRANStatusTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkRANStatusTransferIEs} },

...

}

UplinkRANStatusTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RANStatusTransfer-TransparentContainer		CRITICALITY reject	TYPE RANStatusTransfer-TransparentContainer	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Downlink RAN Status Transfer Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- DOWNLINK RAN STATUS TRANSFER

--

-- **************************************************************

DownlinkRANStatusTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkRANStatusTransferIEs} },

...

}

DownlinkRANStatusTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID			PRESENCE mandatory	}|

{ ID id-RANStatusTransfer-TransparentContainer		CRITICALITY reject	TYPE RANStatusTransfer-TransparentContainer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PAGING

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PagingIEs} },

...

}

PagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UEPagingIdentity				CRITICALITY ignore	TYPE UEPagingIdentity				PRESENCE mandatory	}|

{ ID id-PagingDRX						CRITICALITY ignore	TYPE PagingDRX						PRESENCE optional		}|

{ ID id-TAIListForPaging				CRITICALITY ignore	TYPE TAIListForPaging				PRESENCE mandatory	}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority					PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional		}|

{ ID id-PagingOrigin					CRITICALITY ignore	TYPE PagingOrigin					PRESENCE optional		}|

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- NAS TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL UE MESSAGE

--

-- **************************************************************

InitialUEMessage ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialUEMessage-IEs} },

...

}

InitialUEMessage-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU										PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY reject	TYPE UserLocationInformation						PRESENCE mandatory	}|

{ ID id-RRCEstablishmentCause					CRITICALITY ignore	TYPE RRCEstablishmentCause						PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI							CRITICALITY reject	TYPE FiveG-S-TMSI									PRESENCE optional		}|

{ ID id-AMFSetID								CRITICALITY ignore	TYPE AMFSetID										PRESENCE optional		}|

{ ID id-UEContextRequest						CRITICALITY ignore	TYPE UEContextRequest								PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional		}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- DOWNLINK NAS TRANSPORT

--

-- **************************************************************

DownlinkNASTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkNASTransport-IEs} },

...

}

DownlinkNASTransport-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-OldAMF							CRITICALITY reject	TYPE AMFName						PRESENCE optional		}|

{ ID id-RANPagingPriority				CRITICALITY ignore	TYPE RANPagingPriority				PRESENCE optional		}|

{ ID id-NAS-PDU							CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory	}|

{ ID id-MobilityRestrictionList			CRITICALITY ignore	TYPE MobilityRestrictionList		PRESENCE optional		}|

{ ID id-IndexToRFSP						CRITICALITY ignore	TYPE IndexToRFSP					PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate		CRITICALITY ignore	TYPE UEAggregateMaximumBitRate		PRESENCE optional		}|

{ ID id-AllowedNSSAI					CRITICALITY reject	TYPE AllowedNSSAI					PRESENCE optional		}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UPLINK NAS TRANSPORT

--

-- **************************************************************

UplinkNASTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{ {UplinkNASTransport-IEs} },

...

}

UplinkNASTransport-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- NAS NON DELIVERY INDICATION

--

-- **************************************************************

NASNonDeliveryIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NASNonDeliveryIndication-IEs} },

...

}

NASNonDeliveryIndication-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-NAS-PDU				CRITICALITY ignore	TYPE NAS-PDU			PRESENCE mandatory	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- REROUTE NAS REQUEST

--

-- **************************************************************

RerouteNASRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RerouteNASRequest-IEs} },

...

}

RerouteNASRequest-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-AMF-UE-NGAP-ID							CRITICALITY ignore	TYPE AMF-UE-NGAP-ID									PRESENCE optional	}|

{ ID id-NGAP-Message							CRITICALITY reject	TYPE OCTET STRING									PRESENCE mandatory	}|

{ ID id-AMFSetID								CRITICALITY reject	TYPE AMFSetID										PRESENCE mandatory	}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional	}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- INTERFACE MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- NG Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID			CRITICALITY reject	TYPE GlobalRANNodeID			PRESENCE mandatory	}|

{ ID id-RANNodeName				CRITICALITY ignore	TYPE RANNodeName				PRESENCE optional}|

{ ID id-SupportedTAList			CRITICALITY reject	TYPE SupportedTAList			PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX		CRITICALITY ignore	TYPE PagingDRX					PRESENCE mandatory	}|

{ ID id-UERetentionInformation	CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- NG SETUP RESPONSE

--

-- **************************************************************

NGSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupResponseIEs} },

...

}

NGSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName						CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-ServedGUAMIList				CRITICALITY reject	TYPE ServedGUAMIList			PRESENCE mandatory	}|

{ ID id-RelativeAMFCapacity			CRITICALITY ignore	TYPE RelativeAMFCapacity		PRESENCE mandatory	}|

{ ID id-PLMNSupportList				CRITICALITY reject	TYPE PLMNSupportList			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- NG SETUP FAILURE

--

-- **************************************************************

NGSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupFailureIEs} },

...

}

NGSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- RAN Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- RAN CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

RANConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateAcknowledgeIEs} },

...

}

RANConfigurationUpdateAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- RAN CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

RANConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateFailureIEs} },

...

}

RANConfigurationUpdateFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- AMF Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- AMF CONFIGURATION UPDATE

--

-- **************************************************************

AMFConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFConfigurationUpdateIEs} },

...

}

AMFConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName								CRITICALITY reject	TYPE AMFName								PRESENCE optional		}|

{ ID id-ServedGUAMIList						CRITICALITY reject	TYPE ServedGUAMIList						PRESENCE optional		}|

{ ID id-RelativeAMFCapacity					CRITICALITY ignore	TYPE RelativeAMFCapacity					PRESENCE optional		}|

{ ID id-PLMNSupportList						CRITICALITY reject	TYPE PLMNSupportList						PRESENCE optional		}|

{ ID id-AMF-TNLAssociationToAddList			CRITICALITY ignore	TYPE AMF-TNLAssociationToAddList			PRESENCE optional		}|

{ ID id-AMF-TNLAssociationToRemoveList		CRITICALITY ignore	TYPE AMF-TNLAssociationToRemoveList		PRESENCE optional		}|

{ ID id-AMF-TNLAssociationToUpdateList		CRITICALITY ignore	TYPE AMF-TNLAssociationToUpdateList		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- AMF CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

AMFConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFConfigurationUpdateAcknowledgeIEs} },

...

}

AMFConfigurationUpdateAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-TNLAssociationSetupList				CRITICALITY ignore	TYPE AMF-TNLAssociationSetupList			PRESENCE optional		}|

{ ID id-AMF-TNLAssociationFailedToSetupList		CRITICALITY ignore	TYPE TNLAssociationList							PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- AMF CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

AMFConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFConfigurationUpdateFailureIEs} },

...

}

AMFConfigurationUpdateFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- AMF Status Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- AMF STATUS INDICATION

--

-- **************************************************************

AMFStatusIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFStatusIndicationIEs} },

...

}

AMFStatusIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UnavailableGUAMIList		CRITICALITY reject	TYPE UnavailableGUAMIList		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- NG Reset Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- NG RESET

--

-- **************************************************************

NGReset ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGResetIEs} },

...

}

NGResetIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- NG RESET ACKNOWLEDGE

--

-- **************************************************************

NGResetAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGResetAcknowledgeIEs} },

...

}

NGResetAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalNG-connectionList			CRITICALITY ignore	TYPE UE-associatedLogicalNG-connectionList			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Error Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {ErrorIndicationIEs} },

...

}

ErrorIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE optional		}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE optional		}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- OVERLOAD START

--

-- **************************************************************

OverloadStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {OverloadStartIEs} },

...

}

OverloadStartIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFOverloadResponse						CRITICALITY reject	TYPE OverloadResponse							PRESENCE optional 	}|

{ ID id-AMFTrafficLoadReductionIndication		CRITICALITY ignore	TYPE TrafficLoadReductionIndication		PRESENCE optional		}|

{ ID id-OverloadStartNSSAIList					CRITICALITY ignore	TYPE OverloadStartNSSAIList					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- OVERLOAD STOP

--

-- **************************************************************

OverloadStop ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {OverloadStopIEs} },

...

}

OverloadStopIEs NGAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CONFIGURATION TRANSFER ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UPLINK RAN CONFIGURATION TRANSFER

--

-- **************************************************************

UplinkRANConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkRANConfigurationTransferIEs} },

...

}

UplinkRANConfigurationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransferUL			CRITICALITY ignore	TYPE SONConfigurationTransfer			PRESENCE optional	}|

{ ID id-ENDC-SONConfigurationTransferUL		CRITICALITY ignore	TYPE EN-DCSONConfigurationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- DOWNLINK RAN CONFIGURATION TRANSFER

--

-- **************************************************************

DownlinkRANConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkRANConfigurationTransferIEs} },

...

}

DownlinkRANConfigurationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransferDL			CRITICALITY ignore	TYPE SONConfigurationTransfer			PRESENCE optional	}|

{ ID id-ENDC-SONConfigurationTransferDL		CRITICALITY ignore	TYPE EN-DCSONConfigurationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- WARNING MESSAGE TRANSMISSION ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- Write-Replace Warning Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- WRITE-REPLACE WARNING REQUEST

--

-- **************************************************************

WriteReplaceWarningRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {WriteReplaceWarningRequestIEs} },

...

}

WriteReplaceWarningRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier				CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber					CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-WarningAreaList					CRITICALITY ignore	TYPE WarningAreaList					PRESENCE optional		}|

{ ID id-RepetitionPeriod				CRITICALITY reject	TYPE RepetitionPeriod					PRESENCE mandatory	}|

{ ID id-NumberOfBroadcastsRequested		CRITICALITY reject	TYPE NumberOfBroadcastsRequested		PRESENCE mandatory	}|

{ ID id-WarningType						CRITICALITY ignore	TYPE WarningType						PRESENCE optional		}|

{ ID id-WarningSecurityInfo				CRITICALITY ignore	TYPE WarningSecurityInfo				PRESENCE optional		}|

{ ID id-DataCodingScheme				CRITICALITY ignore	TYPE DataCodingScheme					PRESENCE optional		}|

{ ID id-WarningMessageContents			CRITICALITY ignore	TYPE WarningMessageContents				PRESENCE optional		}|

{ ID id-ConcurrentWarningMessageInd		CRITICALITY reject	TYPE ConcurrentWarningMessageInd		PRESENCE optional		}|

{ ID id-WarningAreaCoordinates			CRITICALITY ignore	TYPE WarningAreaCoordinates				PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- WRITE-REPLACE WARNING RESPONSE

--

-- **************************************************************

WriteReplaceWarningResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {WriteReplaceWarningResponseIEs} },

...

}

WriteReplaceWarningResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier				CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber					CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-BroadcastCompletedAreaList		CRITICALITY ignore	TYPE BroadcastCompletedAreaList			PRESENCE optional		}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PWS Cancel Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS CANCEL REQUEST

--

-- **************************************************************

PWSCancelRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSCancelRequestIEs} },

...

}

PWSCancelRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier			CRITICALITY reject	TYPE MessageIdentifier				PRESENCE mandatory	}|

{ ID id-SerialNumber				CRITICALITY reject	TYPE SerialNumber					PRESENCE mandatory	}|

{ ID id-WarningAreaList				CRITICALITY ignore	TYPE WarningAreaList				PRESENCE optional	}|

{ ID id-CancelAllWarningMessages	CRITICALITY reject	TYPE CancelAllWarningMessages		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- PWS CANCEL RESPONSE

--

-- **************************************************************

PWSCancelResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSCancelResponseIEs} },

...

}

PWSCancelResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier			CRITICALITY reject	TYPE MessageIdentifier				PRESENCE mandatory	}|

{ ID id-SerialNumber				CRITICALITY reject	TYPE SerialNumber					PRESENCE mandatory	}|

{ ID id-BroadcastCancelledAreaList	CRITICALITY ignore	TYPE BroadcastCancelledAreaList		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- PWS Restart Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS RESTART INDICATION

--

-- **************************************************************

PWSRestartIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSRestartIndicationIEs} },

...

}

PWSRestartIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-CellIDListForRestart			CRITICALITY reject	TYPE CellIDListForRestart				PRESENCE mandatory	}|

{ ID id-GlobalRANNodeID					CRITICALITY reject	TYPE GlobalRANNodeID					PRESENCE mandatory	}|

{ ID id-TAIListForRestart				CRITICALITY reject	TYPE TAIListForRestart					PRESENCE mandatory	}|

{ ID id-EmergencyAreaIDListForRestart	CRITICALITY reject	TYPE EmergencyAreaIDListForRestart	PRESENCE optional		},

...

}

-- **************************************************************

--

-- PWS Failure Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

-- PWS FAILURE INDICATION

--

-- **************************************************************

PWSFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSFailureIndicationIEs} },

...

}

PWSFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PWSFailedCellIDList			CRITICALITY reject	TYPE PWSFailedCellIDList	PRESENCE mandatory	}|

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- NRPPA TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- DOWNLINK UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

DownlinkUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkUEAssociatedNRPPaTransportIEs} },

...

}

DownlinkUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UPLINK UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

UplinkUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkUEAssociatedNRPPaTransportIEs} },

...

}

UplinkUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

DownlinkNonUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkNonUEAssociatedNRPPaTransportIEs} },

...

}

DownlinkNonUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UPLINK NON UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

UplinkNonUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkNonUEAssociatedNRPPaTransportIEs} },

...

}

UplinkNonUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation				CRITICALITY ignore	TYPE TraceActivation			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- TRACE FAILURE INDICATION

--

-- **************************************************************

TraceFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceFailureIndicationIEs} },

...

}

TraceFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-NGRANTraceID				CRITICALITY ignore	TYPE NGRANTraceID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-NGRANTraceID				CRITICALITY ignore	TYPE NGRANTraceID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- CELL TRAFFIC TRACE

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CellTrafficTraceIEs} },

...

}

CellTrafficTraceIEs NGAP-PROTOCOL-IES ::= {

{ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ID id-NGRANTraceID						CRITICALITY ignore	TYPE NGRANTraceID				PRESENCE mandatory	}|

{ID id-NGRAN-CGI						CRITICALITY ignore	TYPE NGRAN-CGI					PRESENCE mandatory	}|

{ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- LOCATION REPORTING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- LOCATION REPORTING CONTROL

--

-- **************************************************************

LocationReportingControl ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportingControlIEs} },

...

}

LocationReportingControlIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID						PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID						PRESENCE mandatory	}|

{ ID id-LocationReportingRequestType	CRITICALITY ignore	TYPE LocationReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- LOCATION REPORTING FAILURE INDICATION

--

-- **************************************************************

LocationReportingFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportingFailureIndicationIEs} },

...

}

LocationReportingFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- LOCATION REPORT

--

-- **************************************************************

LocationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportIEs} },

...

}

LocationReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID							PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID						CRITICALITY reject	TYPE RAN-UE-NGAP-ID							PRESENCE mandatory	}|

{ ID id-UserLocationInformation				CRITICALITY ignore	TYPE UserLocationInformation				PRESENCE mandatory	}|

{ ID id-UEPresenceInAreaOfInterestList		CRITICALITY ignore	TYPE UEPresenceInAreaOfInterestList		PRESENCE optional		}|

{ ID id-LocationReportingRequestType		CRITICALITY ignore	TYPE LocationReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE TNLA BINDING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UE TNLA BINDING RELEASE REQUEST

--

-- **************************************************************

UETNLABindingReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {UETNLABindingReleaseRequestIEs} },

...

}

UETNLABindingReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE RADIO CAPABILITY MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UE RADIO CAPABILITY INFO INDICATION

--

-- **************************************************************

UERadioCapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {UERadioCapabilityInfoIndicationIEs} },

...

}

UERadioCapabilityInfoIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE Radio Capability Check Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE RADIO CAPABILITY CHECK REQUEST

--

-- **************************************************************

UERadioCapabilityCheckRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UERadioCapabilityCheckRequestIEs} },

...

}

UERadioCapabilityCheckRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID			PRESENCE mandatory	}|

{ ID id-UERadioCapability		CRITICALITY ignore	TYPE UERadioCapability		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- UE RADIO CAPABILITY CHECK RESPONSE

--

-- **************************************************************

UERadioCapabilityCheckResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UERadioCapabilityCheckResponseIEs} },

...

}

UERadioCapabilityCheckResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY ignore	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY ignore	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-IMSVoiceSupportIndicator		CRITICALITY reject	TYPE IMSVoiceSupportIndicator		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container		{ { PrivateMessageIEs } },

...

}

PrivateMessageIEs NGAP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- DATA USAGE REPORTING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {SecondaryRATDataUsageReportIEs} },

...

}

SecondaryRATDataUsageReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSecondaryRATUsageList		CRITICALITY ignore	TYPE PDUSessionResourceSecondaryRATUsageList			PRESENCE mandatory	}|

{ ID id-HandoverFlag								CRITICALITY ignore	TYPE HandoverFlag												PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation								PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RIM INFORMATION TRANSFER ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UPLINK RIM INFORMATION TRANSFER

--

-- **************************************************************

UplinkRIMInformationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkRIMInformationTransferIEs} },

...

}

UplinkRIMInformationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RIMInformationTransfer	CRITICALITY ignore	TYPE RIMInformationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- DOWNLINK RIM INFORMATION TRANSFER

--

-- **************************************************************

DownlinkRIMInformationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkRIMInformationTransferIEs} },

...

}

DownlinkRIMInformationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RIMInformationTransfer	CRITICALITY ignore	TYPE RIMInformationTransfer	PRESENCE optional	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-Cause,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-ExtendedRATRestrictionInformation,

id-GUAMIType,

id-LastEUTRAN-PLMNIdentity,

id-LocationReportingAdditionalInfo,

id-MaximumIntegrityProtectedDataRate-DL,

id-NetworkInstance,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-PDUSessionAggregateMaximumBitRate,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PSCellInformation,

id-QosFlowAddOrModifyRequestList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-RAT-Information,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TargetRNC-ID,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-MDTConfiguration,

maxnoofAllowedAreas,

maxnoofAllowedS-NSSAIs,

maxnoofBPLMNs,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofForbTACs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNGConnectionsToReset,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofQosFlows,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofTACs,

maxnoofTAIforInactive,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofBluetoothName,

maxnoofWLANName,

maxnoofMDTPLMNs,

maxnoofSensorName

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

#### -- A

AdditionalDLUPTNLInformationForHOList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF AdditionalDLUPTNLInformationForHOItem

AdditionalDLUPTNLInformationForHOItem ::= SEQUENCE {

additionalDL-NGU-UP-TNLInformation				UPTransportLayerInformation,

additionalQosFlowSetupResponseList				QosFlowListWithDataForwarding,

additionalDLForwardingUPTNLInformation			UPTransportLayerInformation 							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalDLUPTNLInformationForHOItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalDLUPTNLInformationForHOItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalQosFlowInformation ::= ENUMERATED {

more-likely,

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevelARP				PriorityLevelARP,

pre-emptionCapability			Pre-emptionCapability,

pre-emptionVulnerability		Pre-emptionVulnerability,

iE-Extensions		ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllowedNSSAI ::= SEQUENCE (SIZE(1..maxnoofAllowedS-NSSAIs)) OF AllowedNSSAI-Item

AllowedNSSAI-Item ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {AllowedNSSAI-Item-ExtIEs} } OPTIONAL,

...

}

AllowedNSSAI-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

AMFName ::= PrintableString (SIZE(1..150, ...))

AMFPagingTarget ::= CHOICE {

globalRANNodeID			GlobalRANNodeID,

tAI						TAI,

choice-Extensions		ProtocolIE-SingleContainer { {AMFPagingTarget-ExtIEs} }

}

AMFPagingTarget-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AMFPointer ::= BIT STRING (SIZE(6))

AMFRegionID ::= BIT STRING (SIZE(8))

AMFSetID ::= BIT STRING (SIZE(10))

AMF-TNLAssociationSetupList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationSetupItem

AMF-TNLAssociationSetupItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationSetupItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-TNLAssociationToAddList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationToAddItem

AMF-TNLAssociationToAddItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

tNLAssociationUsage				TNLAssociationUsage										OPTIONAL,

tNLAddressWeightFactor			TNLAddressWeightFactor,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationToAddItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationToAddItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-TNLAssociationToRemoveList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationToRemoveItem

AMF-TNLAssociationToRemoveItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationToRemoveItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationToRemoveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressNGRAN	CRITICALITY reject	EXTENSION CPTransportLayerInformation	PRESENCE optional},

...

}

AMF-TNLAssociationToUpdateList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationToUpdateItem

AMF-TNLAssociationToUpdateItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

tNLAssociationUsage				TNLAssociationUsage											OPTIONAL,

tNLAddressWeightFactor			TNLAddressWeightFactor										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationToUpdateItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationToUpdateItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)

AreaOfInterest ::= SEQUENCE {

areaOfInterestTAIList			AreaOfInterestTAIList						OPTIONAL,

areaOfInterestCellList			AreaOfInterestCellList						OPTIONAL,

areaOfInterestRANNodeList		AreaOfInterestRANNodeList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterest-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterest-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestCellList ::= SEQUENCE (SIZE(1..maxnoofCellinAoI)) OF AreaOfInterestCellItem

AreaOfInterestCellItem ::= SEQUENCE {

nGRAN-CGI			NGRAN-CGI,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestCellItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestList ::= SEQUENCE (SIZE(1..maxnoofAoI)) OF AreaOfInterestItem

AreaOfInterestItem ::= SEQUENCE {

areaOfInterest						AreaOfInterest,

locationReportingReferenceID		LocationReportingReferenceID,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestRANNodeList ::= SEQUENCE (SIZE(1..maxnoofRANNodeinAoI)) OF AreaOfInterestRANNodeItem

AreaOfInterestRANNodeItem ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestRANNodeItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestRANNodeItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestTAIList ::= SEQUENCE (SIZE(1..maxnoofTAIinAoI)) OF AreaOfInterestTAIItem

AreaOfInterestTAIItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestTAIItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestTAIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForPaging ::= SEQUENCE {

assistanceDataForRecommendedCells		AssistanceDataForRecommendedCells			OPTIONAL,

pagingAttemptInformation				PagingAttemptInformation					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssistanceDataForPaging-ExtIEs} } OPTIONAL,

...

}

AssistanceDataForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForRecommendedCells ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

iE-Extensions		ProtocolExtensionContainer { {AssistanceDataForRecommendedCells-ExtIEs} }	OPTIONAL,

...

}

AssistanceDataForRecommendedCells-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedQosFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF AssociatedQosFlowItem

AssociatedQosFlowItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowMappingIndication		ENUMERATED {ul, dl, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssociatedQosFlowItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedQosFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow ::= INTEGER (0..4095, ...)

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

...

}

AreaScopeOfMDT-EUTRA ::= CHOICE {

cellBased					CellBasedMDT-EUTRA,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

...

}

#### -- B

BitRate	::= INTEGER (0..4000000000000, ...)

BroadcastCancelledAreaList ::= CHOICE {

cellIDCancelledEUTRA				CellIDCancelledEUTRA,

tAICancelledEUTRA					TAICancelledEUTRA,

emergencyAreaIDCancelledEUTRA		EmergencyAreaIDCancelledEUTRA,

cellIDCancelledNR					CellIDCancelledNR,

tAICancelledNR						TAICancelledNR,

emergencyAreaIDCancelledNR			EmergencyAreaIDCancelledNR,

choice-Extensions		ProtocolIE-SingleContainer { {BroadcastCancelledAreaList-ExtIEs} }

}

BroadcastCancelledAreaList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

BroadcastCompletedAreaList ::= CHOICE {

cellIDBroadcastEUTRA				CellIDBroadcastEUTRA,

tAIBroadcastEUTRA					TAIBroadcastEUTRA,

emergencyAreaIDBroadcastEUTRA		EmergencyAreaIDBroadcastEUTRA,

cellIDBroadcastNR					CellIDBroadcastNR,

tAIBroadcastNR						TAIBroadcastNR,

emergencyAreaIDBroadcastNR			EmergencyAreaIDBroadcastNR,

choice-Extensions		ProtocolIE-SingleContainer { {BroadcastCompletedAreaList-ExtIEs} }

}

BroadcastCompletedAreaList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

BroadcastPLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPLMNItem

BroadcastPLMNItem ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

tAISliceSupportList		SliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {BroadcastPLMNItem-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig             BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList     OPTIONAL,

bt-rssi                         ENUMERATED {true, ...}          OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothName

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))

#### -- C

CancelAllWarningMessages ::= ENUMERATED {

true,

...

}

CancelledCellsInEAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CancelledCellsInEAI-EUTRA-Item

CancelledCellsInEAI-EUTRA-Item ::= SEQUENCE {

eUTRA-CGI				EUTRA-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInEAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInEAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellsInEAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CancelledCellsInEAI-NR-Item

CancelledCellsInEAI-NR-Item ::= SEQUENCE {

nR-CGI					NR-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInEAI-NR-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInEAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellsInTAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CancelledCellsInTAI-EUTRA-Item

CancelledCellsInTAI-EUTRA-Item ::= SEQUENCE {

eUTRA-CGI				EUTRA-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInTAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInTAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellsInTAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CancelledCellsInTAI-NR-Item

CancelledCellsInTAI-NR-Item ::= SEQUENCE{

nR-CGI					NR-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInTAI-NR-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInTAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

nas					CauseNas,

protocol			CauseProtocol,

misc				CauseMisc,

choice-Extensions		ProtocolIE-SingleContainer { {Cause-ExtIEs} }

}

Cause-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unknown-PLMN,

unspecified,

...

}

CauseNas ::= ENUMERATED {

normal-release,

authentication-failure,

deregister,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

txnrelocoverall-expiry,

successful-handover,

release-due-to-ngran-generated-reason,

release-due-to-5gc-generated-reason,

handover-cancelled,

partial-handover,

ho-failure-in-target-5GC-ngran-node-or-target-system,

ho-target-not-allowed,

tngrelocoverall-expiry,

tngrelocprep-expiry,

cell-not-available,

unknown-targetID,

no-radio-resources-available-in-target-cell,

unknown-local-UE-NGAP-ID,

inconsistent-remote-UE-NGAP-ID,

handover-desirable-for-radio-reason,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

user-inactivity,

radio-connection-with-ue-lost,

radio-resources-not-available,

invalid-qos-combination,

failure-in-radio-interface-procedure,

interaction-with-other-procedure,

unknown-PDU-session-ID,

unkown-qos-flow-ID,

multiple-PDU-session-ID-instances,

multiple-qos-flow-ID-instances,

encryption-and-or-integrity-protection-algorithms-not-supported,

ng-intra-system-handover-triggered,

ng-inter-system-handover-triggered,

xn-handover-triggered,

not-supported-5QI-value,

ue-context-transfer,

ims-voice-eps-fallback-or-rat-fallback-triggered,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

slice-not-supported,

ue-in-rrc-inactive-state-not-reachable,

redirection,

resources-not-available-for-the-slice,

ue-max-integrity-protected-data-rate-reason,

release-due-to-cn-detected-mobility,

...,

n26-interface-not-available,

release-due-to-pre-emption,

multiple-location-reporting-reference-ID-instances

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CellIDBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDBroadcastEUTRA-Item

CellIDBroadcastEUTRA-Item ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {CellIDBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CellIDBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDBroadcastNR-Item

CellIDBroadcastNR-Item ::= SEQUENCE {

nR-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {CellIDBroadcastNR-Item-ExtIEs} } OPTIONAL,

...

}

CellIDBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDCancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDCancelledEUTRA-Item

CellIDCancelledEUTRA-Item ::= SEQUENCE {

eUTRA-CGI				EUTRA-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CellIDCancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CellIDCancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDCancelledNR ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDCancelledNR-Item

CellIDCancelledNR-Item ::= SEQUENCE {

nR-CGI					NR-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CellIDCancelledNR-Item-ExtIEs} } OPTIONAL,

...

}

CellIDCancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDListForRestart ::= CHOICE {

eUTRA-CGIListforRestart		EUTRA-CGIList,

nR-CGIListforRestart		NR-CGIList,

choice-Extensions		ProtocolIE-SingleContainer { {CellIDListForRestart-ExtIEs} }

}

CellIDListForRestart-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CellSize ::= ENUMERATED {verysmall, small, medium, large, ...}

CellType ::= SEQUENCE {

cellSize		CellSize,

iE-Extensions		ProtocolExtensionContainer { {CellType-ExtIEs} }	OPTIONAL,

...

}

CellType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CNAssistedRANTuning ::= SEQUENCE {

expectedUEBehaviour					ExpectedUEBehaviour												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {CNAssistedRANTuning-ExtIEs} }	OPTIONAL,

...

}

CNAssistedRANTuning-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CNTypeRestrictionsForEquivalent ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF CNTypeRestrictionsForEquivalentItem

CNTypeRestrictionsForEquivalentItem ::= SEQUENCE {

plmnIdentity						PLMNIdentity,

cn-Type								ENUMERATED {epc-forbidden, fiveGC-forbidden, ...},

iE-Extensions						ProtocolExtensionContainer { {CNTypeRestrictionsForEquivalentItem-ExtIEs} }				OPTIONAL,

...

}

CNTypeRestrictionsForEquivalentItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::={

...

}

CNTypeRestrictionsForServing ::= ENUMERATED {

epc-forbidden,

...

}

CommonNetworkInstance ::= OCTET STRING

CompletedCellsInEAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CompletedCellsInEAI-EUTRA-Item

CompletedCellsInEAI-EUTRA-Item ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInEAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellsInEAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellsInEAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CompletedCellsInEAI-NR-Item

CompletedCellsInEAI-NR-Item ::= SEQUENCE {

nR-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInEAI-NR-Item-ExtIEs} }	OPTIONAL,

...

}

CompletedCellsInEAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellsInTAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CompletedCellsInTAI-EUTRA-Item

CompletedCellsInTAI-EUTRA-Item ::= SEQUENCE{

eUTRA-CGI			EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInTAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellsInTAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellsInTAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CompletedCellsInTAI-NR-Item

CompletedCellsInTAI-NR-Item ::= SEQUENCE{

nR-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInTAI-NR-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellsInTAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ConcurrentWarningMessageInd ::= ENUMERATED {

true,

...

}

ConfidentialityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

CoreNetworkAssistanceInformationForInactive ::= SEQUENCE {

uEIdentityIndexValue				UEIdentityIndexValue,

uESpecificDRX						PagingDRX													OPTIONAL,

periodicRegistrationUpdateTimer		PeriodicRegistrationUpdateTimer,

mICOModeIndication					MICOModeIndication											OPTIONAL,

tAIListForInactive					TAIListForInactive,

expectedUEBehaviour					ExpectedUEBehaviour											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {CoreNetworkAssistanceInformationForInactive-ExtIEs} }	OPTIONAL,

...

}

CoreNetworkAssistanceInformationForInactive-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueForPDCP-SN12 ::= SEQUENCE {

pDCP-SN12			INTEGER (0..4095),

hFN-PDCP-SN12		INTEGER (0..1048575),

iE-Extensions		ProtocolExtensionContainer { {COUNTValueForPDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNTValueForPDCP-SN12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueForPDCP-SN18 ::= SEQUENCE {

pDCP-SN18			INTEGER (0..262143),

hFN-PDCP-SN18		INTEGER (0..16383),

iE-Extensions		ProtocolExtensionContainer { {COUNTValueForPDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNTValueForPDCP-SN18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress		TransportLayerAddress,

choice-Extensions		ProtocolIE-SingleContainer { {CPTransportLayerInformation-ExtIEs} }

}

CPTransportLayerInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-EndpointIPAddressAndPort		CRITICALITY reject	TYPE EndpointIPAddressAndPort		PRESENCE mandatory},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode											OPTIONAL,

triggeringMessage				TriggeringMessage										OPTIONAL,

procedureCriticality			Criticality												OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE(1..maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality		Criticality,

iE-ID				ProtocolIE-ID,

typeOfError			TypeOfError,

iE-Extensions		ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}} OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellBasedMDT-NR::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT-NR,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-NR-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-NR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-NR ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF NR-CGI

CellBasedMDT-EUTRA::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-EUTRA-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-EUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF EUTRA-CGI

#### -- D

DataCodingScheme ::= BIT STRING (SIZE(8))

DataForwardingAccepted ::= ENUMERATED {

data-forwarding-accepted,

...

}

DataForwardingNotPossible ::= ENUMERATED {

data-forwarding-not-possible,

...

}

DataForwardingResponseDRBList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

dRB-ID								DRB-ID,

dLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

uLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{DataForwardingResponseDRBItem-ExtIEs}}		OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingResponseERABList ::= SEQUENCE (SIZE(1..maxnoofE-RABs)) OF DataForwardingResponseERABListItem

DataForwardingResponseERABListItem ::= SEQUENCE {

e-RAB-ID			E-RAB-ID,

dLForwarding			DLForwarding																	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {DataForwardingResponseERABListItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingResponseERABListItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DelayCritical ::= ENUMERATED {

delay-critical,

non-delay-critical,

...

}

DLForwarding ::= ENUMERATED {

dl-forwarding-proposed,

...

}

DL-NGU-TNLInformationReused ::= ENUMERATED {

true,

...

}

DirectForwardingPathAvailability ::= ENUMERATED {

direct-path-available,

...

}

DRB-ID ::= INTEGER (1..32, ...)

DRBsSubjectToStatusTransferList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsSubjectToStatusTransferItem

DRBsSubjectToStatusTransferItem ::= SEQUENCE {

dRB-ID				DRB-ID,

dRBStatusUL			DRBStatusUL,

dRBStatusDL			DRBStatusDL,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransferItem-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransferItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-OldAssociatedQosFlowList-ULendmarkerexpected	CRITICALITY reject EXTENSION AssociatedQosFlowList	 PRESENCE optional },

...

}

DRBStatusDL ::= CHOICE {

dRBStatusDL12			DRBStatusDL12,

dRBStatusDL18			DRBStatusDL18,

choice-Extensions		ProtocolIE-SingleContainer { {DRBStatusDL-ExtIEs} }

}

DRBStatusDL-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

DRBStatusDL12 ::= SEQUENCE {

dL-COUNTValue		COUNTValueForPDCP-SN12,

iE-Extension		ProtocolExtensionContainer { {DRBStatusDL12-ExtIEs} }		OPTIONAL,

...

}

DRBStatusDL12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBStatusDL18 ::= SEQUENCE {

dL-COUNTValue		COUNTValueForPDCP-SN18,

iE-Extension		ProtocolExtensionContainer { {DRBStatusDL18-ExtIEs} }		OPTIONAL,

...

}

DRBStatusDL18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBStatusUL ::= CHOICE {

dRBStatusUL12			DRBStatusUL12,

dRBStatusUL18			DRBStatusUL18,

choice-Extensions		ProtocolIE-SingleContainer { {DRBStatusUL-ExtIEs} }

}

DRBStatusUL-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

DRBStatusUL12 ::= SEQUENCE {

uL-COUNTValue					COUNTValueForPDCP-SN12,

receiveStatusOfUL-PDCP-SDUs		BIT STRING (SIZE(1..2048))						OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBStatusUL12-ExtIEs} }		OPTIONAL,

...

}

DRBStatusUL12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBStatusUL18 ::= SEQUENCE {

uL-COUNTValue					COUNTValueForPDCP-SN18,

receiveStatusOfUL-PDCP-SDUs		BIT STRING (SIZE(1..131072))					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBStatusUL18-ExtIEs} }		OPTIONAL,

...

}

DRBStatusUL18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToQosFlowsMappingList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToQosFlowsMappingItem

DRBsToQosFlowsMappingItem ::= SEQUENCE {

dRB-ID								DRB-ID,

associatedQosFlowList				AssociatedQosFlowList,

iE-Extensions		ProtocolExtensionContainer { {DRBsToQosFlowsMappingItem-ExtIEs} }		OPTIONAL,

...

}

DRBsToQosFlowsMappingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Dynamic5QIDescriptor ::= SEQUENCE {

priorityLevelQos			PriorityLevelQos,

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						FiveQI													OPTIONAL,

delayCritical				DelayCritical											OPTIONAL,

-- The above IE shall be present in case of GBR QoS flow

averagingWindow				AveragingWindow											OPTIONAL,

-- The above IE shall be present in case of GBR QoS flow

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs} }	OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

EmergencyAreaID ::= OCTET STRING (SIZE(3))

EmergencyAreaIDBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDBroadcastEUTRA-Item

EmergencyAreaIDBroadcastEUTRA-Item ::= SEQUENCE {

emergencyAreaID					EmergencyAreaID,

completedCellsInEAI-EUTRA		CompletedCellsInEAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

EmergencyAreaIDBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDBroadcastNR-Item

EmergencyAreaIDBroadcastNR-Item ::= SEQUENCE {

emergencyAreaID				EmergencyAreaID,

completedCellsInEAI-NR		CompletedCellsInEAI-NR,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDBroadcastNR-Item-ExtIEs} }	OPTIONAL,

...

}

EmergencyAreaIDBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDCancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDCancelledEUTRA-Item

EmergencyAreaIDCancelledEUTRA-Item ::= SEQUENCE {

emergencyAreaID					EmergencyAreaID,

cancelledCellsInEAI-EUTRA		CancelledCellsInEAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDCancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

EmergencyAreaIDCancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDCancelledNR ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDCancelledNR-Item

EmergencyAreaIDCancelledNR-Item ::= SEQUENCE {

emergencyAreaID				EmergencyAreaID,

cancelledCellsInEAI-NR		CancelledCellsInEAI-NR,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDCancelledNR-Item-ExtIEs} }	OPTIONAL,

...

}

EmergencyAreaIDCancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDList ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaID

EmergencyAreaIDListForRestart ::= SEQUENCE (SIZE(1..maxnoofEAIforRestart)) OF EmergencyAreaID

EmergencyFallbackIndicator ::= SEQUENCE {

emergencyFallbackRequestIndicator		EmergencyFallbackRequestIndicator,

emergencyServiceTargetCN				EmergencyServiceTargetCN						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {EmergencyFallbackIndicator-ExtIEs} }	OPTIONAL,

...

}

EmergencyFallbackIndicator-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyFallbackRequestIndicator ::= ENUMERATED {

emergency-fallback-requested,

...

}

EmergencyServiceTargetCN ::= ENUMERATED {

fiveGC,

epc,

...

}

EN-DCSONConfigurationTransfer ::= OCTET STRING

EndpointIPAddressAndPort ::=SEQUENCE {

endpointIPAddress TransportLayerAddress,

portNumber			PortNumber,

iE-Extensions					ProtocolExtensionContainer { { EndpointIPAddressAndPort-ExtIEs} } OPTIONAL

}

EndpointIPAddressAndPort-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EquivalentPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMNIdentity

EPS-TAC ::= OCTET STRING (SIZE(2))

EPS-TAI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

ePS-TAC				EPS-TAC,

iE-Extensions		ProtocolExtensionContainer { {EPS-TAI-ExtIEs} } OPTIONAL,

...

}

EPS-TAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-ID ::= INTEGER (0..15, ...)

E-RABInformationList ::= SEQUENCE (SIZE(1..maxnoofE-RABs)) OF E-RABInformationItem

E-RABInformationItem ::= SEQUENCE {

e-RAB-ID			E-RAB-ID,

dLForwarding		DLForwarding													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {E-RABInformationItem-ExtIEs} }	OPTIONAL,

...

}

E-RABInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRACellIdentity ::= BIT STRING (SIZE(28))

EUTRA-CGI ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

eUTRACellIdentity		EUTRACellIdentity,

iE-Extensions		ProtocolExtensionContainer { {EUTRA-CGI-ExtIEs} } OPTIONAL,

...

}

EUTRA-CGI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-CGIList ::= SEQUENCE (SIZE(1..maxnoofCellsinngeNB)) OF EUTRA-CGI

EUTRA-CGIListForWarning ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF EUTRA-CGI

EUTRAencryptionAlgorithms ::= BIT STRING (SIZE(16, ...))

EUTRAintegrityProtectionAlgorithms ::= BIT STRING (SIZE(16, ...))

EventType ::= ENUMERATED {

direct,

change-of-serve-cell,

ue-presence-in-area-of-interest,

stop-change-of-serve-cell,

stop-ue-presence-in-area-of-interest,

cancel-location-reporting-for-the-ue,

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod						ExpectedActivityPeriod						OPTIONAL,

expectedIdlePeriod							ExpectedIdlePeriod							OPTIONAL,

sourceOfUEActivityBehaviourInformation		SourceOfUEActivityBehaviourInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEBehaviour ::= SEQUENCE {

expectedUEActivityBehaviour		ExpectedUEActivityBehaviour 					OPTIONAL,

expectedHOInterval				ExpectedHOInterval		 						OPTIONAL,

expectedUEMobility				ExpectedUEMobility								OPTIONAL,

expectedUEMovingTrajectory		ExpectedUEMovingTrajectory						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEMobility ::= ENUMERATED {

stationary,

mobile,

...

}

ExpectedUEMovingTrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsUEMovingTrajectory)) OF ExpectedUEMovingTrajectoryItem

ExpectedUEMovingTrajectoryItem ::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

timeStayedInCell		INTEGER (0..4095)													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEMovingTrajectoryItem-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEMovingTrajectoryItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedRATRestrictionInformation ::= SEQUENCE {

primaryRATRestriction		BIT STRING (SIZE(8, ...)),

secondaryRATRestriction		BIT STRING (SIZE(8, ...)),

iE-Extensions		ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} }	OPTIONAL,

...

}

ExtendedRATRestrictionInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedRNC-ID					::= INTEGER (4096..65535)

EventTrigger::= CHOICE {

outOfCoverage				ENUMERATED {true, ...},

eventA2LoggedMDTConfig		EventA2LoggedMDTConfig,

...

}

EventA2LoggedMDTConfig ::= SEQUENCE {

a2Threshold					MeasurementThresholdA2LoggedMDT,

hysteresis					Hysteresis,

timeToTrigger				TimeToTrigger,

iE-Extensions				ProtocolExtensionContainer { { EventA2LoggedMDTConfig-ExtIEs} } OPTIONAL,

...

}

EventA2LoggedMDTConfig-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementThresholdA2LoggedMDT ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

...

}

#### -- F

FiveG-S-TMSI ::= SEQUENCE {

aMFSetID			AMFSetID,

aMFPointer			AMFPointer,

fiveG-TMSI			FiveG-TMSI,

iE-Extensions		ProtocolExtensionContainer { {FiveG-S-TMSI-ExtIEs} }	OPTIONAL,

...

}

FiveG-S-TMSI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-TMSI ::= OCTET STRING (SIZE(4))

FiveQI ::= INTEGER (0..255, ...)

ForbiddenAreaInformation ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenAreaInformation-Item

ForbiddenAreaInformation-Item ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

forbiddenTACs		ForbiddenTACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenAreaInformation-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenAreaInformation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF TAC

#### -- G

GBR-QosInformation ::= SEQUENCE {

maximumFlowBitRateDL		BitRate,

maximumFlowBitRateUL		BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			NotificationControl									OPTIONAL,

maximumPacketLossRateDL		PacketLossRate										OPTIONAL,

maximumPacketLossRateUL		PacketLossRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {GBR-QosInformation-ExtIEs} }	OPTIONAL,

...

}

GBR-QosInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...}

GlobalGNB-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

gNB-ID				GNB-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalGNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalGNB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalN3IWF-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

n3IWF-ID			N3IWF-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalN3IWF-ID-ExtIEs} } OPTIONAL,

...

}

GlobalN3IWF-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalNgENB-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

ngENB-ID			NgENB-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalNgENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalNgENB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalRANNodeID ::= CHOICE {

globalGNB-ID		GlobalGNB-ID,

globalNgENB-ID		GlobalNgENB-ID,

globalN3IWF-ID		GlobalN3IWF-ID,

choice-Extensions		ProtocolIE-SingleContainer { {GlobalRANNodeID-ExtIEs} }

}

GlobalRANNodeID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

GNB-ID ::= CHOICE {

gNB-ID		BIT STRING (SIZE(22..32)),

choice-Extensions		ProtocolIE-SingleContainer { {GNB-ID-ExtIEs} }

}

GNB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

GTP-TEID ::= OCTET STRING (SIZE(4))

GTPTunnel ::= SEQUENCE {

transportLayerAddress		TransportLayerAddress,

gTP-TEID					GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {GTPTunnel-ExtIEs} } OPTIONAL,

...

}

GTPTunnel-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

aMFRegionID			AMFRegionID,

aMFSetID			AMFSetID,

aMFPointer			AMFPointer,

iE-Extensions		ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,

...

}

GUAMI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMIType ::= ENUMERATED {native, mapped, ...}

#### -- H

HandoverCommandTransfer ::= SEQUENCE {

dLForwardingUP-TNLInformation		UPTransportLayerInformation						OPTIONAL,

qosFlowToBeForwardedList			QosFlowToBeForwardedList						OPTIONAL,

dataForwardingResponseDRBList		DataForwardingResponseDRBList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverCommandTransfer-ExtIEs} } OPTIONAL,

...

}

HandoverCommandTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDLForwardingUPTNLInformation	CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList		PRESENCE optional}|

{ ID id-ULForwardingUP-TNLInformation			CRITICALITY reject	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ ID id-AdditionalULForwardingUPTNLInformation	CRITICALITY reject	EXTENSION UPTransportLayerInformationList	PRESENCE optional}|

{ ID id-DataForwardingResponseERABList			CRITICALITY ignore	EXTENSION DataForwardingResponseERABList	PRESENCE optional},

...

}

HandoverFlag ::= ENUMERATED {

handover-preparation,

...

}

HandoverPreparationUnsuccessfulTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {HandoverPreparationUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverPreparationUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverRequestAcknowledgeTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dLForwardingUP-TNLInformation		UPTransportLayerInformation										OPTIONAL,

securityResult						SecurityResult													OPTIONAL,

qosFlowSetupResponseList			QosFlowListWithDataForwarding,

qosFlowFailedToSetupList			QosFlowListWithCause											OPTIONAL,

dataForwardingResponseDRBList		DataForwardingResponseDRBList									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverRequestAcknowledgeTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverRequestAcknowledgeTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDLUPTNLInformationForHOList		CRITICALITY ignore	EXTENSION AdditionalDLUPTNLInformationForHOList		PRESENCE optional	}|

{ ID id-ULForwardingUP-TNLInformation				CRITICALITY reject	EXTENSION UPTransportLayerInformation					PRESENCE optional}|

{ ID id-AdditionalULForwardingUPTNLInformation		CRITICALITY reject	EXTENSION UPTransportLayerInformationList				PRESENCE optional}|

{ ID id-DataForwardingResponseERABList				CRITICALITY ignore	EXTENSION DataForwardingResponseERABList 				PRESENCE optional},

...

}

HandoverRequiredTransfer ::= SEQUENCE {

directForwardingPathAvailability		DirectForwardingPathAvailability				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverRequiredTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverRequiredTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverResourceAllocationUnsuccessfulTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverResourceAllocationUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverResourceAllocationUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverType ::= ENUMERATED {

intra5gs,

fivegs-to-eps,

eps-to-5gs,

...,

fivegs-to-utran

}

Hysteresis ::=                      INTEGER (0..30)

#### -- I

IMSVoiceSupportIndicator ::= ENUMERATED {

supported,

not-supported,

...

}

IndexToRFSP ::= INTEGER (1..256, ...)

InfoOnRecommendedCellsAndRANNodesForPaging ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

recommendRANNodesForPaging		RecommendedRANNodesForPaging,

iE-Extensions		ProtocolExtensionContainer { {InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

IntendedNumberOfPagingAttempts ::= INTEGER (1..16, ...)

InterfacesToTrace ::= BIT STRING (SIZE(8))

ImmediateMDTNr ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1Configuration				M1Configuration		OPTIONAL,

m4Configuration				M4Configuration		OPTIONAL,

m5Configuration				M5Configuration		OPTIONAL,

m6Configuration				M6Configuration		OPTIONAL,

m7Configuration				M7Configuration		OPTIONAL,

m8Configuration				M8Configuration		OPTIONAL,

m9Configuration				M9Configuration		OPTIONAL,

mDT-Location-Info 			MDT-Location-Info 	OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDTNr-ExtIEs} } OPTIONAL,

...

}

ImmediateMDTNr-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- J

#### -- K

#### -- L

LAC	::= OCTET STRING (SIZE (2))

LAI ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

lAC					LAC,

iE-Extensions		ProtocolExtensionContainer { {LAI-ExtIEs} } OPTIONAL,

...

}

LAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedCellInformation ::= CHOICE {

nGRANCell		LastVisitedNGRANCellInformation,

eUTRANCell		LastVisitedEUTRANCellInformation,

uTRANCell		LastVisitedUTRANCellInformation,

gERANCell		LastVisitedGERANCellInformation,

choice-Extensions		ProtocolIE-SingleContainer { {LastVisitedCellInformation-ExtIEs} }

}

LastVisitedCellInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LastVisitedCellItem ::= SEQUENCE {

lastVisitedCellInformation		LastVisitedCellInformation,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedCellItem-ExtIEs} }	OPTIONAL,

...

}

LastVisitedCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation ::= OCTET STRING

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedUTRANCellInformation ::= OCTET STRING

LocationReportingAdditionalInfo ::= ENUMERATED {

includePSCell,

...

}

LocationReportingReferenceID ::= INTEGER (1..64, ...)

LocationReportingRequestType ::= SEQUENCE {

eventType										EventType,

reportArea										ReportArea,

areaOfInterestList								AreaOfInterestList								OPTIONAL,

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID					OPTIONAL,

-- The above IE shall be present if the event type is set to “stop reporting UE presence in the area of interest”

iE-Extensions		ProtocolExtensionContainer { {LocationReportingRequestType-ExtIEs} }		OPTIONAL,

...

}

LocationReportingRequestType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LocationReportingAdditionalInfo	CRITICALITY ignore	EXTENSION LocationReportingAdditionalInfo		PRESENCE optional },

...

}

LoggedMDTNr ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

loggedMDTTrigger					LoggedMDTTrigger,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration	OPTIONAL,

wLANMeasurementConfiguration		WLANMeasurementConfiguration		OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {LoggedMDTNr-ExtIEs} } OPTIONAL,

...

}

LoggedMDTNr-ExtIEs	NGAP-PROTOCOL-EXTENSION ::= {

...

}

LoggingInterval ::= ENUMERATED {ms128, ms256, ms512, ms1024, ms2048, ms3072, ms4096, ms6144}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120}

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

LoggedMDTTrigger ::= CHOICE{

periodical		NULL,

eventTrigger	EventTrigger,

...

}

#### -- M

MaskedIMEISV ::= BIT STRING (SIZE(64))

MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)

MessageIdentifier ::= BIT STRING (SIZE(16))

MaximumIntegrityProtectedDataRate ::= ENUMERATED {

bitrate64kbs,

maximum-UE-rate,

...

}

MICOModeIndication ::= ENUMERATED {

true,

...

}

MobilityRestrictionList ::= SEQUENCE {

servingPLMN					PLMNIdentity,

equivalentPLMNs				EquivalentPLMNs											OPTIONAL,

rATRestrictions				RATRestrictions											OPTIONAL,

forbiddenAreaInformation	ForbiddenAreaInformation								OPTIONAL,

serviceAreaInformation		ServiceAreaInformation									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MobilityRestrictionList-ExtIEs} }	OPTIONAL,

...

}

MobilityRestrictionList-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastEUTRAN-PLMNIdentity		CRITICALITY ignore	EXTENSION PLMNIdentity		PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForServing		CRITICALITY ignore	EXTENSION CNTypeRestrictionsForServing		PRESENCE optional}|

{ ID id-CNTypeRestrictionsForEquivalent		CRITICALITY ignore	EXTENSION CNTypeRestrictionsForEquivalent		PRESENCE optional},

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMNIdentity

MDT-Configuration ::= SEQUENCE {

mdt-Config-NR		MDT-Configuration-NR		OPTIONAL,

mdt-Config-EUTRA	MDT-Configuration-EUTRA		OPTIONAL,

...

}

MDT-Configuration-NR ::= SEQUENCE {

mdt-Activation		MDT-Activation,

areaScopeOfMDT		AreaScopeOfMDT-NR,

mDTModeNr			MDTModeNr,

signallingBasedMDTPLMNList MDTPLMNList,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-NR-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-NR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Configuration-EUTRA ::= SEQUENCE {

mdt-Activation		MDT-Activation,

areaScopeOfMDT		AreaScopeOfMDT-EUTRA,

mDTMode				MDTModeEutra,

signallingBasedMDTPLMNList MDTPLMNList,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-EUTRA-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-EUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Activation 	::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

logged-MDT-only,

...

}

MDTModeNr ::= CHOICE {

immediateMDTNr				ImmediateMDTNr,

loggedMDTNr					LoggedMDTNr,

...

}

MDTModeEutra ::= OCTET STRING

MeasurementsToActivate ::= BIT STRING(SIZE(8))

M1Configuration ::= SEQUENCE {

m1reportingTrigger			M1ReportingTrigger,

m1thresholdEventA2			M1ThresholdEventA2				OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting				OPTIONAL,

-- Included in case of periodic or event-triggered periodic reporting

iE-Extensions				ProtocolExtensionContainer { { M1Configuration-ExtIEs} } OPTIONAL,

...

}

M1Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

a2eventtriggered-periodic,

...

}

M1ThresholdEventA2 ::= SEQUENCE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

threshold-SINR				Threshold-SINR,

iE-Extensions			ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M3Configuration ::= SEQUENCE {

--FFS: m3period			M3period,

iE-Extensions		ProtocolExtensionContainer { { M3Configuration-ExtIEs} } OPTIONAL,

...

}

M3Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M3period ::= ENUMERATED {ms100, ms1000, ms10000, ...,ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, min1 }

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

m6delay-threshold	M6delay-threshold		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M6delay-threshold ::= ENUMERATED { ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, ... }

M6report-Interval ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, ... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

M8Configuration ::= SEQUENCE {

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration,

iE-Extensions		ProtocolExtensionContainer { { M8Configuration-ExtIEs} } 	OPTIONAL,

...

}

M8Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M9Configuration ::= SEQUENCE {

wLANMeasurementConfiguration		WLANMeasurementConfiguration,

iE-Extensions		ProtocolExtensionContainer { { M9Configuration-ExtIEs} } 	OPTIONAL,

...

}

M9Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Location-Info ::= BIT STRING (SIZE (8))

#### -- N

N3IWF-ID ::= CHOICE {

n3IWF-ID				BIT STRING (SIZE(16)),

choice-Extensions		ProtocolIE-SingleContainer { {N3IWF-ID-ExtIEs} }

}

N3IWF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NAS-PDU ::= OCTET STRING

NASSecurityParametersFromNGRAN ::= OCTET STRING

NetworkInstance ::= INTEGER (1..256, ...)

NewSecurityContextInd ::= ENUMERATED {

true,

...

}

NextHopChainingCount ::= INTEGER (0..7)

NextPagingAreaScope ::= ENUMERATED {

same,

changed,

...

}

NgENB-ID ::= CHOICE {

macroNgENB-ID			BIT STRING (SIZE(20)),

shortMacroNgENB-ID		BIT STRING (SIZE(18)),

longMacroNgENB-ID		BIT STRING (SIZE(21)),

choice-Extensions		ProtocolIE-SingleContainer { {NgENB-ID-ExtIEs} }

}

NgENB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NGRAN-CGI ::= CHOICE {

nR-CGI			NR-CGI,

eUTRA-CGI		EUTRA-CGI,

choice-Extensions		ProtocolIE-SingleContainer { {NGRAN-CGI-ExtIEs} }

}

NGRAN-CGI-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NGRAN-TNLAssociationToRemoveList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF NGRAN-TNLAssociationToRemoveItem

NGRAN-TNLAssociationToRemoveItem::= SEQUENCE {

tNLAssociationTransportLayerAddress		CPTransportLayerInformation,

tNLAssociationTransportLayerAddressAMF		CPTransportLayerInformation		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { NGRAN-TNLAssociationToRemoveItem-ExtIEs} } OPTIONAL

}

NGRAN-TNLAssociationToRemoveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRANTraceID ::= OCTET STRING (SIZE(8))

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQos			PriorityLevelQos										OPTIONAL,

averagingWindow				AveragingWindow											OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs} }	OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NotAllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

NotificationCause ::= ENUMERATED {

fulfilled,

not-fulfilled,

...

}

NotificationControl ::= ENUMERATED {

notification-requested,

...

}

NRCellIdentity ::= BIT STRING (SIZE(36))

NR-CGI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

nRCellIdentity		NRCellIdentity,

iE-Extensions		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } OPTIONAL,

...

}

NR-CGI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-CGIList ::= SEQUENCE (SIZE(1..maxnoofCellsingNB)) OF NR-CGI

NR-CGIListForWarning ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF NR-CGI

NRencryptionAlgorithms ::= BIT STRING (SIZE(16, ...))

NRintegrityProtectionAlgorithms ::= BIT STRING (SIZE(16, ...))

NRPPa-PDU ::= OCTET STRING

NumberOfBroadcasts ::= INTEGER (0..65535)

NumberOfBroadcastsRequested ::= INTEGER (0..65535)

#### -- O

OverloadAction ::= ENUMERATED {

reject-non-emergency-mo-dt,

reject-rrc-cr-signalling,

permit-emergency-sessions-and-mobile-terminated-services-only,

permit-high-priority-sessions-and-mobile-terminated-services-only,

...

}

OverloadResponse ::= CHOICE {

overloadAction			OverloadAction,

choice-Extensions		ProtocolIE-SingleContainer { {OverloadResponse-ExtIEs} }

}

OverloadResponse-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

OverloadStartNSSAIList ::= SEQUENCE (SIZE (1..maxnoofSliceItems)) OF OverloadStartNSSAIItem

OverloadStartNSSAIItem ::= SEQUENCE {

sliceOverloadList						SliceOverloadList,

sliceOverloadResponse					OverloadResponse								OPTIONAL,

sliceTrafficLoadReductionIndication		TrafficLoadReductionIndication				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {OverloadStartNSSAIItem-ExtIEs} }	OPTIONAL,

...

}

OverloadStartNSSAIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pERScalar		INTEGER (0..9, ...),

pERExponent		INTEGER (0..9, ...),

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PacketLossRate ::= INTEGER (0..1000, ...)

PagingAttemptInformation ::= SEQUENCE {

pagingAttemptCount					PagingAttemptCount,

intendedNumberOfPagingAttempts		IntendedNumberOfPagingAttempts,

nextPagingAreaScope					NextPagingAreaScope									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PagingAttemptInformation-ExtIEs} }	OPTIONAL,

...

}

PagingAttemptInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptCount ::= INTEGER (1..16, ...)

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingOrigin ::= ENUMERATED {

non-3gpp,

...

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PathSwitchRequestAcknowledgeTransfer ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation											OPTIONAL,

securityIndication				SecurityIndication													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestAcknowledgeTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestAcknowledgeTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalNGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList		PRESENCE optional	},

...

}

PathSwitchRequestSetupFailedTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestSetupFailedTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestSetupFailedTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PathSwitchRequestTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dL-NGU-TNLInformationReused			DL-NGU-TNLInformationReused							OPTIONAL,

userPlaneSecurityInformation		UserPlaneSecurityInformation						OPTIONAL,

qosFlowAcceptedList					QosFlowAcceptedList,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDLQosFlowPerTNLInformation	CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList	PRESENCE optional	},

...

}

PathSwitchRequestUnsuccessfulTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionAggregateMaximumBitRate ::= SEQUENCE {

pDUSessionAggregateMaximumBitRateDL		BitRate,

pDUSessionAggregateMaximumBitRateUL		BitRate,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

PDUSessionAggregateMaximumBitRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionID ::= INTEGER (0..255)

PDUSessionResourceAdmittedList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceAdmittedItem

PDUSessionResourceAdmittedItem ::= SEQUENCE {

pDUSessionID							PDUSessionID,

handoverRequestAcknowledgeTransfer		OCTET STRING (CONTAINING HandoverRequestAcknowledgeTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceAdmittedItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceAdmittedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToModifyListModCfm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToModifyItemModCfm

PDUSessionResourceFailedToModifyItemModCfm ::= SEQUENCE {

pDUSessionID												PDUSessionID,

pDUSessionResourceModifyIndicationUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyIndicationUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToModifyItemModCfm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToModifyItemModCfm-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToModifyListModRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToModifyItemModRes

PDUSessionResourceFailedToModifyItemModRes ::= SEQUENCE {

pDUSessionID										PDUSessionID,

pDUSessionResourceModifyUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToModifyItemModRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToModifyItemModRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListCxtFail ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemCxtFail

PDUSessionResourceFailedToSetupItemCxtFail ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceSetupUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemCxtFail-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemCxtFail-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListCxtRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemCxtRes

PDUSessionResourceFailedToSetupItemCxtRes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceSetupUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemCxtRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemCxtRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListHOAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemHOAck

PDUSessionResourceFailedToSetupItemHOAck ::= SEQUENCE {

pDUSessionID										PDUSessionID,

handoverResourceAllocationUnsuccessfulTransfer		OCTET STRING (CONTAINING HandoverResourceAllocationUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemHOAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemHOAck-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListPSReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemPSReq

PDUSessionResourceFailedToSetupItemPSReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestSetupFailedTransfer		OCTET STRING (CONTAINING PathSwitchRequestSetupFailedTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemPSReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemPSReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListSURes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemSURes

PDUSessionResourceFailedToSetupItemSURes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceSetupUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemSURes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemSURes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceHandoverList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceHandoverItem

PDUSessionResourceHandoverItem ::= SEQUENCE {

pDUSessionID						PDUSessionID,

handoverCommandTransfer				OCTET STRING (CONTAINING HandoverCommandTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceHandoverItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceHandoverItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceInformationList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceInformationItem

PDUSessionResourceInformationItem ::= SEQUENCE {

pDUSessionID					PDUSessionID,

qosFlowInformationList			QosFlowInformationList,

dRBsToQosFlowsMappingList		DRBsToQosFlowsMappingList										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceInformationItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceListCxtRelCpl ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemCxtRelCpl

PDUSessionResourceItemCxtRelCpl ::= SEQUENCE {

pDUSessionID		PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemCxtRelCpl-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemCxtRelCpl-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionResourceReleaseResponseTransfer	CRITICALITY ignore	EXTENSION OCTET STRING (CONTAINING PDUSessionResourceReleaseResponseTransfer)	PRESENCE optional	},

...

}

PDUSessionResourceListCxtRelReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemCxtRelReq

PDUSessionResourceItemCxtRelReq ::= SEQUENCE {

pDUSessionID		PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemCxtRelReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemCxtRelReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceListHORqd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemHORqd

PDUSessionResourceItemHORqd ::= SEQUENCE {

pDUSessionID							PDUSessionID,

handoverRequiredTransfer				OCTET STRING (CONTAINING HandoverRequiredTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemHORqd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemHORqd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyConfirmTransfer ::= SEQUENCE {

qosFlowModifyConfirmList			QosFlowModifyConfirmList,

uLNGU-UP-TNLInformation				UPTransportLayerInformation,

additionalNG-UUPTNLInformation		UPTransportLayerInformationPairList									OPTIONAL,

qosFlowFailedToModifyList			QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyConfirmTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyConfirmTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyIndicationUnsuccessfulTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyIndicationUnsuccessfulTransfer-ExtIEs} }		OPTIONAL,

...

}

PDUSessionResourceModifyIndicationUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyRequestTransferIEs} },

...

}

PDUSessionResourceModifyRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionAggregateMaximumBitRate	CRITICALITY reject	TYPE PDUSessionAggregateMaximumBitRate		PRESENCE	optional		}|

{ ID id-UL-NGU-UP-TNLModifyList				CRITICALITY reject	TYPE UL-NGU-UP-TNLModifyList					PRESENCE	optional		}|

{ ID id-NetworkInstance						CRITICALITY reject	TYPE NetworkInstance							PRESENCE optional		}|

{ ID id-QosFlowAddOrModifyRequestList		CRITICALITY reject	TYPE QosFlowAddOrModifyRequestList			PRESENCE optional		}|

{ ID id-QosFlowToReleaseList				CRITICALITY reject	TYPE QosFlowListWithCause						PRESENCE optional		}|

{ ID id-AdditionalUL-NGU-UP-TNLInformation	CRITICALITY reject	TYPE UPTransportLayerInformationList		PRESENCE optional		}|

{ ID id-CommonNetworkInstance				CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		},

...

}

PDUSessionResourceModifyResponseTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation				UPTransportLayerInformation											OPTIONAL,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation											OPTIONAL,

qosFlowAddOrModifyResponseList			QosFlowAddOrModifyResponseList										OPTIONAL,

additionalDLQosFlowPerTNLInformation	QosFlowPerTNLInformationList										OPTIONAL,

qosFlowFailedToAddOrModifyList			QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalNGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList	PRESENCE optional	},

...

}

PDUSessionResourceModifyIndicationTransfer ::= SEQUENCE {

dLQosFlowPerTNLInformation					QosFlowPerTNLInformation,

additionalDLQosFlowPerTNLInformation		QosFlowPerTNLInformationList 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyIndicationTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyIndicationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	}|

{ ID id-SecurityResult						CRITICALITY ignore	EXTENSION SecurityResult						PRESENCE optional	},

...

}

PDUSessionResourceModifyListModCfm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModCfm

PDUSessionResourceModifyItemModCfm ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionResourceModifyConfirmTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyConfirmTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModCfm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModCfm-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyListModInd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModInd

PDUSessionResourceModifyItemModInd ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceModifyIndicationTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyIndicationTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModInd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModInd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyListModReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModReq

PDUSessionResourceModifyItemModReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

nAS-PDU										NAS-PDU													OPTIONAL,

pDUSessionResourceModifyRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-S-NSSAI		CRITICALITY reject	EXTENSION S-NSSAI		PRESENCE optional	},

...

}

PDUSessionResourceModifyListModRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModRes

PDUSessionResourceModifyItemModRes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceModifyResponseTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyUnsuccessfulTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceNotifyItem

PDUSessionResourceNotifyItem ::= SEQUENCE {

pDUSessionID						PDUSessionID,

pDUSessionResourceNotifyTransfer	OCTET STRING (CONTAINING PDUSessionResourceNotifyTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceNotifyReleasedTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyReleasedTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyReleasedTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	},

...

}

PDUSessionResourceNotifyTransfer ::= SEQUENCE {

qosFlowNotifyList		QosFlowNotifyList														OPTIONAL,

qosFlowReleasedList		QosFlowListWithCause													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	},

...

}

PDUSessionResourceReleaseCommandTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleaseCommandTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleaseCommandTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListNot ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemNot

PDUSessionResourceReleasedItemNot ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceNotifyReleasedTransfer		OCTET STRING (CONTAINING PDUSessionResourceNotifyReleasedTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemNot-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemNot-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListPSAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemPSAck

PDUSessionResourceReleasedItemPSAck ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestUnsuccessfulTransfer		OCTET STRING (CONTAINING PathSwitchRequestUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemPSAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemPSAck-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListPSFail ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemPSFail

PDUSessionResourceReleasedItemPSFail ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestUnsuccessfulTransfer		OCTET STRING (CONTAINING PathSwitchRequestUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemPSFail-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemPSFail-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListRelRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemRelRes

PDUSessionResourceReleasedItemRelRes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceReleaseResponseTransfer		OCTET STRING (CONTAINING PDUSessionResourceReleaseResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemRelRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemRelRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleaseResponseTransfer ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleaseResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleaseResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	},

...

}

PDUSessionResourceSecondaryRATUsageList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSecondaryRATUsageItem

PDUSessionResourceSecondaryRATUsageItem ::= SEQUENCE {

pDUSessionID							PDUSessionID,

secondaryRATDataUsageReportTransfer		OCTET STRING (CONTAINING SecondaryRATDataUsageReportTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSecondaryRATUsageItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSecondaryRATUsageItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListCxtReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemCxtReq

PDUSessionResourceSetupItemCxtReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

nAS-PDU										NAS-PDU												OPTIONAL,

s-NSSAI										S-NSSAI,

pDUSessionResourceSetupRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemCxtReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemCxtReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListCxtRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemCxtRes

PDUSessionResourceSetupItemCxtRes ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionResourceSetupResponseTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemCxtRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemCxtRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListHOReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemHOReq

PDUSessionResourceSetupItemHOReq ::= SEQUENCE {

pDUSessionID				PDUSessionID,

s-NSSAI						S-NSSAI,

handoverRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemHOReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemHOReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListSUReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemSUReq

PDUSessionResourceSetupItemSUReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionNAS-PDU							NAS-PDU												OPTIONAL,

s-NSSAI										S-NSSAI,

pDUSessionResourceSetupRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemSUReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemSUReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListSURes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemSURes

PDUSessionResourceSetupItemSURes ::= SEQUENCE {

pDUSessionID											PDUSessionID,

pDUSessionResourceSetupResponseTransfer					OCTET STRING (CONTAINING PDUSessionResourceSetupResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemSURes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemSURes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceSetupRequestTransferIEs} },

...

}

PDUSessionResourceSetupRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionAggregateMaximumBitRate	CRITICALITY reject	TYPE PDUSessionAggregateMaximumBitRate		PRESENCE optional		}|

{ ID id-UL-NGU-UP-TNLInformation			CRITICALITY reject	TYPE UPTransportLayerInformation				PRESENCE mandatory	}|

{ ID id-AdditionalUL-NGU-UP-TNLInformation	CRITICALITY reject	TYPE UPTransportLayerInformationList		PRESENCE optional		}|

{ ID id-DataForwardingNotPossible			CRITICALITY reject	TYPE DataForwardingNotPossible				PRESENCE optional		}|

{ ID id-PDUSessionType						CRITICALITY reject	TYPE PDUSessionType								PRESENCE mandatory	}|

{ ID id-SecurityIndication					CRITICALITY reject	TYPE SecurityIndication							PRESENCE optional		}|

{ ID id-NetworkInstance						CRITICALITY reject	TYPE NetworkInstance							PRESENCE optional		}|

{ ID id-QosFlowSetupRequestList				CRITICALITY reject	TYPE QosFlowSetupRequestList					PRESENCE mandatory	}|

{ ID id-CommonNetworkInstance				CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-DirectForwardingPathAvailability	CRITICALITY ignore	TYPE DirectForwardingPathAvailability		PRESENCE optional	 	},

...

}

PDUSessionResourceSetupResponseTransfer ::= SEQUENCE {

dLQosFlowPerTNLInformation				QosFlowPerTNLInformation,

additionalDLQosFlowPerTNLInformation	QosFlowPerTNLInformationList										OPTIONAL,

securityResult							SecurityResult														OPTIONAL,

qosFlowFailedToSetupList				QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

PDUSessionResourceSetupResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupUnsuccessfulTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSwitchedList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSwitchedItem

PDUSessionResourceSwitchedItem ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestAcknowledgeTransfer		OCTET STRING (CONTAINING PathSwitchRequestAcknowledgeTransfer),

iE-Extensions		ProtocolExtensionContainer { { PDUSessionResourceSwitchedItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSwitchedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceToBeSwitchedDLList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceToBeSwitchedDLItem

PDUSessionResourceToBeSwitchedDLItem ::= SEQUENCE {

pDUSessionID					PDUSessionID,

pathSwitchRequestTransfer		OCTET STRING (CONTAINING PathSwitchRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { { PDUSessionResourceToBeSwitchedDLItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToBeSwitchedDLItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceToReleaseListHOCmd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceToReleaseItemHOCmd

PDUSessionResourceToReleaseItemHOCmd ::= SEQUENCE {

pDUSessionID								PDUSessionID,

handoverPreparationUnsuccessfulTransfer		OCTET STRING (CONTAINING HandoverPreparationUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceToReleaseItemHOCmd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToReleaseItemHOCmd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceToReleaseListRelCmd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceToReleaseItemRelCmd

PDUSessionResourceToReleaseItemRelCmd ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceReleaseCommandTransfer		OCTET STRING (CONTAINING PDUSessionResourceReleaseCommandTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceToReleaseItemRelCmd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToReleaseItemRelCmd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionType ::= ENUMERATED {

ipv4,

ipv6,

ipv4v6,

ethernet,

unstructured,

...

}

PDUSessionUsageReport ::= SEQUENCE {

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

pDUSessionTimedReportList			VolumeTimedReportList,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionUsageReport-ExtIEs} } OPTIONAL,

...

}

PDUSessionUsageReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PeriodicRegistrationUpdateTimer ::= BIT STRING (SIZE(8))

PLMNIdentity ::= OCTET STRING (SIZE(3))

PLMNSupportList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF PLMNSupportItem

PLMNSupportItem ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

sliceSupportList		SliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {PLMNSupportItem-ExtIEs} } OPTIONAL,

...

}

PLMNSupportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PortNumber ::= OCTET STRING (SIZE(2))

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption,

...

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable,

...

}

PriorityLevelARP ::= INTEGER (1..15)

PriorityLevelQos ::= INTEGER (1..127, ...)

PWSFailedCellIDList ::= CHOICE {

eUTRA-CGI-PWSFailedList		EUTRA-CGIList,

nR-CGI-PWSFailedList		NR-CGIList,

choice-Extensions		ProtocolIE-SingleContainer { {PWSFailedCellIDList-ExtIEs} }

}

PWSFailedCellIDList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

#### -- Q

QosCharacteristics ::= CHOICE {

nonDynamic5QI		NonDynamic5QIDescriptor,

dynamic5QI			Dynamic5QIDescriptor,

choice-Extensions		ProtocolIE-SingleContainer { {QosCharacteristics-ExtIEs} }

}

QosCharacteristics-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

QosFlowAcceptedList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAcceptedItem

QosFlowAcceptedItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAcceptedItem-ExtIEs} } OPTIONAL,

...

}

QosFlowAcceptedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowAddOrModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAddOrModifyRequestItem

QosFlowAddOrModifyRequestItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowLevelQosParameters		QosFlowLevelQosParameters									OPTIONAL,

e-RAB-ID						E-RAB-ID													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAddOrModifyRequestItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowAddOrModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowAddOrModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAddOrModifyResponseItem

QosFlowAddOrModifyResponseItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAddOrModifyResponseItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowAddOrModifyResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowIdentifier ::= INTEGER (0..63, ...)

QosFlowInformationList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowInformationItem

QosFlowInformationItem ::= SEQUENCE {

qosFlowIdentifier	QosFlowIdentifier,

dLForwarding		DLForwarding													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowInformationItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ULForwarding	CRITICALITY reject	EXTENSION ULForwarding	PRESENCE optional},

...

}

QosFlowLevelQosParameters ::= SEQUENCE {

qosCharacteristics					QosCharacteristics,

allocationAndRetentionPriority		AllocationAndRetentionPriority,

gBR-QosInformation					GBR-QosInformation									OPTIONAL,

reflectiveQosAttribute				ReflectiveQosAttribute								OPTIONAL,

additionalQosFlowInformation		AdditionalQosFlowInformation						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowLevelQosParameters-ExtIEs} }	OPTIONAL,

...

}

QosFlowLevelQosParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-QosMonitoringRequest	CRITICALITY ignore	EXTENSION QosMonitoringRequest	PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QosFlowListWithCause ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowWithCauseItem

QosFlowWithCauseItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

cause					Cause,

iE-Extensions		ProtocolExtensionContainer { {QosFlowWithCauseItem-ExtIEs} } OPTIONAL,

...

}

QosFlowWithCauseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowModifyConfirmList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowModifyConfirmItem

QosFlowModifyConfirmItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowModifyConfirmItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowModifyConfirmItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowNotifyList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowNotifyItem

QosFlowNotifyItem ::= SEQUENCE {

qosFlowIdentifier			QosFlowIdentifier,

notificationCause			NotificationCause,

iE-Extensions		ProtocolExtensionContainer { {QosFlowNotifyItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowNotifyItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowPerTNLInformation ::= SEQUENCE {

uPTransportLayerInformation		UPTransportLayerInformation,

associatedQosFlowList			AssociatedQosFlowList,

iE-Extensions		ProtocolExtensionContainer { { QosFlowPerTNLInformation-ExtIEs} }	OPTIONAL,

...

}

QosFlowPerTNLInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowPerTNLInformationList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF QosFlowPerTNLInformationItem

QosFlowPerTNLInformationItem ::= SEQUENCE {

qosFlowPerTNLInformation				QosFlowPerTNLInformation,

iE-Extensions		ProtocolExtensionContainer { { QosFlowPerTNLInformationItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowPerTNLInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowSetupRequestItem

QosFlowSetupRequestItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowLevelQosParameters		QosFlowLevelQosParameters,

e-RAB-ID						E-RAB-ID											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowSetupRequestItem-ExtIEs} } OPTIONAL,

...

}

QosFlowSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowListWithDataForwarding ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowItemWithDataForwarding

QosFlowItemWithDataForwarding ::= SEQUENCE {

qosFlowIdentifier			QosFlowIdentifier,

dataForwardingAccepted		DataForwardingAccepted											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowItemWithDataForwarding-ExtIEs} }	OPTIONAL,

...

}

QosFlowItemWithDataForwarding-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowToBeForwardedList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowToBeForwardedItem

QosFlowToBeForwardedItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowToBeForwardedItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowToBeForwardedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsUsageReportList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QoSFlowsUsageReport-Item

QoSFlowsUsageReport-Item ::= SEQUENCE {

qosFlowIdentifier					QosFlowIdentifier,

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

qoSFlowsTimedReportList				VolumeTimedReportList,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

QoSFlowsUsageReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- R

RANNodeName ::= PrintableString (SIZE(1..150, ...))

RANPagingPriority ::= INTEGER (1..256)

RANStatusTransfer-TransparentContainer ::= SEQUENCE {

dRBsSubjectToStatusTransferList		DRBsSubjectToStatusTransferList,

iE-Extensions		ProtocolExtensionContainer { {RANStatusTransfer-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

RANStatusTransfer-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RAN-UE-NGAP-ID ::= INTEGER (0..4294967295)

RAT-Information ::= ENUMERATED {

unlicensed,

...

}

RATRestrictions ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF RATRestrictions-Item

RATRestrictions-Item ::= SEQUENCE {

pLMNIdentity					PLMNIdentity,

rATRestrictionInformation		RATRestrictionInformation,

iE-Extensions		ProtocolExtensionContainer { {RATRestrictions-Item-ExtIEs} }		OPTIONAL,

...

}

RATRestrictions-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ExtendedRATRestrictionInformation		CRITICALITY ignore	EXTENSION ExtendedRATRestrictionInformation		PRESENCE optional},

...

}

RATRestrictionInformation ::= BIT STRING (SIZE(8, ...))

RecommendedCellsForPaging ::= SEQUENCE {

recommendedCellList			RecommendedCellList,

iE-Extensions		ProtocolExtensionContainer { {RecommendedCellsForPaging-ExtIEs} }	OPTIONAL,

...

}

RecommendedCellsForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedCellList ::= SEQUENCE (SIZE(1..maxnoofRecommendedCells)) OF RecommendedCellItem

RecommendedCellItem ::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

timeStayedInCell		INTEGER (0..4095)		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RecommendedCellItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedRANNodesForPaging ::= SEQUENCE {

recommendedRANNodeList		RecommendedRANNodeList,

iE-Extensions		ProtocolExtensionContainer { {RecommendedRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

RecommendedRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedRANNodeList::= SEQUENCE (SIZE(1..maxnoofRecommendedRANNodes)) OF RecommendedRANNodeItem

RecommendedRANNodeItem ::= SEQUENCE {

aMFPagingTarget		AMFPagingTarget,

iE-Extensions		ProtocolExtensionContainer { {RecommendedRANNodeItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedRANNodeItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RedirectionVoiceFallback ::= ENUMERATED {

possible,

not-possible,

...

}

ReflectiveQosAttribute ::= ENUMERATED {

subject-to,

...

}

RelativeAMFCapacity ::= INTEGER (0..255)

ReportArea ::= ENUMERATED {

cell,

...

}

RepetitionPeriod ::= INTEGER (0..131071)

ResetAll ::= ENUMERATED {

reset-all,

...

}

ResetType ::= CHOICE {

nG-Interface			ResetAll,

partOfNG-Interface		UE-associatedLogicalNG-connectionList,

choice-Extensions		ProtocolIE-SingleContainer { {ResetType-ExtIEs} }

}

ResetType-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

RNC-ID ::= INTEGER (0..4095)

RoutingID ::= OCTET STRING

RRCContainer ::= OCTET STRING

RRCEstablishmentCause ::= ENUMERATED {

emergency,

highPriorityAccess,

mt-Access,

mo-Signalling,

mo-Data,

mo-VoiceCall,

mo-VideoCall,

mo-SMS,

mps-PriorityAccess,

mcs-PriorityAccess,

...,

notAvailable

}

RRCInactiveTransitionReportRequest ::= ENUMERATED {

subsequent-state-transition-report,

single-rrc-connected-state-report,

cancel-report,

...

}

RRCState ::= ENUMERATED {

inactive,

connected,

...

}

RIMInformationTransfer ::= SEQUENCE {

targetRANNodeID				TargetRANNodeID,

sourceRANNodeID				SourceRANNodeID,

rIMInformation				RIMInformation,

iE-Extensions				ProtocolExtensionContainer { {RIMInformationTransfer-ExtIEs} }	OPTIONAL,

...

}

RIMInformationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RIMInformation	::= SEQUENCE		{

targetgNBSetID			GNBSetID,

rIM-RSDetection			ENUMERATED	{rs-detected, rs-disappeared, ...},

...

}

GNBSetID	::= BIT STRING (SIZE(22))

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, rinfinity}

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60}

#### -- S

SCTP-TLAs	::= SEQUENCE (SIZE(1..maxnoofXnTLAs)) OF TransportLayerAddress

SD ::= OCTET STRING (SIZE(3))

SecondaryRATUsageInformation ::= SEQUENCE {

pDUSessionUsageReport		PDUSessionUsageReport											OPTIONAL,

qosFlowsUsageReportList		QoSFlowsUsageReportList											OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATUsageInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SecondaryRATDataUsageReportTransfer ::= SEQUENCE {

secondaryRATUsageInformation		SecondaryRATUsageInformation									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SecondaryRATDataUsageReportTransfer-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATDataUsageReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityContext ::= SEQUENCE {

nextHopChainingCount		NextHopChainingCount,

nextHopNH					SecurityKey,

iE-Extensions		ProtocolExtensionContainer { {SecurityContext-ExtIEs} }	OPTIONAL,

...

}

SecurityContext-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			IntegrityProtectionIndication,

confidentialityProtectionIndication		ConfidentialityProtectionIndication,

maximumIntegrityProtectedDataRate-UL	MaximumIntegrityProtectedDataRate		OPTIONAL,

-- The above IE shall be present if integrity protection is required or preferred

iE-Extensions		ProtocolExtensionContainer { {SecurityIndication-ExtIEs} }	OPTIONAL,

...

}

SecurityIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumIntegrityProtectedDataRate-DL	CRITICALITY ignore	EXTENSION MaximumIntegrityProtectedDataRate	PRESENCE optional	},

...

}

SecurityKey	::= BIT STRING (SIZE(256))

SecurityResult ::= SEQUENCE {

integrityProtectionResult			IntegrityProtectionResult,

confidentialityProtectionResult		ConfidentialityProtectionResult,

iE-Extensions		ProtocolExtensionContainer { {SecurityResult-ExtIEs} }	OPTIONAL,

...

}

SecurityResult-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SerialNumber ::= BIT STRING (SIZE(16))

ServedGUAMIList ::= SEQUENCE (SIZE(1..maxnoofServedGUAMIs)) OF ServedGUAMIItem

ServedGUAMIItem ::= SEQUENCE {

gUAMI				GUAMI,

backupAMFName		AMFName													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedGUAMIItem-ExtIEs} }	OPTIONAL,

...

}

ServedGUAMIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-GUAMIType		CRITICALITY ignore	EXTENSION GUAMIType		PRESENCE optional},

...

}

ServiceAreaInformation ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ServiceAreaInformation-Item

ServiceAreaInformation-Item ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

allowedTACs			AllowedTACs																OPTIONAL,

notAllowedTACs		NotAllowedTACs															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServiceAreaInformation-Item-ExtIEs} }		OPTIONAL,

...

}

ServiceAreaInformation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SgNB-UE-X2AP-ID ::= INTEGER (0..4294967295)

SliceOverloadList ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SliceOverloadItem

SliceOverloadItem ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {SliceOverloadItem-ExtIEs} }	OPTIONAL,

...

}

SliceOverloadItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupportList ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {SliceSupportItem-ExtIEs} }	OPTIONAL,

...

}

SliceSupportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

S-NSSAI ::= SEQUENCE {

sST					SST,

sD					SD													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { S-NSSAI-ExtIEs} }	OPTIONAL,

...

}

S-NSSAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONConfigurationTransfer ::= SEQUENCE {

targetRANNodeID				TargetRANNodeID,

sourceRANNodeID				SourceRANNodeID,

sONInformation				SONInformation,

xnTNLConfigurationInfo		XnTNLConfigurationInfo										OPTIONAL,

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE set to “Xn TNL Configuration Info”

iE-Extensions		ProtocolExtensionContainer { {SONConfigurationTransfer-ExtIEs} }	OPTIONAL,

...

}

SONConfigurationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONInformation ::= CHOICE {

sONInformationRequest		SONInformationRequest,

sONInformationReply			SONInformationReply,

choice-Extensions		ProtocolIE-SingleContainer { {SONInformation-ExtIEs} }

}

SONInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

SONInformationReply ::= SEQUENCE {

xnTNLConfigurationInfo		XnTNLConfigurationInfo									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SONInformationReply-ExtIEs} }		OPTIONAL,

...

}

SONInformationReply-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONInformationRequest ::= ENUMERATED {

xn-TNL-configuration-info,

...

}

SourceNGRANNode-ToTargetNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer							RRCContainer,

pDUSessionResourceInformationList		PDUSessionResourceInformationList													OPTIONAL,

e-RABInformationList					E-RABInformationList																OPTIONAL,

targetCell-ID							NGRAN-CGI,

indexToRFSP								IndexToRFSP																			OPTIONAL,

uEHistoryInformation					UEHistoryInformation,

iE-Extensions		ProtocolExtensionContainer { {SourceNGRANNode-ToTargetNGRANNode-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

SourceNGRANNode-ToTargetNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SgNB-UE-X2AP-ID	CRITICALITY ignore	EXTENSION SgNB-UE-X2AP-ID 	PRESENCE optional},

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

SourceRANNodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {SourceRANNodeID-ExtIEs} } OPTIONAL,

...

}

SourceRANNodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SourceToTarget-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the source RAN node to the target RAN node.

-- The octets of the OCTET STRING are encoded according to the specifications of the target system.

SourceToTarget-AMFInformationReroute ::= SEQUENCE {

configuredNSSAI						ConfiguredNSSAI													OPTIONAL,

rejectedNSSAIinPLMN					RejectedNSSAIinPLMN												OPTIONAL,

rejectedNSSAIinTA						RejectedNSSAIinTA													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SourceToTarget-AMFInformationReroute-ExtIEs} }	OPTIONAL,

...

}

SourceToTarget-AMFInformationReroute-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- This IE includes information from the source Core node to the target Core node for reroute information provide by NSSF.

-- The octets of the OCTET STRING are encoded according to the specifications of the Core network.

SRVCCOperationPossible ::= ENUMERATED {

possible, notPossible,

...

}

ConfiguredNSSAI  ::=  OCTET STRING (SIZE(128))

RejectedNSSAIinPLMN ::= OCTET STRING (SIZE(32))

RejectedNSSAIinTA ::= OCTET STRING (SIZE(32))

SST ::= OCTET STRING (SIZE(1))

SupportedTAList ::= SEQUENCE (SIZE(1..maxnoofTACs)) OF SupportedTAItem

SupportedTAItem ::= SEQUENCE {

tAC									TAC,

broadcastPLMNList		BroadcastPLMNList,

iE-Extensions		ProtocolExtensionContainer { {SupportedTAItem-ExtIEs} } OPTIONAL,

...

}

SupportedTAItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-RAT-Information		CRITICALITY reject	EXTENSION RAT-Information		PRESENCE optional},

...

}

SensorMeasurementConfiguration ::=	SEQUENCE {

sensorMeasConfig            SensorMeasConfig,

sensorMeasConfigName		SensorMeasConfigName            OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SensorMeasurementConfiguration-ExtIEs} } OPTIONAL,

...

}

SensorMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasConfigName ::= SEQUENCE (SIZE(1..maxnoofSensorName)) OF SensorNameConfig

SensorMeasConfig::= ENUMERATED {setup,...}

SensorNameConfig ::= CHOICE {

uncompensattedBarometericConfig		ENUMERATED {true, ...},

ueSpeedConfig						ENUMERATED {true, ...},

ueOrientationConfig					ENUMERATED {true, ...},

...

}

#### -- T

TAC ::= OCTET STRING (SIZE(3))

TAI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

TAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAIBroadcastEUTRA-Item

TAIBroadcastEUTRA-Item ::= SEQUENCE {

tAI								TAI,

completedCellsInTAI-EUTRA		CompletedCellsInTAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {TAIBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

TAIBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAIBroadcastNR-Item

TAIBroadcastNR-Item ::= SEQUENCE {

tAI							TAI,

completedCellsInTAI-NR		CompletedCellsInTAI-NR,

iE-Extensions		ProtocolExtensionContainer { {TAIBroadcastNR-Item-ExtIEs} } OPTIONAL,

...

}

TAIBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAICancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAICancelledEUTRA-Item

TAICancelledEUTRA-Item ::= SEQUENCE {

tAI								TAI,

cancelledCellsInTAI-EUTRA		CancelledCellsInTAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {TAICancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

TAICancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAICancelledNR ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAICancelledNR-Item

TAICancelledNR-Item ::= SEQUENCE {

tAI							TAI,

cancelledCellsInTAI-NR		CancelledCellsInTAI-NR,

iE-Extensions		ProtocolExtensionContainer { {TAICancelledNR-Item-ExtIEs} } OPTIONAL,

...

}

TAICancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForInactive ::= SEQUENCE (SIZE(1..maxnoofTAIforInactive)) OF TAIListForInactiveItem

TAIListForInactiveItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {TAIListForInactiveItem-ExtIEs} } OPTIONAL,

...

}

TAIListForInactiveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForPaging ::= SEQUENCE (SIZE(1..maxnoofTAIforPaging)) OF TAIListForPagingItem

TAIListForPagingItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {TAIListForPagingItem-ExtIEs} } OPTIONAL,

...

}

TAIListForPagingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForRestart ::= SEQUENCE (SIZE(1..maxnoofTAIforRestart)) OF TAI

TAIListForWarning ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI

TargeteNB-ID ::= SEQUENCE {

globalENB-ID		GlobalNgENB-ID,

selected-EPS-TAI	EPS-TAI,

iE-Extensions		ProtocolExtensionContainer { {TargeteNB-ID-ExtIEs} } OPTIONAL,

...

}

TargeteNB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetID ::= CHOICE {

targetRANNodeID		TargetRANNodeID,

targeteNB-ID		TargeteNB-ID,

choice-Extensions		ProtocolIE-SingleContainer { {TargetID-ExtIEs} }

}

TargetID-ExtIEs NGAP-PROTOCOL-IES ::= {

{ID id-TargetRNC-ID		CRITICALITY reject	TYPE TargetRNC-ID PRESENCE mandatory },

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer		RRCContainer,

iE-Extensions		ProtocolExtensionContainer { {TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetRANNodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetRANNodeID-ExtIEs} } OPTIONAL,

...

}

TargetRANNodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetRNC-ID ::= SEQUENCE {

lAI					LAI,

rNC-ID				RNC-ID,

extendedRNC-ID		ExtendedRNC-ID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TargetRNC-ID-ExtIEs} } OPTIONAL,

...

}

TargetRNC-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetToSource-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the target RAN node to the source RAN node.

-- The octets of the OCTET STRING are encoded according to the specifications of the target system.

TimerApproachForGUAMIRemoval ::= ENUMERATED {

apply-timer,

...

}

TimeStamp ::= OCTET STRING (SIZE(4))

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TimeUEStayedInCell ::= INTEGER (0..4095)

TimeUEStayedInCellEnhancedGranularity ::= INTEGER (0..40950)

TNLAddressWeightFactor ::= INTEGER (0..255)

TNLAssociationList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLAssociationItem

TNLAssociationItem ::= SEQUENCE {

tNLAssociationAddress		CPTransportLayerInformation,

cause						Cause,

iE-Extensions		ProtocolExtensionContainer { {TNLAssociationItem-ExtIEs} } OPTIONAL,

...

}

TNLAssociationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TraceActivation ::= SEQUENCE {

nGRANTraceID						NGRANTraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MDTConfiguration	CRITICALITY ignore	EXTENSION MDT-Configuration		PRESENCE optional },

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TrafficLoadReductionIndication ::= INTEGER (1..99)

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Threshold-RSRP ::= INTEGER(0..127)

Threshold-RSRQ ::= INTEGER(0..127)

Threshold-SINR ::= INTEGER(0..127)

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAI

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TimeToTrigger ::= ENUMERATED {ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEAggregateMaximumBitRateDL		BitRate,

uEAggregateMaximumBitRateUL		BitRate,

iE-Extensions		ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UEAggregateMaximumBitRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UE-associatedLogicalNG-connectionList ::= SEQUENCE (SIZE(1..maxnoofNGConnectionsToReset)) OF UE-associatedLogicalNG-connectionItem

UE-associatedLogicalNG-connectionItem ::= SEQUENCE {

aMF-UE-NGAP-ID		AMF-UE-NGAP-ID																		OPTIONAL,

rAN-UE-NGAP-ID		RAN-UE-NGAP-ID																		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UE-associatedLogicalNG-connectionItem-ExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalNG-connectionItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...}

UEContextRequest ::= ENUMERATED {requested, ...}

UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCellItem

UEIdentityIndexValue ::= CHOICE {

indexLength10			BIT STRING (SIZE(10)),

choice-Extensions		ProtocolIE-SingleContainer { {UEIdentityIndexValue-ExtIEs} }

}

UEIdentityIndexValue-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UE-NGAP-IDs ::= CHOICE {

uE-NGAP-ID-pair		UE-NGAP-ID-pair,

aMF-UE-NGAP-ID		AMF-UE-NGAP-ID,

choice-Extensions		ProtocolIE-SingleContainer { {UE-NGAP-IDs-ExtIEs} }

}

UE-NGAP-IDs-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UE-NGAP-ID-pair ::= SEQUENCE{

aMF-UE-NGAP-ID		AMF-UE-NGAP-ID,

rAN-UE-NGAP-ID		RAN-UE-NGAP-ID,

iE-Extensions		ProtocolExtensionContainer { {UE-NGAP-ID-pair-ExtIEs} } OPTIONAL,

...

}

UE-NGAP-ID-pair-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UEPagingIdentity ::= CHOICE {

fiveG-S-TMSI		FiveG-S-TMSI,

choice-Extensions		ProtocolIE-SingleContainer { {UEPagingIdentity-ExtIEs} }

}

UEPagingIdentity-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UEPresence ::= ENUMERATED {in, out, unknown, ...}

UEPresenceInAreaOfInterestList ::= SEQUENCE (SIZE(1..maxnoofAoI)) OF UEPresenceInAreaOfInterestItem

UEPresenceInAreaOfInterestItem ::= SEQUENCE {

locationReportingReferenceID		LocationReportingReferenceID,

uEPresence							UEPresence,

iE-Extensions		ProtocolExtensionContainer { {UEPresenceInAreaOfInterestItem-ExtIEs} }	OPTIONAL,

...

}

UEPresenceInAreaOfInterestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UERadioCapability ::= OCTET STRING

UERadioCapabilityForPaging ::= SEQUENCE {

uERadioCapabilityForPagingOfNR		UERadioCapabilityForPagingOfNR			OPTIONAL,

uERadioCapabilityForPagingOfEUTRA		UERadioCapabilityForPagingOfEUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UERadioCapabilityForPaging-ExtIEs} }	OPTIONAL,

...

}

UERadioCapabilityForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UERadioCapabilityForPagingOfNR ::= OCTET STRING

UERadioCapabilityForPagingOfEUTRA ::= OCTET STRING

UERetentionInformation ::= ENUMERATED {

ues-retained,

...

}

UESecurityCapabilities ::= SEQUENCE {

nRencryptionAlgorithms					NRencryptionAlgorithms,

nRintegrityProtectionAlgorithms			NRintegrityProtectionAlgorithms,

eUTRAencryptionAlgorithms				EUTRAencryptionAlgorithms,

eUTRAintegrityProtectionAlgorithms		EUTRAintegrityProtectionAlgorithms,

iE-Extensions		ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} }	OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UL-NGU-UP-TNLModifyList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivity)) OF UL-NGU-UP-TNLModifyItem

UL-NGU-UP-TNLModifyItem ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation,

dL-NGU-UP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {UL-NGU-UP-TNLModifyItem-ExtIEs} } OPTIONAL,

...

}

UL-NGU-UP-TNLModifyItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UnavailableGUAMIList ::= SEQUENCE (SIZE(1..maxnoofServedGUAMIs)) OF UnavailableGUAMIItem

UnavailableGUAMIItem ::= SEQUENCE {

gUAMI								GUAMI,

timerApproachForGUAMIRemoval		TimerApproachForGUAMIRemoval					OPTIONAL,

backupAMFName						AMFName											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UnavailableGUAMIItem-ExtIEs} }	OPTIONAL,

...

}

UnavailableGUAMIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ULForwarding ::= ENUMERATED {

ul-forwarding-proposed,

...

}

UPTransportLayerInformation ::= CHOICE {

gTPTunnel				GTPTunnel,

choice-Extensions		ProtocolIE-SingleContainer { {UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UPTransportLayerInformationList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF UPTransportLayerInformationItem

UPTransportLayerInformationItem ::= SEQUENCE {

nGU-UP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {UPTransportLayerInformationItem-ExtIEs} } OPTIONAL,

...

}

UPTransportLayerInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UPTransportLayerInformationPairList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF UPTransportLayerInformationPairItem

UPTransportLayerInformationPairItem ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation,

dL-NGU-UP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {UPTransportLayerInformationPairItem-ExtIEs} } OPTIONAL,

...

}

UPTransportLayerInformationPairItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformation ::= CHOICE {

userLocationInformationEUTRA	UserLocationInformationEUTRA,

userLocationInformationNR		UserLocationInformationNR,

userLocationInformationN3IWF	UserLocationInformationN3IWF,

choice-Extensions		ProtocolIE-SingleContainer { {UserLocationInformation-ExtIEs} }

}

UserLocationInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UserLocationInformationEUTRA ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

tAI					TAI,

timeStamp			TimeStamp																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationEUTRA-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationEUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation	CRITICALITY ignore	EXTENSION NGRAN-CGI				PRESENCE optional},

...

}

UserLocationInformationN3IWF ::= SEQUENCE {

iPAddress			TransportLayerAddress,

portNumber			PortNumber,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationN3IWF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationN3IWF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformationNR ::= SEQUENCE {

nR-CGI				NR-CGI,

tAI					TAI,

timeStamp			TimeStamp															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationNR-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation	CRITICALITY ignore	EXTENSION NGRAN-CGI				PRESENCE optional},

...

}

UserPlaneSecurityInformation ::= SEQUENCE {

securityResult			SecurityResult,

securityIndication		SecurityIndication,

iE-Extensions		ProtocolExtensionContainer { {UserPlaneSecurityInformation-ExtIEs} }	OPTIONAL,

...

}

UserPlaneSecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- V

#### VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnoofTimePeriods)) OF VolumeTimedReport-Item

#### VolumeTimedReport-Item ::= SEQUENCE {

#### startTimeStamp				OCTET STRING (SIZE(4)),

#### endTimeStamp				OCTET STRING (SIZE(4)),

#### usageCountUL				INTEGER (0..18446744073709551615),

#### usageCountDL				INTEGER (0..18446744073709551615),

#### iE-Extensions		ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,

#### ...

#### }

#### VolumeTimedReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

#### ...

#### }

#### -- W

WarningAreaCoordinates ::= OCTET STRING (SIZE(1..1024))

WarningAreaList ::= CHOICE {

eUTRA-CGIListForWarning			EUTRA-CGIListForWarning,

nR-CGIListForWarning			NR-CGIListForWarning,

tAIListForWarning				TAIListForWarning,

emergencyAreaIDList				EmergencyAreaIDList,

choice-Extensions		ProtocolIE-SingleContainer { {WarningAreaList-ExtIEs} }

}

WarningAreaList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

WarningMessageContents ::= OCTET STRING (SIZE(1..9600))

WarningSecurityInfo ::= OCTET STRING (SIZE(50))

WarningType ::= OCTET STRING (SIZE(2))

WLANMeasurementConfiguration ::= SEQUENCE {

wlanMeasConfig             WLANMeasConfig,

wlanMeasConfigNameList		WLANMeasConfigNameList            OPTIONAL,

wlan-rssi                  ENUMERATED {true, ...}            OPTIONAL,

wlan-rtt                   ENUMERATED {true, ...}            OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementConfiguration-ExtIEs } } OPTIONAL,

...

}

WLANMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANName

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))

#### -- X

XnExtTLAs ::= SEQUENCE (SIZE(1..maxnoofXnExtTLAs)) OF XnExtTLA-Item

XnExtTLA-Item ::= SEQUENCE {

iPsecTLA					TransportLayerAddress		OPTIONAL,

gTP-TLAs					XnGTP-TLAs					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {XnExtTLA-Item-ExtIEs} } OPTIONAL,

...

}

XnExtTLA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SCTP-TLAs		CRITICALITY ignore	EXTENSION SCTP-TLAs		PRESENCE optional },

...

}

XnGTP-TLAs ::= SEQUENCE (SIZE(1..maxnoofXnGTP-TLAs)) OF TransportLayerAddress

XnTLAs ::= SEQUENCE (SIZE(1..maxnoofXnTLAs)) OF TransportLayerAddress

XnTNLConfigurationInfo ::= SEQUENCE {

xnTransportLayerAddresses			XnTLAs,

xnExtendedTransportLayerAddresses	XnExtTLAs										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {XnTNLConfigurationInfo-ExtIEs} }	OPTIONAL,

...

}

XnTNLConfigurationInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

NGAP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0..65535),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..65535)

ProtocolIE-ID		::= INTEGER (0..65535)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessfull-outcome }

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

id-DownlinkNASTransport						ProcedureCode ::= 4

id-DownlinkNonUEAssociatedNRPPaTransport	ProcedureCode ::= 5

id-DownlinkRANConfigurationTransfer			ProcedureCode ::= 6

id-DownlinkRANStatusTransfer				ProcedureCode ::= 7

id-DownlinkUEAssociatedNRPPaTransport		ProcedureCode ::= 8

id-ErrorIndication							ProcedureCode ::= 9

id-HandoverCancel							ProcedureCode ::= 10

id-HandoverNotification						ProcedureCode ::= 11

id-HandoverPreparation						ProcedureCode ::= 12

id-HandoverResourceAllocation				ProcedureCode ::= 13

id-InitialContextSetup						ProcedureCode ::= 14

id-InitialUEMessage							ProcedureCode ::= 15

id-LocationReportingControl					ProcedureCode ::= 16

id-LocationReportingFailureIndication		ProcedureCode ::= 17

id-LocationReport							ProcedureCode ::= 18

id-NASNonDeliveryIndication					ProcedureCode ::= 19

id-NGReset									ProcedureCode ::= 20

id-NGSetup									ProcedureCode ::= 21

id-OverloadStart							ProcedureCode ::= 22

id-OverloadStop								ProcedureCode ::= 23

id-Paging									ProcedureCode ::= 24

id-PathSwitchRequest						ProcedureCode ::= 25

id-PDUSessionResourceModify					ProcedureCode ::= 26

id-PDUSessionResourceModifyIndication		ProcedureCode ::= 27

id-PDUSessionResourceRelease				ProcedureCode ::= 28

id-PDUSessionResourceSetup					ProcedureCode ::= 29

id-PDUSessionResourceNotify					ProcedureCode ::= 30

id-PrivateMessage							ProcedureCode ::= 31

id-PWSCancel								ProcedureCode ::= 32

id-PWSFailureIndication						ProcedureCode ::= 33

id-PWSRestartIndication						ProcedureCode ::= 34

id-RANConfigurationUpdate					ProcedureCode ::= 35

id-RerouteNASRequest						ProcedureCode ::= 36

id-RRCInactiveTransitionReport				ProcedureCode ::= 37

id-TraceFailureIndication					ProcedureCode ::= 38

id-TraceStart								ProcedureCode ::= 39

id-UEContextModification					ProcedureCode ::= 40

id-UEContextRelease							ProcedureCode ::= 41

id-UEContextReleaseRequest					ProcedureCode ::= 42

id-UERadioCapabilityCheck					ProcedureCode ::= 43

id-UERadioCapabilityInfoIndication			ProcedureCode ::= 44

id-UETNLABindingRelease						ProcedureCode ::= 45

id-UplinkNASTransport						ProcedureCode ::= 46

id-UplinkNonUEAssociatedNRPPaTransport		ProcedureCode ::= 47

id-UplinkRANConfigurationTransfer			ProcedureCode ::= 48

id-UplinkRANStatusTransfer					ProcedureCode ::= 49

id-UplinkUEAssociatedNRPPaTransport			ProcedureCode ::= 50

id-WriteReplaceWarning						ProcedureCode ::= 51

id-SecondaryRATDataUsageReport				ProcedureCode ::= 52

id-UplinkRIMInformationTransfer				ProcedureCode ::= 53

id-DownlinkRIMInformationTransfer			ProcedureCode ::= 54

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedS-NSSAIs				INTEGER ::= 8

maxnoofBPLMNs						INTEGER ::= 12

maxnoofCellIDforWarning				INTEGER ::= 65535

maxnoofCellinAoI					INTEGER ::= 256

maxnoofCellinEAI					INTEGER ::= 65535

maxnoofCellinTAI					INTEGER ::= 65535

maxnoofCellsingNB					INTEGER ::= 16384

maxnoofCellsinngeNB					INTEGER ::= 256

maxnoofCellsinUEHistoryInfo			INTEGER ::= 16

maxnoofCellsUEMovingTrajectory		INTEGER ::= 16

maxnoofDRBs							INTEGER ::= 32

maxnoofEmergencyAreaID				INTEGER ::= 65535

maxnoofEAIforRestart				INTEGER ::= 256

maxnoofEPLMNs						INTEGER ::= 15

maxnoofEPLMNsPlusOne				INTEGER ::= 16

maxnoofE-RABs						INTEGER ::= 256

maxnoofErrors						INTEGER ::= 256

maxnoofForbTACs						INTEGER ::= 4096

maxnoofMultiConnectivity				INTEGER ::= 4

maxnoofMultiConnectivityMinusOne	INTEGER ::= 3

maxnoofNGConnectionsToReset			INTEGER ::= 65536

maxnoofPDUSessions					INTEGER ::= 256

maxnoofPLMNs						INTEGER ::= 12

maxnoofQosFlows						INTEGER ::= 64

maxnoofRANNodeinAoI					INTEGER ::= 64

maxnoofRecommendedCells				INTEGER ::= 16

maxnoofRecommendedRANNodes			INTEGER ::= 16

maxnoofAoI							INTEGER ::= 64

maxnoofServedGUAMIs					INTEGER ::= 256

maxnoofSliceItems					INTEGER ::= 1024

maxnoofTACs							INTEGER ::= 256

maxnoofTAIforInactive				INTEGER ::= 16

maxnoofTAIforPaging					INTEGER ::= 16

maxnoofTAIforRestart				INTEGER ::= 2048

maxnoofTAIforWarning				INTEGER ::= 65535

maxnoofTAIinAoI						INTEGER ::= 16

maxnoofTimePeriods					INTEGER ::= 2

maxnoofTNLAssociations				INTEGER ::= 32

maxnoofXnExtTLAs					INTEGER ::= 16

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofMDTPLMNs						INTEGER ::= 16

maxnoofCellIDforMDT					INTEGER ::= 32

maxnoofTAforMDT						INTEGER ::= 8

maxnoofBluetoothName				INTEGER ::= 4

maxnoofWLANName						INTEGER ::= 4

maxnoofSensorName					INTEGER ::= 3

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

id-AMF-TNLAssociationToAddList							ProtocolIE-ID ::= 6

id-AMF-TNLAssociationToRemoveList						ProtocolIE-ID ::= 7

id-AMF-TNLAssociationToUpdateList						ProtocolIE-ID ::= 8

id-AMFTrafficLoadReductionIndication					ProtocolIE-ID ::= 9

id-AMF-UE-NGAP-ID										ProtocolIE-ID ::= 10

id-AssistanceDataForPaging								ProtocolIE-ID ::= 11

id-BroadcastCancelledAreaList							ProtocolIE-ID ::= 12

id-BroadcastCompletedAreaList							ProtocolIE-ID ::= 13

id-CancelAllWarningMessages								ProtocolIE-ID ::= 14

id-Cause												ProtocolIE-ID ::= 15

id-CellIDListForRestart									ProtocolIE-ID ::= 16

id-ConcurrentWarningMessageInd							ProtocolIE-ID ::= 17

id-CoreNetworkAssistanceInformationForInactive			ProtocolIE-ID ::= 18

id-CriticalityDiagnostics								ProtocolIE-ID ::= 19

id-DataCodingScheme										ProtocolIE-ID ::= 20

id-DefaultPagingDRX										ProtocolIE-ID ::= 21

id-DirectForwardingPathAvailability						ProtocolIE-ID ::= 22

id-EmergencyAreaIDListForRestart						ProtocolIE-ID ::= 23

id-EmergencyFallbackIndicator							ProtocolIE-ID ::= 24

id-EUTRA-CGI											ProtocolIE-ID ::= 25

id-FiveG-S-TMSI											ProtocolIE-ID ::= 26

id-GlobalRANNodeID										ProtocolIE-ID ::= 27

id-GUAMI												ProtocolIE-ID ::= 28

id-HandoverType											ProtocolIE-ID ::= 29

id-IMSVoiceSupportIndicator								ProtocolIE-ID ::= 30

id-IndexToRFSP											ProtocolIE-ID ::= 31

id-InfoOnRecommendedCellsAndRANNodesForPaging			ProtocolIE-ID ::= 32

id-LocationReportingRequestType							ProtocolIE-ID ::= 33

id-MaskedIMEISV											ProtocolIE-ID ::= 34

id-MessageIdentifier									ProtocolIE-ID ::= 35

id-MobilityRestrictionList								ProtocolIE-ID ::= 36

id-NASC													ProtocolIE-ID ::= 37

id-NAS-PDU												ProtocolIE-ID ::= 38

id-NASSecurityParametersFromNGRAN						ProtocolIE-ID ::= 39

id-NewAMF-UE-NGAP-ID									ProtocolIE-ID ::= 40

id-NewSecurityContextInd								ProtocolIE-ID ::= 41

id-NGAP-Message											ProtocolIE-ID ::= 42

id-NGRAN-CGI											ProtocolIE-ID ::= 43

id-NGRANTraceID											ProtocolIE-ID ::= 44

id-NR-CGI												ProtocolIE-ID ::= 45

id-NRPPa-PDU											ProtocolIE-ID ::= 46

id-NumberOfBroadcastsRequested							ProtocolIE-ID ::= 47

id-OldAMF												ProtocolIE-ID ::= 48

id-OverloadStartNSSAIList								ProtocolIE-ID ::= 49

id-PagingDRX											ProtocolIE-ID ::= 50

id-PagingOrigin											ProtocolIE-ID ::= 51

id-PagingPriority										ProtocolIE-ID ::= 52

id-PDUSessionResourceAdmittedList						ProtocolIE-ID ::= 53

id-PDUSessionResourceFailedToModifyListModRes			ProtocolIE-ID ::= 54

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 55

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 56

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 57

id-PDUSessionResourceFailedToSetupListSURes				ProtocolIE-ID ::= 58

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListCxtRelCpl						ProtocolIE-ID ::= 60

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61

id-PDUSessionResourceModifyListModCfm					ProtocolIE-ID ::= 62

id-PDUSessionResourceModifyListModInd					ProtocolIE-ID ::= 63

id-PDUSessionResourceModifyListModReq					ProtocolIE-ID ::= 64

id-PDUSessionResourceModifyListModRes					ProtocolIE-ID ::= 65

id-PDUSessionResourceNotifyList							ProtocolIE-ID ::= 66

id-PDUSessionResourceReleasedListNot					ProtocolIE-ID ::= 67

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 68

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 69

id-PDUSessionResourceReleasedListRelRes					ProtocolIE-ID ::= 70

id-PDUSessionResourceSetupListCxtReq					ProtocolIE-ID ::= 71

id-PDUSessionResourceSetupListCxtRes					ProtocolIE-ID ::= 72

id-PDUSessionResourceSetupListHOReq						ProtocolIE-ID ::= 73

id-PDUSessionResourceSetupListSUReq						ProtocolIE-ID ::= 74

id-PDUSessionResourceSetupListSURes						ProtocolIE-ID ::= 75

id-PDUSessionResourceToBeSwitchedDLList					ProtocolIE-ID ::= 76

id-PDUSessionResourceSwitchedList						ProtocolIE-ID ::= 77

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 78

id-PDUSessionResourceToReleaseListRelCmd				ProtocolIE-ID ::= 79

id-PLMNSupportList										ProtocolIE-ID ::= 80

id-PWSFailedCellIDList									ProtocolIE-ID ::= 81

id-RANNodeName											ProtocolIE-ID ::= 82

id-RANPagingPriority									ProtocolIE-ID ::= 83

id-RANStatusTransfer-TransparentContainer				ProtocolIE-ID ::= 84

id-RAN-UE-NGAP-ID										ProtocolIE-ID ::= 85

id-RelativeAMFCapacity									ProtocolIE-ID ::= 86

id-RepetitionPeriod										ProtocolIE-ID ::= 87

id-ResetType											ProtocolIE-ID ::= 88

id-RoutingID											ProtocolIE-ID ::= 89

id-RRCEstablishmentCause								ProtocolIE-ID ::= 90

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 91

id-RRCState												ProtocolIE-ID ::= 92

id-SecurityContext										ProtocolIE-ID ::= 93

id-SecurityKey											ProtocolIE-ID ::= 94

id-SerialNumber											ProtocolIE-ID ::= 95

id-ServedGUAMIList										ProtocolIE-ID ::= 96

id-SliceSupportList										ProtocolIE-ID ::= 97

id-SONConfigurationTransferDL							ProtocolIE-ID ::= 98

id-SONConfigurationTransferUL							ProtocolIE-ID ::= 99

id-SourceAMF-UE-NGAP-ID									ProtocolIE-ID ::= 100

id-SourceToTarget-TransparentContainer					ProtocolIE-ID ::= 101

id-SupportedTAList										ProtocolIE-ID ::= 102

id-TAIListForPaging										ProtocolIE-ID ::= 103

id-TAIListForRestart									ProtocolIE-ID ::= 104

id-TargetID												ProtocolIE-ID ::= 105

id-TargetToSource-TransparentContainer					ProtocolIE-ID ::= 106

id-TimeToWait											ProtocolIE-ID ::= 107

id-TraceActivation										ProtocolIE-ID ::= 108

id-TraceCollectionEntityIPAddress						ProtocolIE-ID ::= 109

id-UEAggregateMaximumBitRate							ProtocolIE-ID ::= 110

id-UE-associatedLogicalNG-connectionList				ProtocolIE-ID ::= 111

id-UEContextRequest										ProtocolIE-ID ::= 112

id-UE-NGAP-IDs											ProtocolIE-ID ::= 114

id-UEPagingIdentity										ProtocolIE-ID ::= 115

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 116

id-UERadioCapability									ProtocolIE-ID ::= 117

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 118

id-UESecurityCapabilities								ProtocolIE-ID ::= 119

id-UnavailableGUAMIList									ProtocolIE-ID ::= 120

id-UserLocationInformation								ProtocolIE-ID ::= 121

id-WarningAreaList										ProtocolIE-ID ::= 122

id-WarningMessageContents								ProtocolIE-ID ::= 123

id-WarningSecurityInfo									ProtocolIE-ID ::= 124

id-WarningType											ProtocolIE-ID ::= 125

id-AdditionalUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 126

id-DataForwardingNotPossible							ProtocolIE-ID ::= 127

id-DL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 128

id-NetworkInstance										ProtocolIE-ID ::= 129

id-PDUSessionAggregateMaximumBitRate					ProtocolIE-ID ::= 130

id-PDUSessionResourceFailedToModifyListModCfm			ProtocolIE-ID ::= 131

id-PDUSessionResourceFailedToSetupListCxtFail			ProtocolIE-ID ::= 132

id-PDUSessionResourceListCxtRelReq						ProtocolIE-ID ::= 133

id-PDUSessionType										ProtocolIE-ID ::= 134

id-QosFlowAddOrModifyRequestList						ProtocolIE-ID ::= 135

id-QosFlowSetupRequestList								ProtocolIE-ID ::= 136

id-QosFlowToReleaseList									ProtocolIE-ID ::= 137

id-SecurityIndication									ProtocolIE-ID ::= 138

id-UL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 139

id-UL-NGU-UP-TNLModifyList								ProtocolIE-ID ::= 140

id-WarningAreaCoordinates								ProtocolIE-ID ::= 141

id-PDUSessionResourceSecondaryRATUsageList				ProtocolIE-ID ::= 142

id-HandoverFlag											ProtocolIE-ID ::= 143

id-SecondaryRATUsageInformation							ProtocolIE-ID ::= 144

id-PDUSessionResourceReleaseResponseTransfer			ProtocolIE-ID ::= 145

id-RedirectionVoiceFallback								ProtocolIE-ID ::= 146

id-UERetentionInformation								ProtocolIE-ID ::= 147

id-S-NSSAI												ProtocolIE-ID ::= 148

id-PSCellInformation									ProtocolIE-ID ::= 149

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType						ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation						ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-ManagementBasedMDTPLMNList							ProtocolIE-ID ::= 888	--will be assigned by MCC

id-MDTConfiguration										ProtocolIE-ID ::= 889

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

NGAP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

Presence,

PrivateIE-ID,

ProtocolExtensionID,

ProtocolIE-ID

FROM NGAP-CommonDataTypes

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs

FROM NGAP-Constants;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

NGAP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID					UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

NGAP-PROTOCOL-IES-PAIR ::= CLASS {

&id					ProtocolIE-ID				UNIQUE,

&firstCriticality	Criticality,

&FirstValue,

&secondCriticality	Criticality,

&SecondValue,

&presence			Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

NGAP-PROTOCOL-EXTENSION ::= CLASS {

&id				ProtocolExtensionID			UNIQUE,

&criticality	Criticality,

&Extension,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

EXTENSION		&Extension

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

NGAP-PRIVATE-IES ::= CLASS {

&id				PrivateIE-ID,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {NGAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {NGAP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {NGAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				NGAP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		NGAP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			NGAP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {NGAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {NGAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					NGAP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	NGAP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			NGAP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	NGAP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			NGAP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, NGAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-SingleContainer {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, NGAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {NGAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {NGAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					NGAP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			NGAP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		NGAP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {NGAP-PRIVATE-IES : IEsSetParam } ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {NGAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					NGAP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			NGAP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				NGAP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

-- ASN1STOP

End Change