3GPP TSG-RAN WG3 Meeting # 108-e	R3-203054

# 1st – 12th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1682 | rev | 12 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MT Early Data Transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5, NB_IOTenh3 |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Mobile Terminated Early Data Transmission is not supported in TS 36.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new IE (Data size IE ) is added to the S1 PAGING message. eNB uses this to to decide whether to initiate MT-EDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.8.2, 8.5.2, 8.6.2.1, 9.1.6, 9.2.3.55, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: removal of indicator of Initial UE Message Rev 2: rebaselining to TS 36.413 15.6.0 Rev 3: include changes agreed in R3-194777 (replace MT EDT Session IE with Data size IE, plus editor notes) Rev4: rebaselining to TS 36.413 v15.7.0 for endorsement at RAN3#105bis Rev5: add changes agreed in R3-196236 (removal of Editor Notes, generalization of EDT Session IE for MO and MT in Initial UE Message, and change in procedural text for the Data Size IE, “may, if supported” => “shall, if supported”) Rev6: presentation to RAN3#106 Rev7: include changes agreed in R3-197757 Rev8: rebaselining and addition of ASN.1, also addition of changes in clause 9.2.3.55 from R3-197757, missing in rev7. Rev9: include changes from R3-201266 Rev10: Rebaselining to v16.1.0 Rev11: include changes from R3-201799 Rev12: presentation to RAN3#108-e |  |  |  |  |  |  |  |  |

### 8.3.8	UE Context Resume

#### 8.3.8.1	General

The purpose of the UE Context Resume procedure is to indicate to the MME that the UE has resumed the suspended RRC connection or accesses for early data transmission and to request the MME to resume the UE context, UE-associated logical S1-connection and the related bearer contexts in the EPC.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: UE Context Resume procedure. Successful operation.

The eNB initiates the procedure by sending the UE CONTEXT RESUME REQUEST message to the MME. If the eNB is not able to admit all suspended E-RABs the eNB shall indicate this in the E-RABs Failed To Resume List IE.

Upon receipt of the UE CONTEXT RESUME REQUEST message the MME shall act as defined in TS 23.401 [11] and respond with the UE CONTEXT RESUME RESPONSE. If the MME is not able to admit all suspended E-RABs the MME shall indicate this in the E-RABs Failed To Resume List IE.

The eNB shall release resources for each E-RAB failed to resume and shall assume that the EPC has released respective resources as well.

If the Security Context IE is included in the UE CONTEXT RESUME RESPONSE message, the eNB shall store the received Security Context IE in the UE context and the eNB shall use it for the next suspend/resume or X2 handover or Intra eNB handovers as specified in TS 33.401 [15].

If the Pending Data Indication IE is included in the UE CONTEXT RESUME RESPONSE message, the eNB shall use it as defined in TS 23.401 [11]. If the UE Context Resume procedure was initiated as a result of an EDT session as described in TS 36.300 [14] and the Pending Data Indication IE is received in the UE CONTEXT RESUME RESPONSE message, the eNB shall, if supported, use it to decide whether to proceed to set up an RRC connection for the UE.

>>>> NEXT CHANGE <<<<

## 8.5	Paging

### 8.5.1	General

The purpose of the Paging procedure is to enable the MME to page a UE in the specific eNB.

### 8.5.2	Successful Operation

Figure 8.5.2-1: Paging procedure

The MME initiates the paging procedure by sending the PAGING message to the eNB.

At the reception of the PAGING message, the eNB shall perform paging of the UE in cells which belong to tracking areas as indicated in the List of TAIs IE.

The CN Domain IE shall be transferred transparently to the UE.

The Paging DRX IE may be included in the PAGING message, and if present the eNB shall use it according to TS 36.304 [20].

A list of CSG IDs may be included in the PAGING message.

If included, the E-UTRAN may use the list of CSG IDs to avoid paging the UE at CSG cells whose CSG ID does not appear in the list.

For each cell that belongs to any of the TAs indicated in the List of TAIs IE, the eNB shall generate one page on the radio interface.

The Paging Priority IE may be included in the PAGING message, and if present the eNB may use it according to TS 23.401 [11] and TS 23.272 [17].

If the UE Radio Capability for Paging IE is included in the PAGING message, the eNB may use it to apply specific paging schemes. If the Enhanced Coverage Restricted IE is included in the PAGING message, the eNB shall, if supported, use it as defined in TS 23.401 [11].

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present according to TS 36.300 [14].

If the Assistance Data for CE capable UEs IE is included in the Assistance Data for Paging IE, it may be used for paging the indicated CE capable UE, together with the Paging Attempt Information IE according to TS 36.300 [14].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be used for paging the UE according to TS 36.300 [14].

If the Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to TS 36.304 [20]. If the Paging Time Window IE is included in the Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

If the Extended UE Identity Index Value IE is included in the PAGING message, the eNB shall, if supported, use it to identify the paging resources to be used according to TS 36.304 [20]. The MME shall, if supported, include the Extended UE Identity Index Value IE in the PAGING message.

If the NB-IoT Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to TS 36.304 [20]. If the NB-IoT Paging Time Window IE is included in the NB-IoT Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

If the NB-IoT UE Identity Index Value IE is included in the PAGING message, the eNB shall, if supported, use it to identify the paging resources to be used according to TS 36.304 [20].

If the CE-Mode-B Restricted IE is included in the PAGING message and the Enhanced Coverage Restricted IE is not set to restricted, the eNB shall use it as defined in TS 23.401 [11].

If the Data Size IE is included in the PAGING message, the eNB shall, if supported, use it to decide whether to initiate Mobile Terminated EDT procedures towards the UE as described in TS 36.300 [14].

>>>> NEXT CHANGE <<<<

#### 8.6.2.1	Initial UE Message

Figure 8.6.2.1-1: Initial UE Message procedure

When the eNB has received from the radio interface the first UL NAS message transmitted via RRC message to be forwarded to an MME, the eNB shall invoke the NAS Transport procedure and send the INITIAL UE MESSAGE message to the MME including the NAS message as a NAS-PDU IE. The eNB shall allocate a unique eNB UE S1AP ID to be used for the UE and the eNB shall include this identity in the INITIAL UE MESSAGE message. In case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message. When the eNB has received from the radio interface the S-TMSI IE, it shall include it in the INITIAL UE MESSAGE message. If the eNB does not support NNSF and the eNB has received from the radio interface the GUMMEI IE, the eNB may include it in the INITIAL UE MESSAGE message. If the eNB does not support NNSF and the eNB has received from the radio interface the GUMMEI Type IE, the eNB may include it in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment originating from a CSG cell, the CSG Id IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment originating from a Hybrid cell, the CSG Id IE and the Cell Access Mode IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment triggered by a Relay Node as defined in TS 36.300 [14], the GW Transport Layer Address IE and the Relay Node Indicator IE shall be included in the INITIAL UE MESSAGE message.

If the eNB has a L-GW function for LIPA operation, it shall include the GW Transport Layer Address IE in the INITIAL UE MESSAGE message.

If the SIPTO L-GW Transport Layer Address IE is received in the INITIAL UE MESSAGE message, the MME shall, if supported, use it for SIPTO@LN operation as sepecified in TS 23.401 [11].

If the LHN ID IE is included in the INITIAL UE MESSAGE message, the MME shall, if supported, use it as specified in TS 23.401 [11].

If the Tunnel Information for BBF IE is received in the INITIAL UE MESSAGE message, the MME shall, if supported, use it in the core network as specified in TS 23.139 [37].

If the MME Group ID IE is included in the INITIAL UE MESSAGE message this indicates that the message is a rerouted message and the MME shall, if supported, use the IE as described in TS 23.401 [11].

If the UE Usage Type IE is included in the INITIAL UE MESSAGE message, then the selected MME in the DCN shall if supported, use it as defined in TS 23.401 [48].

If the DCN ID IE is included in the INITIAL UE MESSAGE message, the MME shall, if supported, use it as defined in TS 23.401 [48].

NOTE:	The first UL NAS message is always received in the RRC CONNECTION SETUP COMPLETE message, except that in cases of NB-IoT and MTC the first UL NAS message may be received in RRCEarlyDataRequest message.

If the CE-mode-B Support Indicator IE is included in the INITIAL UE MESSAGE message and set to "supported", the MME shall, if supported, use the extended NAS timer settings for the UE as specified in TS 24.301[24].

If the Coverage Level IE is included in the INITIAL UE MESSAGE message, the MME shall, if supported, use it as specified in TS 24.301[24].

If the UE Application Layer Measurement Capability IE is included in the INITIAL UE MESSAGE message, the MME shall, if supported, use it when initiating UE Application Layer Measurement.

If the EDT Session IE set to “true” is included in the INITIAL UE MESSAGE message, the MME shall, if supported, consider that the message has been received as a result of an EDT session as described in TS 36.300 [14].

>>>> NEXT CHANGE <<<<

### 9.1.6	PAGING

This message is sent by the MME and is used to page a UE in one or several tracking areas.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.2.3.10 |  | YES | ignore |
| UE Paging Identity | M |  | 9.2.3.13 |  | YES | ignore |
| Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| CN Domain | M |  | 9.2.3.22 |  | YES | ignore |
| List of TAIs |  | 1 |  |  | YES | ignore |
| >TAI List Item |  | 1 .. <maxnoofTAIs> |  |  | EACH | ignore |
| >>TAI | M |  | 9.2.3.16 |  | - |  |
| CSG Id List |  | 0..1 |  |  | GLOBAL | ignore |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Paging Priority | O |  | 9.2.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.2.1.103 |  | YES | ignore |
| Paging eDRX Information | O |  | 9.2.1.111 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.46 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.2.1.115 |  | YES | ignore |
| NB-IoT UE Identity Index value | O |  | 9.2.3.47 |  | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| CE-Mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Data size | O |  | INTEGER (1..4095, …) | The unit is: bit | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIs | Maximum no. of TAIs. Value is 256. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |

>>>> NEXT CHANGE <<<<

#### 9.2.3.55	Pending Data Indication

This IE indicates that the MME is aware of pending signalling or data in the network for the UE, or that the MME expects a response from the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Pending Data Indication | M |  | ENUMERATED (true, …) |  |

>>>> NEXT CHANGE <<<

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

-- skip unchanged text --

PSCellInformation,

NR-CGI,

ConnectedengNBList,

EN-DCSONConfigurationTransfer,

TimeSinceSecondaryNodeRelease,

AdditionalRRMPriorityIndex,

DataSize

FROM S1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

S1AP-PRIVATE-IES,

S1AP-PROTOCOL-EXTENSION,

S1AP-PROTOCOL-IES,

S1AP-PROTOCOL-IES-PAIR

FROM S1AP-Containers

id-AssistanceDataForPaging,

id-AerialUEsubscriptionInformation,

id-uEaggregateMaximumBitrate,

id-BearerType,

-- skip unchanged text –

id-ConnectedengNBToAddList,

id-ConnectedengNBToRemoveList,

id-EN-DCSONConfigurationTransfer-ECT,

id-EN-DCSONConfigurationTransfer-MCT,

id-TimeSinceSecondaryNodeRelease,

id-DataSize

FROM S1AP-Constants;

-- skip unchanged text --

-- **************************************************************

--

##### -- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{PagingIEs}},

...

}

PagingIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue			CRITICALITY ignore	TYPE UEIdentityIndexValue				PRESENCE mandatory}|

{ ID id-UEPagingID						CRITICALITY ignore	TYPE UEPagingID							PRESENCE mandatory}|

{ ID id-pagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional}|

{ ID id-CNDomain						CRITICALITY ignore	TYPE CNDomain							PRESENCE mandatory}|

{ ID id-TAIList							CRITICALITY ignore	TYPE TAIList							PRESENCE mandatory}|

{ ID id-CSG-IdList						CRITICALITY ignore	TYPE CSG-IdList							PRESENCE optional}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional}|

-- Extension for Release 13 to support Paging Optimisation and Coverage Enhancement paging –-

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional}|

{ ID id-Paging-eDRXInformation			CRITICALITY ignore	TYPE Paging-eDRXInformation				PRESENCE optional}|

{ ID id-extended-UEIdentityIndexValue	CRITICALITY ignore	TYPE Extended-UEIdentityIndexValue	PRESENCE optional}|

{ ID id-NB-IoT-Paging-eDRXInformation	CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInformation	PRESENCE optional}|

{ ID id-NB-IoT-UEIdentityIndexValue		CRITICALITY ignore	TYPE NB-IoT-UEIdentityIndexValue		PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted			PRESENCE optional}|

{ ID id-DataSize						CRITICALITY ignore	TYPE DataSize							PRESENCE optional},

...

}

>>>> NEXT CHANGE <<<<

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

-- skip unchanged text --

#### -- D

DataCodingScheme ::= BIT STRING (SIZE (8))

DataSize ::= INTEGER(1..4095, ...)

DCN-ID ::= INTEGER (0..65535)

ServedDCNs ::= SEQUENCE (SIZE(0..maxnoofDCNs)) OF ServedDCNsItem

-- skip unchanged text --

PDCP-SNExtended ::= INTEGER (0..32767)

PDCP-SNlength18 ::= INTEGER (0..262143)

PendingDataIndication ::= ENUMERATED {

true,

...

}

>>>> NEXT CHANGE <<<<

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-- skip unchanged text --

id-ConnectedengNBToAddList							ProtocolIE-ID ::= 292

id-ConnectedengNBToRemoveList						ProtocolIE-ID ::= 293

id-EN-DCSONConfigurationTransfer-ECT				ProtocolIE-ID ::= 294

id-EN-DCSONConfigurationTransfer-MCT				ProtocolIE-ID ::= 295

id-IMSvoiceEPSfallbackfrom5G						ProtocolIE-ID ::= 296

id-TimeSinceSecondaryNodeRelease					ProtocolIE-ID ::= 297

id-RequestTypeAdditionalInfo						ProtocolIE-ID ::= 298

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 299

id-ContextatSource									ProtocolIE-ID ::= 300

id-DataSize											ProtocolIE-ID ::= XXX

END