3GPP TSG-RAN3 Meeting #108-e 			         R3-203142

E-meeting, June 1 – June 11, 2020

Agenda Item	: 8.1

Source	: LG Electronics Inc.

Title	: Issue on Storing Inactive AS Context in SCG

Document for	: Decision

1.	Introduction

In the RAN2 #109bis-e meeting, a LS “R3-203110 / R2-2004242” [1] was sent to RAN3. According to the LS, this paper provides the analysis on RAN3 impacts, give our proposal and the corresponding CR.

2.	Discussion

The LS “R3-203110 / R2-2004242” [1] from RAN2 is shown as follows:

1. Overall Description: RAN2 has decided to update the Inactive AS context related to the stored SCG configuration. As a result, the following agreement was made during RAN2#109bis-e meeting. Agreement: The sPCellConfigCommon for the PSCell is saved as part of the UE AS Inactive AS context.. 2. Actions: To RAN3 group ACTION: RAN2 respectfully asks RAN3 group to take the above information into account.

From TS 38.331, the agreement above corresponds to the following highlighted part in green:

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                    OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity              OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                 OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                             OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                        OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                   OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                    OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                              OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxLC-ID-Iab-r16)) OF BH-RLC-ChannelConfig-r16    OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxLC-ID-Iab-r16)) OF BH-LogicalChannelIdentity-r16 OPTIONAL, -- Need N

dormancySCellGroups                        DormancySCellGroups                                                 OPTIONAL,   -- Need N

simultaneousTCI-UpdateList-r16             SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousTCI-UpdateListSecond-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList-r16        SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedListSecond-r16  SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL    -- Need R

]]

}

DormancySCellGroups::=               SEQUENCE {

withinActiveTimeToAddModList         SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroup-r16    OPTIONAL,   -- Need N

withinActiveTimeToReleaseList        SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroupID-r16  OPTIONAL,   -- Need N

outsideActiveTimeToAddModList        SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroup-r16    OPTIONAL,   -- Cond DormancyWUS

outsideActiveTimeToReleaseList       SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroupID-r16  OPTIONAL    -- Need N

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                         OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]]

}

On the other hand, the following highlighted part (Lower Layer presence status change IE in UE CONTEXT MODIFICATION REQUEST message) in TS 38.473 (F1AP), which was agreed in RAN3#106, should be revised to align with RAN2 agreement above. The reason is that “spCellConfigCommon” in “ReconfigurationWithSync” should be stored in DU, while other parameters such RNTI, t304, rach-ConfigDedicated and smtc are the exceptions.

9.3.1.94	Lower Layer Presence Status Change

This IE indicates lower layer resources’ presence status shall be changed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Lower Layer Presence Status Change | M |  | ENUMERATED (suspend lower layers, resume lower layers ...) | “suspend lower layers” will store CellGroupConfig except ReconfigurationWithSync “resume lower layers” shall restore SCG and only set after "suspend lower layers" has been indicated Editor Note: The usage of this IE may need to be refined. |

The following change is proposed:

#### 9.3.1.94	Lower Layer Presence Status Change

This IE indicates lower layer resources’ presence status shall be changed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Lower Layer Presence Status Change | M |  | ENUMERATED (suspend lower layers, resume lower layers ...) | “suspend lower layers” will store CellGroupConfig except RNTI, t304, rach-ConfigDedicated and smtc in ReconfigurationWithSync. “resume lower layers” shall restore SCG and only set after "suspend lower layers" has been indicated |

Proposal 1: Semantics description of Lower Layer presence status change IE in UE CONTEXT MODIFICATION REQUEST message should be revised for F1AP, shown as above.

Proposal 2: To agree the corresponding CR in [6].

3.	Conclusion

In this contribution, the issue on inactive AS context in SCG was discussed based on RAN2’ incoming LS. The following proposals are suggested:

Proposal 1: Semantics description of Lower Layer presence status change IE in UE CONTEXT MODIFICATION REQUEST message should be revised for F1AP, shown as above.

Proposal 2: To agree the corresponding CR in [6].

4.	Reference

[1] R3-203110 / R2-2004242, “LS on updated Inactive AS context”

[2] R2-2002135, Summary of MCG SCell and SCG Configuration with RRC Resume, ZTE

[3] 3GPP TS 38.331, v16.0.0

[4] 3GPP TS 36.331, v16.0.0

[5] 3GPP TS 38.473, v16.1.0

[6] R2-20xxxx, “CR for TS38.473 on storing inactive AS context in SCG”, LG Electronics