3GPP TSG-RAN3 Meeting #108-e	R3-203207

E-meeting, 01 - 11 June 2020

Title: 	Consideration on Selected PLMN ID for untrusted non-3GPP access

Source: 	Huawei

Agenda item:	9.3.4

Document Type:	Discussion

1. Introduction

In RAN3#107bis-e meeting, the Selected PLMN ID in INITIAL UE MESSAGE for untrusted non-3GPP access were discussed [1] [2], and it was understood that the selected PLMN id is missing for untrusted non-3GPP access, and there was no agreement on where to insert the IE in the message.

In this contribution, we further discuss the topic, and propose the corresponding solution.

3. Discussion

Based on TS23.502, the selected PLMN ID needs to be provided from NG-RAN node to AMF in case of N3IWF, and also other scenarios:

| Section 4.12.2.2 Registration procedure for untrusted non-3GPP access | 6. The N3IWF shall select an AMF based on the received AN parameters and local policy, as specified in TS 23.501 [2], clause 6.3.5. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message. This message contains N2 parameters that include the Selected PLMN ID and the Establishment cause. |
| --- | --- |
| Section 4.2.2.2.2 Registration procedure | 3. (R)AN to new AMF: N2 message (N2 parameters, Registration Request (as described in step 1) and [LTE-M Indication]. When NG-RAN is used, the N2 parameters include the Selected PLMN ID (or PLMN ID and NID, see TS 23.501 [2], clause 5.30), Location Information and Cell Identity related to the cell in which the UE is camping, UE Context Request which indicates that a UE context including security information needs to be setup at the NG-RAN |
| Section 4.2.3.2 UE Triggered Service Request | 2. (R)AN to AMF: N2 Message (N2 parameters, Service Request). Details of this step are described in TS 38.413 [10]. If the AMF can't handle the Service Request it will reject it. When NG-RAN is used, the N2 parameters include the 5G-S-TMSI, Selected PLMN ID (or PLMN ID and NID, see TS 23.501 [2], clause 5.30), Location information and Establishment cause, UE Context Request. |

In NGAP: INITIAL UE MESSAGE message, it is clarified that in case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message.

As the TAI is not provided in case of N3IWF, the selected PLMN info is missing, two solutions were mentioned in last meeting:

- Option 1: Include Selected PLMN ID in the N3IWF branch of User Location Information, as mandatory IE [1][2];

- Option 2: Include Selected PLMN ID in the INITIAL UE MESSAGE message, i.e. outside of ULI, as optional IE;

Considering that option1 is not backward compatible from functional point of view, and if option 2 is adopted, the same new IE can be reused by other scenarios, e.g.  WWC TNGF, TWIF, etc WWC TNGF, TWIF, etc. it is preferred to select option 2 as the way to introduce selected PLMN ID.

Proposal: Include Selected PLMN ID in the INITIAL UE MESSAGE message, i.e. outside of ULI, as new optional IE;

4. Proposals

In this contribution, how to provide the selected PLMN id for N3IWF is discussed, with the following proposals:

Proposal: Include Selected PLMN ID in the INITIAL UE MESSAGE message, i.e. outside of ULI, as new optional IE;

- It is also proposed to agree the NGAP CRs provided in [3] and [4], and CR to TS29.413 provided in [5].

5. Reference

- R3-202628 Selected PLMN ID in INITIAL UE MESSAGE for untrusted non-3GPP access, Ericsson, CR#0384 to TS38.413, Rel-15

- R3-202629 Selected PLMN ID in INITIAL UE MESSAGE for untrusted non-3GPP access, Ericsson, CR#0385 to TS38.413, Rel-16

- R3-203208 Selected PLMN ID for untrusted non-3GPP access, Huawei, CR#0391 to TS38.413, Rel-15

- R3-203209 Selected PLMN ID for untrusted non-3GPP access, Huawei, CR#0392 to TS38.413, Rel-16

- R3-203210 Selected PLMN ID for untrusted non-3GPP access, Huawei, CR#0005 to TS29.413, Rel-15