3GPP TSG-RAN3 Meeting #108-e	R3-203226

E-meeting, 01 - 11 June 2020

Title: 	(TP to NGAP BL CR#0173 “Introduction of Control Plane CIoT 5GS Optimisation for NB-IOT and eMTC”) Criticality of AMF CP RELOCATION INDICATION

Source: 	Huawei

Agenda item:	14.3.2.9

Document Type:	other

1. Introduction

In CIoT discussion, in order to support the RRC re-esatblishment for NB-IoT UEs using CP CIoT EPS optimization, two procedures were introduced, i.e. eNB CP Relocation, and MME CP Relocation, the first one only applies to NB-IoT, and the latter one applies to CIoT, therefore when connecting to 5GC, the AMF CP Relocation procedure is included in BL CR CR#0173.

The purpose of the AMF CP Relocation Indication procedure is to inform the NG-RAN node that the UE’s connection is to be relocated to another NG-RAN node, the NG-RAN node may initiate NAS Non Delivery Indication procedure(s) to report the non-delivery of any NAS PDUs previously received from the AMF.

Considering that the support of NAS recovery is important, in S1AP, the Criticality for that message is reject:

#### 9.1.4.24 MME CP RELOCATION INDICATION This message is sent by the MME to inform the eNB that the UE is to be relocated as described in TS. 36.300 [14]. Direction: MME eNB. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.1.1; YES; reject. MME UE S1AP ID; M; 9.2.3.3; YES; reject. eNB UE S1AP ID; M; 9.2.3.4; YES; reject.

mMECPRelocationIndication S1AP-ELEMENTARY-PROCEDURE ::= { INITIATING MESSAGE MMECPRelocationIndication PROCEDURE CODE id-MMECPRelocationIndication CRITICALITY reject }

It is noticed that the Criticality used in NGAP BL CR is ignore, it is better to update to align with S1AP.

The Text Proposal to NGAP BL CR#0173 is provided in Section 2.

2. Text Proposal

----Start of the First Change----

9.2.2.B	AMF CP RELOCATION INDICATION

This message is sent by the AMF to inform the NG-RAN node that the UE is to be relocated as described in TS. 38.300 [8].

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| S-NSSAI | O |  | 9.3.1.24 |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network | YES | ignore |

----Start of the Next Change----

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

//skip the unchanged part

downlinkRIMInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRIMInformationTransfer

PROCEDURE CODE			id-DownlinkRIMInformationTransfer

CRITICALITY				ignore

}

connectionEstablishmentIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConnectionEstablishmentIndication

PROCEDURE CODE			id-ConnectionEstablishmentIndication

CRITICALITY				reject

}

aMFCPRelocationIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFCPRelocationIndication

PROCEDURE CODE			id-AMFCPRelocationIndication

CRITICALITY				reject

}

END

-- ASN1STOP

----End of the Changes----