3GPP TSG-RAN WG3 Meeting #108-e	R3-203239

1 - 11 June 2020, E-Meeting

Agenda item:		20.2.2

Source:	LG Electronics Inc.

Title:	Remaining issues on F1 support for 5G sidelink resource modes

Document for:		Discussion

1. Introduction

In last meeting, some of the F1 issues were solved, but still there are some remaining issues to handle. This paper is to further investigate the open issues. The corresponding proposals and TP for BLCR are also provided.

2. Discussion

In last meeting, the following agreements were achieved:

  - For the sidelink resource request from CU to DU, the existing UE Context Setup/Modification procedures will be reused.

  - Introduce the new SidelinkUEInformation to the RRC container.

  - Introduce the UEAssistanceInformationEUTRA IE in the CU to DU RRC Information IE.

  - Update the SIBs naming to SIB 12, 13, and 14.

  - PDCP SN length is removed from SL DRB QoS, and the flow list mapped to DRB is kept.

  - Remove Range and PC5 link AMBR within PC5 QoS parameters

The following issues should be investigated more:

- The SL DRB QoS is kept as FFS and other FFSs within PC5 QoS parameters

- Define a new IE named PC5 link AMBR in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST messages with FFS.

- Other issues

Issue 1: FFS on whether to transmit SL DRB QoS to DU and other FFSs within PC5 QoS parameters

In the summary [3], the issue on whether SL DRB QoS is necessary or not in the DU was discussed. Most of the companies think that it should be passed to DU. We also have the same view for the following reasons:

Basically, the SDAP and PDCP related configuration of a SL DRB is configured by gNB-CU, while the RLC/MAC related configuration of the SL DRB is configured by gNB-DU. From CU to DU, SL DRB ID and flow mapped to SL DRB list are passed to DU, which means that DU is doing per DRB level radio configuration instead of per flow level. So the DRB level QoS should also be received from CU. If DU only relies on the per flow level PC5 QoS parameters from the UE, this means that DU makes a decision for a SL DRB QoS parameters if several flows are mapped to a SL DRB. In this way function of SDAP is moved to DU, which is not appropriate. Therefore, the SL DRB QoS should be passed to DU.

Proposal 1): To pass the SL DRB QoS parameters to DU and remove FFS.

On the format of SL DRB QoS parameter to DU, an example is to follow the format we did for Uu DRB QoS. The followings are from current 38.473.

| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |

9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow or to a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| … |  |  |  |  |  |  |

From the table above, it can be seen that the reference for Uu DRB QoS follows “QoS Flow Level QoS Parameters”. The other point is that “QoS Flow Level QoS Parameters” not only include the standardized 5QI but also contains the non-standardized 5QI.

On the other hand, from RAN2 38.331 (version 16.0.0), UE reports the SL QoS profile to CU in the SL UE Information message according to the following format:

#### –	SL-QoS-Profile

The IE SL-QoS-Profile is used to give the QoS parameters for a sidelink QoS flow.

SL-QoS-Profile information element

-- ASN1START

-- TAG-SL-QOS-PROFILE-START

SL-QoS-Profile-r16 ::=        SEQUENCE {

sl-PQI-r16                    SL-PQI-r16                                                  OPTIONAL,

sl-GFBR-r16                   INTEGER (0..4000000000)                                     OPTIONAL,

sl-MFBR-r16                   INTEGER (0..4000000000)                                     OPTIONAL,

sl-Range-r16                  INTEGER (1..1000)                                           OPTIONAL,

...

}

SL-PQI-r16 ::=                CHOICE {

sl-StandardizedPQI-r16        INTEGER (1..83),

sl-Non-StandardizedPQI-r16    SEQUENCE {

sl-ResourceType-r16           ENUMERATED {gbr, non-GBR, delayCriticalGBR, spare1}     OPTIONAL,

sl-PriorityLevel-r16          INTEGER (0..7)                                          OPTIONAL,

sl-PacketDelayBudget-r16      INTEGER (0..1023)                                       OPTIONAL,

sl-PacketErrorRate-r16        INTEGER (0..9)                                          OPTIONAL,

sl-AveragingWindow-r16        INTEGER (0..4095)                                       OPTIONAL,

sl-MaxDataBurstVolume-r16     INTEGER (0..4095)                                       OPTIONAL,

...

}

}

-- TAG-SL-QOS-PROFILE-STOP

-- ASN1STOP

It can be seen that from UE to CU both the standardized PQI and the non-standardized PQI are included in the SL UE Information message. Thus, from CU to DU, the SL DRB QoS should also include both the standardized PQI and the non-standardized PQI. The following is an example:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic PQI |  |  |  |  | - |  |
| >>Non Dynamic PQI Descriptor | M |  | 9.3.1.xx8 |  | - |  |
| >Dynamic PQI |  |  |  |  | - |  |
| >>Dynamic PQI Descriptor | M |  | 9.3.1.xx9 |  | - |  |
| PC5 Flow Bit Rates | O |  |  | Only applies for GBR QoS Flows. | - |  |
| >Guaranteed Flow Bit Rate | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. | - |  |
| >Maximum Flow Bit Rate | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. | - |  |

Proposal 2): To revise the format of PC5 QoS parameters so that both the standardized PQI and the non-standardized PQI are included as a choice by referring to Uu case in 9.3.1.45.

Issue 2: FFS on whether defining a new IE named PC5 link AMBR in the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST messages

According to SA2 spec TS 23.287, the PC5 QoS parameters are given as follows:

5.4.2	PC5 QoS parameters

5.4.2.1	PQI

A PQI is a special 5QI, as defined in clause 5.7.2.1 of TS 23.501 [6], and is used as a reference to PC5 QoS characteristics defined in clause 5.4.3, i.e. parameters that control QoS forwarding treatment for the packets over PC5 reference point.

Standardized PQI values have one-to-one mapping to a standardized combination of PC5 QoS characteristics as specified in Table 5.4.4-1.

5.4.2.2	PC5 Flow Bit Rates

For GBR QoS Flows only, the following additional PC5 QoS parameters exist:

-	Guaranteed Flow Bit Rate (GFBR);

-	Maximum Flow Bit Rate (MFBR).

The GFBR and MFBR as defined in clause 5.7.2.5 of TS 23.501 [6] are used for bit rate control on PC5 reference point over the Averaging Time Window. For PC5 communication, the same GFBR and MFBR are used for both directions.

5.4.2.3	PC5 Link Aggregated Bit Rates

A PC5 unicast link is associated with the following aggregate rate limit QoS parameter:

-	per link Aggregate Maximum Bit Rate (PC5 LINK-AMBR).

The PC5 LINK-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR QoS Flows with a peer UE over PC5 unicast link. The PC5 LINK-AMBR is measured over an AMBR averaging window which is a standardized value. The PC5 LINK-AMBR is not applicable to GBR QoS Flows. PC5 LINK-AMBR is applied to one PC5 unicast link, which means aggregate bit rate of one PC5 unicast link should not exceed PC5 LINK-AMBR.

NOTE:	The AMBR averaging window is only applied to PC5 LINK-AMBR measurement.

Therefore, to align with the agreement of SA2, in the past meetings we have agreed to introduce PC5 QoS parameters for NG/Xn and S1/X2 interfaces, which include PC5 Link Aggregated Bit Rates. So now the parameters can be passed to RAN side.

However, in case of CU/DU split, it is necessary to check whether the parameters should be passed to DU.

Fig. 1: Example of PC5 Unicast Links (Figure 5.2.1.4-1 in TS 23.287)

According to TS 23.287 shown in Fig.1 above, one unicast link may consist of several flows between the UE A and UE B. PC5 Link Aggregated Bit Rate is to give a limit for UE in one link. This parameter is very similar to UE sidelink AMBR, which was agreed to be passed to DU since DU is in charge of the low layers and only it knows the exact bit rate from flow/link/UE point of view. So it should be passed to DU for performing the limitation.

There was several concerns during email discussion in last meeting.

Firstly, how DU can perform the limit for a link based on PC5 Link Aggregated Bit Rate?  Basically this parameter only applies to unicast and non-GBR. The cast type of a flow and whether it is GBR or non-GBR can be known by DU through the SidelinkUEInformation reported by UE, which goes to DU finally.

Secondly, on whether CU can perform the limit based on PC5 Link Aggregated Bit Rate for a link? Basically, CU can perform the authorization on whether a QoS flow requested by UE is included in the PC5 QoS parameters received from CN. However, it is not possible for CU to perform the bit limit since this is low layer parameter.

Thirdly, there was a concern that this is just subscription related information to check if what the UE requested is inline or not with its subscription. However, we have different understanding. UE does not report PC5 Link Aggregated Bit Rate itself to CU/DU, which cannot perform the verification only. This parameter is similar to UE sidelink AMBR, for which the function requested to DU is also very similar.

Based on the analysis above, the following proposal is suggested:

Proposal 3): To add PC5 Link Aggregated Bit Rates IE into the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message and the corresponding descriptions as in [4].

Issue 3: other issues

In last meeting, the following agreement has been achieved:

- For the sidelink resource request from CU to DU, the existing UE Context Setup/Modification procedures will be reused.

- Introduce the new SidelinkUEInformation to the RRC container.

However, the specification has not been changed to reflect the agreement above. Especially on CG-ConfigInfo, which was specially only for DC. Now if it is used for SidelinkUEInformation, descriptions should be added in the UE Context Setup and UE Context Modification sections for clarifying the usage.

Proposal 4): To add descriptions in UE Context Setup and UE Context Modification sections for clarifying the usage of CG-ConfigInfo for SidelinkUEInformation as in [4].

Proposal 5): To agree the TP in [4] for BL CR TS 38.473.

3. Conclusion

In this contribution, the remaining issues on F1 interface for supporting V2X were investigated. The following proposals are suggested to RAN3:

Proposal 1): To pass the SL DRB QoS parameters to DU and remove FFS.

Proposal 2): To revise the format of PC5 QoS parameters so that both the standardized PQI and the non-standardized PQI are included as a choice by referring to Uu case in 9.3.1.45.

Proposal 3): To add PC5 Link Aggregated Bit Rates IE into the UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message and the corresponding descriptions as in [4].

Proposal 4): To add descriptions in UE Context Setup and UE Context Modification sections for clarifying the usage of CG-ConfigInfo for SidelinkUEInformation as in [4].

Proposal 5): To agree the TP in [4] for BL CR TS 38.473.

4. References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- 3GPP TR 38.885 V1.0.0: Study on Vehicle-to-Everything

- R3-202542, “Summary of offline discussion on F1 impacts for V2X”, Huawei

- R3-203240, “(TP for BLCR 38.473) remaining Issues on F1”, LG Electronics