3GPP TSG-RAN WG3 #108-e	R3-203268

# 1-12 June 2020

# E-Meeting

Agenda item:	16.6

Source: 	ZTE

Title: 	Open Issues of NPN on E1

Document for:	Discussion and Approval

1 Introduction

In the last RAN3#108e meeting, it was acknowledged that E1 bearer management procedures support indication of the NID. However, the PNI-NPN specific impacts and cause value on the E1 needs to be further discussed. In this contribution, we further discuss the open issue on E1 and providing the corresponding TP in [1] based on the BLCR [2] for TS38.463.

2 Discussion

2.1 PNI-NPN Aspects

It has been acknowledged that SNPN support information is provided at E1 setup, and BLCR was updated accordingly. For the PNI-NPN aspects, a list of the supported CAG per PLMN configured by the gNB-CU-CP shall be sent to the gNB-CU-CP upon the similar reason as the SNPN.  The gNB-CU-CP would select the appropriate gNB-CU-UP according to the PNI-NPN support information indicated by the gNB-CU-CP and its allowed CAG list.

Proposal 1: The gNB-CU-UP shall provide the supported CAGs per PLMN to the gNB-CU-CP.

2.2 Cause Value

In the last RAN3#107bis-e meeting,  it was concluded that whether to introduce cause values over E1 for E1 setup failure need to further discussed. It’s positive for us to introduce the cause value, i.e, “SNPN not supported” and “CAG not supported” for E1 setup failure case. For SNPN case, the E1 interface will be not established if there is no common NIDs between the gNB-CU-CP and gNB-CU-UP, namely the received NIDs from gNB-CU-UP may not be recognized by the gNB-CU-CP. For the PNI-NPN case with the same reason as the SNPN, the  gNB-CU-CP shall response a E1 SETUP FAILURE with an appropriate cause value.

Proposal 2:  Add new E1 cause value i.e., “SNPN not supported”, “CAG not supported” for E1 setup failure case.

3 Conclusions

Proposal 1:  The gNB-CU-UP shall provide the supported NIDs/CAG IDs per PLMN to gNB-CU-CP via E1AP.

Proposal 2:  Add new E1 cause value i.e., “SNPN not supported”, “CAG not supported” for E1 setup failure case.

4 References

- R3-203269, TP for NPN support to TS38.463,  ZTE

- R3-203063, BLCR for TS38.463 on NPN, ZTE