3GPP TSG-RAN WG3 Meeting #108-e	R3-203276

E-meeting, 1-12 Jun 2020

Agenda item:	15.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_Mob_enh BL CR for TS 38.423): Consideration on the fallback indication

Document for:	Endorsement

# 1	Introduction

At RAN3 #107-e meeting, an indication was agreed to inform the source whether the target accepted the HO as a DAPS HO, or as a classic HO. This was further discussed at RAN3 #107-bis-e, but not concluded. Here, we remain the arguments used then.

# 2	Introduction

In the BL CR to XnAP, the fallback indicator has 2 values: “DAPS HO accepted” and “fallback to legacy HO”. The first problem is the naming – there is no “legacy” HO, because a classic HO exists and is still used.

Second issue concerns the usage: the DAPS Response Information IE must be optional for backward-compatibility reasons. And also, for backward-compatibility reasons, an acknowledgement of a HO request without the indication means the target accepted a classic HO (perhaps, because it does not understand DAPS request – the indication in the request is “ignore”). Therefore, the “fallback to legacy HO” code is not needed – it is redundant with not including the flag at all.

Proposal: The redundant “fallback to legacy HO” code is removed to maintain functional backward-compatibility of the HO procedure.

# 3	Text proposal

The proposed change is implemented on the BL CR for XnAP [R3-202909].

*** First change, skipped text not changed ***

#### 9.2.1.EE	DAPS Response Information

The DAPS Response Indicator IE indicates that the response to a requested DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, …) | Indicates if the DAPS Handover is accepted |

*** Next change, skipped text not changed ***

DAPSResponseInfo ::= SEQUENCE {

dapsResponseIndicator					ENUMERATED {daps-HO-accepted, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSResponseInfo-ExtIEs} } OPTIONAL,

...

}

*** Remaining text not changed ***