3GPP TSG-RAN WG3 Meeting #108-e	R3-203277

E-meeting, 1-12 Jun 2020

Agenda item:	15.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_Mob_enh BL CR for TS 38.413): Consideration on the fallback indication

Document for:	Endorsement

# 1	Introduction

At RAN3 #107-e meeting, an indication was agreed to inform the source whether the target accepted the HO as a DAPS HO, or as a classic HO. This topic is further discussed in the context of NR in [R3-203276].

# 2	Text proposal

The proposed change is implemented on the BL CR for NGAP [R3-202930].

*** First change, skipped text not changed ***

#### 9.2.1.EE	DAPS Response Information

The DAPS Response Indicator IE indicates that the response to a requested DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, …) | Indicates if the DAPS Handover is accepted |

*** Next change, skipped text not changed ***

DAPSResponseInfo ::= SEQUENCE {

dapsResponseIndicator					ENUMERATED {daps-HO-accepted, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSResponseInfo-ExtIEs} } OPTIONAL,

...

}

*** Remaining text not changed ***