3GPP TSG-RAN WG3 Meeting #108-e	R3-203278

E-meeting, 1-12 Jun 2020

Agenda item:	15.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for LTE_feMob BL CR for TS 36.423): Consideration on the fallback indication

Document for:	Endorsement

# 1	Introduction

At RAN3 #107-e meeting, an indication was agreed to inform the source whether the target accepted the HO as a DAPS HO, or as a classic HO. This was further discussed at RAN3 #107-bis-e, but not concluded. Here, we remain the arguments used then.

# 2	Introduction

In the BL CR to X2AP, the fallback indicator has 3 values: “DAPS HO accepted”, “fallback to legacy HO” and “fallback to Rel14 MBB”.

Fallback to legacy HO:

The first problem is the naming – there is no “legacy” HO, because a classic HO exists and is still used. Second issue concerns the usage: the DAPS Response Information IE must be optional for backward-compatibility reasons. And also, for backward-compatibility reasons, an acknowledgement of a HO request without the indication means the target accepted a classic HO (perhaps, because it does not understand DAPS request – the indication in the request is “ignore”). Therefore, the “fallback to legacy HO” code is not needed – it is redundant with not including the flag at all.

Proposal 1: The redundant “fallback to legacy HO” code is removed to maintain functional backward-compatibility of the HO procedure.

Fallback to Rel14 MBB:

This is valid response from the target – there is no other way to indicate that the target intends to accept the HO as a Rel.14 MBB HO. However, the question is: is it needed at all?

At RAN3 #107-bis-e, this topic was discussed briefly and it seem that this could work, if the source and the target both support the MBB HO. With this in mind, the code point for the MBB HO may remain in LTE. However, popularity of the MBB is low so enabling the fallback from DAPS HO to MBB HO will not be used often. Therefore, to keep the LTE and NR solutions similar (and thus easy to implement), the code point for MBB HO could be left out.

It may be added in future though, if found needed based on actual implementations of DAPS HO.

Proposal 2: The “fallback to Rel14 MBB” code is removed for the time being. The fallback to MBB HO may be reconsidered in future, once proved needed in actual implementations of DAPS HO.

# 3	Text proposal

The proposed change is implemented on the BL CR for X2AP [R3-202888].

*** First change, skipped text not changed ***

### 9.2.y	DAPS Response Information

The DAPS Response Indicator IE indicates the response to a requested DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, …) | Indicates if the DAPS Handover is accepted |

*** Next change, skipped text not changed ***

DAPSResponseInfo ::= SEQUENCE {

dAPSResponseIndicator					ENUMERATED { daps-HO-accepted,...},

iE-Extensions				ProtocolExtensionContainer { { DAPSResponseInfo-ExtIEs} } OPTIONAL,

...

}

*** Remaining text not changed ***