3GPP TSG-RAN WG3 Meeting #108-e	R3-203325

E-meeting, Jun 1st – Jun 12th, 2020

Agenda item:	15.3.1.3

Source:	Qualcomm Incorporated

Title:	(TP for NR_Mob_enh-Core BL CR for TS 38.423) Remaining issues in CHO modification

WID/SID:	NR_Mob_enh-Core – Release 16

Document for:	Discussion and Decision

# Introduction

RAN3 #107-e meeting agreement and FFS item on target-initiated CHO modification:

Reuse existing CHO Cancel and HO Request to enable target node to inform source node the change of reserved CHO resources for the UE.

It is FFS whether an explicit indicator is needed in conditional HO cancel message or a new cause value is sufficient.

It was agreed in RAN3 #107-e meeting [1] that target-initiated CHO modification procedure is supported and that CHO Cancel followed by Handover Request can be used for the procedure. The agreement is as follows:

RAN3 #107-bis-e meeting FFS items on CHO modification:

Discussion on possible information to the source on the target’s willingness to keep 2 configs.

Discussion on indication of possible re-initialisation of the CHO in the target-initiated cancellation.

In this contribution we address the FFS item above on whether an explicit indicator is needed in the CHO Cancel message or a new cause value is sufficient. We also address the following two FFS items on CHO modification identified in RAN3 #107-bis-e meeting [2].

# On whether target needs to indicate source if it can keep two configurations upon CHO modification

In an email discussion (#20) [2] in the last RAN3 #107-bis-e meeting, it was discussed whether the target needs to indicate to the source whether it can keep two configurations for some time upon CHO modification. This was to handle the race condition scenario whereby UE accesses the target with the old configuration when CHO gets triggered before it receives the RRC reconfiguration containing the new configuration.

In the email discussion, the following points were noted:

- Source knows about the target implementation whether the target can keep two configurations for some time upon CHO modification. The source knows about target implementation either (a) from OAM or (b) through explicit signalling by target to source.

There are two sub-cases:

  - Target is capable of storing two configurations: In this case, assuming that the target can determine which configuration the UE is using during CHO execution, the source is not required to cancel the configuration at UE before starting the modification procedure.

  - Target does not store two configurations: In this case, the source should cancel the configuration at UE before starting modification to ensure that CHO execution does not fail in the race condition scenario mentioned above.

- Source does not know about the target implementation: In this case, the source should again cancel the configuration at UE before starting modification to ensure there is no failure in the race condition scenario.

From the above discussion, it might seem that the fail-safe way to ensure that CHO execution does not fail is for the source to always cancel the configuration at UE before starting modification. However, that may not always be a desirable solution because of the following reasons:

- Cancelling the configuration at UE before starting modification introduces delay in the modification procedure (the source waits for a response after sending an RRC reconfiguration to the UE to cancel the configuration);

- Cancelling the configuration at UE may impact performance because UE may miss performing a handover when it should be doing so. UE may then be forced to continue to be served by a source cell offering poor coverage.

Observation. The solution where the source always cancels configuration at UE before starting modification with target may not always be desirable because of the following reasons:

- Cancelling the configuration at UE before starting modification introduces delay in the modification procedure (the source waits for a response to the RRC reconfiguration message sent to UE to cancel the configuration).

- Cancelling the configuration at UE may impact performance because UE may miss performing a handover when it should be doing so. UE may then be forced to continue to be served by a source cell offering poor coverage.

Given the above observation, it seems that the source should know about the target implementation whether the target can keep two configurations for some time upon CHO modification. Among the two options to obtain this information mentioned above we slightly prefer the explicit signalling from target to source because it incurs minimum signalling cost and is a simple solution.

Proposal 1. The target explicitly indicates to source whether it can keep two configurations for some time upon CHO modification. The target provides this information in the HANDOVER REQUEST ACKNOWLEDGE message during initial CHO preparation.

# CHO Cancel message in target-initiated CHO modification

It was agreed in RAN3 previously to support target-initiated CHO modification procedure. Target-initiated CHO modification may be needed if there is any change of reserved CHO resources for UE, e.g., CFRA resources (preambles), and also if the target node decides to change the parameters or other parts of the target cell configuration provided to the UE due to change of reserved CHO resources. It was agreed that the procedure would consist of the following:

- The target node sends a CHO Cancel message to the source with an indication which differentiates handling by the source node from a regular CHO Cancel message;

- The source initiates a new CHO preparation procedure to the target by sending the Handover Request message.

We think that since the general mechanism of using cause values in various Xn/X2 messages is already available, we could reuse it in this context as in the following proposal.

Proposal 2. In target-initiated CHO modification, the target sends a CHO Cancel message to the source including a new cause value “CHO modification required”.

# Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following observations and proposals:

Observation. The solution where the source always cancels configuration at UE before starting modification with target may not always be desirable because of the following reasons:

- Cancelling the configuration at UE before starting modification introduces delay in the modification procedure (the source waits for response to the RRC reconfiguration message sent to UE to cancel the configuration).

- Cancelling the configuration at UE may impact performance because UE may miss performing a handover when it should be doing so. UE may then be forced to continue to be served by a source cell offering poor coverage.

Proposal 1. The target explicitly indicates to source whether it can keep two configurations for some time upon CHO modification. The target provides this information in the HANDOVER REQUEST ACKNOWLEDGE message during initial CHO preparation.

Proposal 2. In target-initiated CHO modification, the target sends a CHO Cancel message to the source including a new cause value “CHO modification required”.

In the annex below, we highlight the changes required in BL CR to the XnAP specification, TS 38.423.

# Annex: TP for BL CR for TS 38.423

------------------------------ Change #1, Section 8.2.B ------------------------------------------------------------------------------

### 8.2.B	Conditional Handover Cancel

#### 8.2.B.1	General

The Conditional Handover Cancel procedure is used to enable a target NG-RAN node to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

8.2.B.2	Successful Operation

Figure 8.2.B.2-1: Conditional Handover Cancel, successful operation

The target NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source NG-RAN node. The target NG-RAN node shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

At the reception of the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved for candidate cells associated to the UE-associated signalling identified by the Source NG-RAN node UE XnAP ID IE and the Target NG-RAN node UE XnAP ID IE.If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

If the cause value indicated in the CONDITIONAL HANDOVER CANCEL message is “CHO modification required”, at the reception of the message, the source NG-RAN node shall initiate a conditional handover preparation procedure with the target NG-RAN node.

#### 8.2.B.3	Unsuccessful Operation

Not applicable.

#### 8.2.B.4	Abnormal Conditions

If the CONDITIONAL HANDOVER CANCEL message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the CONDITIONAL HANDOVER CANCEL message were not prepared using the same UE-associated signaling connection, the source NG-RAN node shall ignore those non-associated candidate cells.

#### 9.1.1.BB	CONDITIONAL HANDOVER CANCEL

This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared conditional handover.

#### Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional handover. Value is 16. |

#### ------------------------------ End of change #1, Section 8.2.B ---------------------------------------------------------------------

#### ------------------------------ Change #2, Section 9.1.1.2 ----------------------------------------------------------------------------

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

#### Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Reponse Information | O |  | 9.2.1.EE |  | YES | reject |
| Conditional Handover Information | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message |  |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.FF |  |  |  |
| >Previous Configuration Kept Upon CHO Modification | M |  | ENUMERATED (true, false, …) | Indicates whether or not the target NG-RAN node keeps the previous target configuration upon CHO modification. |  |  |

#### ------------------------------ End of change #2, Section 9.1.1.2 -------------------------------------------------------------------

#### ------------------------------ Change #3, Section 9.2.3.2 ---------------------------------------------------------------------------

#### 9.2.3.2	Cause

#### The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, CHO modification required, Unspecified, …, UE Context ID not known, Non-relocation of context) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

#### The meaning of the different cause values is specified in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Cell not Available | The concerned cell is not available. |
| Handover Desirable for Radio Reasons | The reason for requesting handover is radio related. |
| Handover Target not Allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| Invalid AMF Set ID | The target NG-RAN node doesn’t belong to the same AMF Set of the source NG-RAN node, i.e. NG handovers should be attempted instead. |
| No Radio Resources Available in Target Cell | The target cell doesn’t have sufficient radio resources available. |
| Partial Handover | Provides a reason for the handover cancellation. The target NG-RAN node did not admit all PDU Sessions included in the HANDOVER REQUEST and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Reduce Load in Serving Cell | Load in serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| Resource Optimisation Handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Time Critical Handover | Handover is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| TXnRELOCoverall Expiry | The reason for the action is expiry of timer TXnRELOCoverall. |
| TXnRELOCprep Expiry | Handover Preparation procedure is cancelled when timer TXnRELOCprep expires. |
| Unknown GUAMI ID | The target NG-RAN node belongs to the same AMF Set of the source NG-RAN node and recognizes the AMF Set ID. However, the GUAMI value is unknown to the target NG-RAN node. |
| Unknown Local NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node does not recognise the local NG-RAN node UE XnAP ID. |
| Inconsistent Remote NG-RAN node UE XnAP ID | The action failed because the receiving NG-RAN node considers that the received remote NG-RAN node UE XnAP ID is inconsistent.. |
| Encryption And/Or Integrity Protection Algorithms Not Supported | The target NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided to the NG-RAN node. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Switch Off Ongoing | The reason for the action is an ongoing switch off i.e. the concerned cell will be switched off after offloading and not be available. It aides the receiving NG-RAN node in taking subsequent actions, e.g. selecting the target cell for subsequent handovers. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| TXnDCoverall Expiry | The reason for the action is expiry of timer TXnDCoverall. |
| TXnDCprep Expiry | The reason for the action is expiry of timer TXnDCprep |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. In the current version of this specification applicable for Dual Connectivity only. |
| Reduce Load | Load in the cell(group) served by the requesting node needs to be reduced. In the current version of this specification applicable for Dual Connectivity only. |
| Resource Optimisation | The reason for requesting this action is to improve the load distribution with the neighbour cells. In the current version of this specification applicable for Dual Connectivity only. |
| Time Critical action | The action is requested for time critical reason i.e. this cause value is reserved to represent all critical cases where radio resources are likely to be dropped if the requested action is not performed. In the current version of this specification applicable for Dual Connectivity only. |
| Target not Allowed | Requested action towards the indicated target cell is not allowed for the UE in question. In the current version of this specification applicable for Dual Connectivity only. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. In the current version of this specification applicable for Dual Connectivity only. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. In the current version of this specification applicable for Dual Connectivity only. |
| Encryption Algorithms Not Supported | The requested NG-RAN node is unable to support any of the encryption algorithms supported by the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. In the current version of this specification applicable for Dual Connectivity only. |
| RRM purpose | The procedure is initiated due to node internal RRM purposes. In the current version of this specification applicable for Dual Connectivity only. |
| Improve User Bit Rate | The reason for requesting this action is to improve the user bit rate. In the current version of this specification applicable for Dual Connectivity only. |
| User Inactivity | The action is requested due to user inactivity on all PDU Sessions. The action may be performed on several levels: - on UE Context level, if NG is requested to be released in order to optimise the radio resources; or S-NG-RAN node didn’t see activity on the PDU session recently. - on PDU Session Resource or DRB or QoS flow level, e.g. if Activity Notification indicate lack of activity In the current version of this specification applicable for Dual Connectivity only. |
| Radio Connection With UE Lost | The action is requested due to losing the radio connection to the UE. In the current version of this specification applicable for Dual Connectivity only. |
| Failure in the Radio Interface Procedure | Radio interface procedure has failed. In the current version of this specification applicable for Dual Connectivity only. |
| Bearer Option not Supported | The requested bearer option is not supported by the sending node. In the current version of this specification applicable for Dual Connectivity only. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE Maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| CP Integrity Protection Failure | The request is not accepted due to failed control plane integrity protection. |
| UP Integrity Protection Failure | The procedure is initiated because the SN (hosting node) detected an Integrity Protection failure in the UL PDU coming from the MN. |
| Slice(s) not supported by NG-RAN | The failure is due to slice(s) not supported by the NG-RAN node. |
| MN Mobility | The procedure is initiated due to relocation of the M-NG-RAN node UE context. |
| SN Mobility | The procedure is initiated due to relocation of the S-NG-RAN node UE context. |
| Count reaches max value, | Indicates the PDCP COUNT for UL or DL reached the max value and the bearer may be released. |
| Unknown Old NG-RAN node UE XnAP ID | The action failed because the Old NG-RAN node UE XnAP ID or the S-NG-RAN node UE XnAP ID is unknown. |
| PDCP Overload | The procedure is initiated due to PDCP resource limitation. |
| DRB ID not available | The action failed because the M-NG-RAN node is not able to provide additional DRB IDs to the S-NG-RAN node. |
| CHO modification required | The procedure is initiated because the target NG-RAN node has determined that modification of a prepared conditional handover is required. |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UE Context ID not known | The context retrieval procedure cannot be performed because the UE context cannot be identified. |
| Non-relocation of context | The context retrieval procedure is not performed because the old RAN node has decided not to relocate the UE context. |

#### ------------------------------ No further changes required in this section -------------------------------------------------

------------------------------ End of change #3, Section 9.2.3.2 --------------------------------------------------------------

# References

[1] Chairman’s notes, RAN3 #107-e meeting.

[2] Chairman’s notes, RAN3 #107-bis-e meeting.