3GPP TSG-RAN WG3 Meeting #108-e	R3-203326

E-meeting, Jun 1st – Jun 12th, 2020

Agenda item:	15.2.1.2

Source:	Qualcomm Incorporated

Title:	(TP for LTE_feMob-Core BL CR for TS 36.423) Remaining issues in DAPS handover over X2

WID/SID:	LTE_feMob-Core – Release 16

Document for:	Discussion and Decision

# Introduction

It was agreed in the previous RAN3 #107-e meeting [1] that DAPS handover (HO) indication per E-RAB/DRB will be supported in Xn/X2 signaling of DAPS HO request. It was also agreed that a fallback mechanism will be supported in case the target node is not able to support DAPS HO.

RAN3 #107-bis-e agreement on DAPS HO indicator:

Do not introduce one shot DAPS HO proposal from source. DAPS HO proposal is per E-RAB/DRB

Furthermore, in RAN3 #107-bis-e meeting [2], the following was confirmed regarding DAPS HO indication per E-RAB/DRB in Xn/X2 signaling of DAPS handover request.

In this contribution, we discuss the following remaining issues in X2 based DAPS HO identified in RAN3 #107-bis-e meeting [2].

- Whether DAPS response in X2AP is per E-RAB or one shot for all requested E-RABs. If per E-RAB, encode under E-RABs Admitted List; if one shot, encode within HO REQ ACK.

- Whether or not the target explicitly indicates in the DAPS Response Info IE, if admitted as classic HO instead.

- Whether to introduce “fallback to rel-14 MBB” in X2AP.

# Target node response to a DAPS handover request

Regarding the question whether the target node should provide a per DRB response to the per DRB DAPS HO request, we have the following observation.

Observation 1. Depending on the current traffic load the target node is handling, it may be able to support DAPS HO for only a subset of DRBs for which DAPS HO is requested by the source node in the Handover Request message.

Consequently, the target node should be allowed to indicate for each DRB it accepts for handover, whether it can support a DAPS HO.

Proposal 1. The target node provides a per DRB response in the Handover Request Acknowledge message to the per DRB request for DAPS HO in Handover Request message.

## Regarding fallback to Rel-14 MBB handover

We note the following facts regarding Rel-14 MBB handover:

- If Rel-14 MBB handover is configured, the connection to the source cell is maintained after the reception of RRCConnectionReconfiguration message with mobilityControlInformation IE by the UE, and before the UE executes initial uplink transmission to the target cell. If Rel-14 MBB handover is configured, the source eNB decides when to stop transmitting to the UE.

- The target eNB behaviour is the same for Rel-14 MBB handover as for legacy handover.

- The source eNB in HandoverPreparationInformation included in HANDOVER REQUEST message, requests the target eNB to add the Make-Before-Break indication (makeBeforeBreakReq-r14 IE) in the mobilityControlInfo IE included in the RRCConnectionReconfiguration message sent to the UE. The target provides the Make-Before-Break indication (makeBeforeBreak-r14 IE) in the Target eNB To Source eNB Transparent Container in HANDOVER REQUEST ACKNOWLEDGE message if it receives the request. The Rel-14 MBB HO indication is for all DRBs.

We thus have the following observations.

Observation 2. The UE is configured with Rel-14 MBB handover by target eNB if the source eNB in HandoverPreparationInformation included in HANDOVER REQUEST message requests the target eNB to add the Make-Before-Break indication in the RRCConnectionReconfiguration message (Handover Command) to be sent to the UE.

Observation 3. There already exists signalling in the HANDOVER REQUEST and HANDOVER REQUEST ACKNOWLEDGE messages for the source to indicate to the target support for Rel-14 MBB handover and for the target to provide in the response Rel-14 MBB handover indication.

Observation 4. The Rel-14 MBB handover indication is for all DRBs.

Based on the above observations, we have the following two proposals.

Proposal 2. In the target response to DAPS handover request received from source, the explicit indication of fallback to Rel-14 MBB is not needed.

Proposal 3. If the target accepts the DAPS HO for a DRB, it includes the following response indication for the DRB in the Handover Request Acknowledge message: DAPS handover accepted.

If the target does not accept the DAPS HO for a DRB, it does not explicitly indicate support of legacy HO fallback in the Handover Request Acknowledge message.

# Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following proposals:

Observation 1. Depending on the current traffic load the target node is handling, it may be able to support DAPS HO for only a subset of DRBs for which DAPS HO is requested by the source node in the Handover Request message.

Proposal 1. The target node provides a per DRB response in the Handover Request Acknowledge message to the per DRB request for DAPS HO in Handover Request message.

Observation 2. The UE is configured with Rel-14 MBB handover by target eNB if the source eNB in HandoverPreparationInformation included in HANDOVER REQUEST message requests the target eNB to add the Make-Before-Break indication in the RRCConnectionReconfiguration message (Handover Command) to be sent to the UE.

Observation 3. There already exists signalling in the HANDOVER REQUEST and HANDOVER REQUEST ACKNOWLEDGE messages for the source to indicate to the target support for Rel-14 MBB handover and for the target to provide in the response Rel-14 MBB handover indication.

Observation 4. The Rel-14 MBB handover indication is for all DRBs.

Proposal 2. In the target response to DAPS handover request received from source, the explicit indication of fallback to Rel-14 MBB is not needed.

Proposal 3. If the target accepts the DAPS HO for a DRB, it includes the following response indication for the DRB in the Handover Request Acknowledge message: DAPS handover accepted.

If the target does not accept the DAPS HO for a DRB, it does not explicitly indicate support of legacy HO fallback in the Handover Request Acknowledge message.

In the annex below, we highlight the changes required in the BL CR to TS 36.423.

# Annex: TP for BL CR for TS 36.423

------------------------------ Change #1, Section 9.1.1.2 --------------------------------------------------------------------------

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target eNB to inform the source eNB about the prepared resources at the target.

#### Direction: target eNB  source eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | YES | ignore |
| E-RABs Admitted List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer. used for forwarding of DL PDUs | – |  |
| >>DAPS Response Information | O |  | 9.2.y |  |  |  |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| Target eNB To Source eNB Transparent Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 in TS 36.331 [9] | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.85 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | YES | reject |
| WT UE Context Kept Indicator | O |  | UE Context Kept Indicator 9.2.85 | Indicates that the WT has acknowledged to keep the UE context | YES | ignore |
| Conditional Handover Information | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | ECGI 9.2.14 | Target cell indicated in the corresponding HANDOVER REQUEST message | YES | reject |
| >Maximum Number of CHO Preparations | O |  | 9.2.C |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

------------------------------ End of change #1, Section 9.1.1.2 ------------------------------------------------------------------

------------------------------ Change #2, Section 9.2.y -------------------------------------------------------------------------

### 9.2.y	DAPS Response Information

### The DAPS Response Indicator IE indicates the response to a requested DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, …) | Indicates if the DAPS Handover is accepted |

------------------------------ End of change #2, Section 9.2.y ----------------------------------------------------------------

# References

[1] Chairman’s notes, RAN3 #107-e.

[2] Chairman’s notes, RAN3 #107-bis-e.