3GPP TSG-RAN WG3 Meeting #108-e	R3-203327

E-meeting, Jun 1st – Jun 12th, 2020

Agenda item:	15.2.1.2

Source:	Qualcomm Incorporated

Title:	(TP for NR_Mob_enh-Core BL CR for TS 38.423) Remaining issues in DAPS handover over Xn

WID/SID:	NR_Mob_enh-Core – Release 16

Document for:	Discussion and Decision

# Introduction

It was agreed in the previous RAN3 #107-e meeting [1] that DAPS handover (HO) indication per E-RAB/DRB would be supported in Xn/X2 signaling of DAPS HO request. It was also agreed that a fallback mechanism will be supported in case the target node is not able to support DAPS HO.

RAN3 #107-bis-e agreement on DAPS HO indicator:

Do not introduce one shot DAPS HO proposal from source. DAPS HO proposal is per E-RAB/DRB

Furthermore, in RAN3 #107-bis-e meeting [2], the following was confirmed regarding DAPS handover indication per E-RAB/DRB in Xn/X2 signaling of DAPS handover request.

In this contribution, we discuss the following remaining issues in Xn based DAPS HO identified in RAN3 #107-bis-e meeting [2].

- Whether DAPS response in XnAP is per DRB or one shot for all requested DRBs. If one shot, encode within HO REQ ACK.

- Whether or not the target explicitly indicates in the DAPS Response Info IE, if admitted as classic HO instead.

# Target node response to a DAPS handover request

Regarding the question whether the target node should provide a per DRB response to the per DRB DAPS HO request, we have the following observation.

Observation. Depending on the current traffic load the target node is handling, it may be able to support DAPS HO for only a subset of DRBs for which DAPS HO is requested by the source node in the Handover Request message.

Consequently, the target node should be allowed to indicate for each DRB it accepts for handover, whether it can support a DAPS HO.

Proposal 1. The target node provides a per DRB response in the Handover Request Acknowledge message to the per DRB request for DAPS HO in Handover Request message.

Regarding whether the target should explicitly indicate the fallback mechanism of legacy HO in case it is not able to support DAPS HO, since there is only one possible fallback mechanism over XnAP, i.e., legacy HO, it is not required to indicate that explicitly.

Proposal 2. If the target node does not accept the DAPS HO for a DRB, it does not explicitly indicate support of legacy HO fallback in the Handover Request Acknowledge message. The only valid per DRB response indication in the Handover Request Acknowledge is thus:

- DAPS handover accepted.

As usual, if the target node does not admit at least one PDU session resource, the target node transmits a Handover Preparation Failure message to the source node to indicate reject for the handover.

# Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following proposals:

Observation. Depending on the current traffic load the target node is handling, it may be able to support DAPS HO for only a subset of DRBs for which DAPS HO is requested by the source node in the Handover Request message.

Proposal 1. The target node provides a per DRB response in the Handover Request Acknowledge message to the per DRB request for DAPS HO in Handover Request message.

Proposal 2. If the target node does not accept the DAPS HO for a DRB, it does not explicitly indicate support of legacy HO fallback in the Handover Request Acknowledge message. The only valid per DRB response indication in the Handover Request Acknowledge is thus:

- DAPS handover accepted.

In the annex below, we highlight the changes required in the BL CR to TS 38.423.

# Annex: TP for BL CR for TS 38.423

------------------------------ Change #1, Section 9.2.1.16 --------------------------------------------------------------------------

#### 9.2.1.16	Data Forwarding Info from target NG-RAN node

This IE contains TNL information for the establishment of data forwarding tunnels towards the target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flows Accepted For Data Forwarding List |  | 1 |  |  |
| >QoS Flows Accepted For Data Forwarding Item |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Identifier | M |  | 9.2.3.10 |  |
| PDU Session level DL data forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | To forward NG-U DL SDAP SDUs to the target node. |
| PDU Session level UL data forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | To forward NG-U UL SDAP SDU to the target node. |
| Data Forwarding Response DRB List |  | 0..1 |  |  |
| >Data Forwarding Response DRB Item |  | 1..<maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 |  |
| >>UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 |  |
| >>DAPS Response Information | O |  | 9.2.1.EE |  |

------------------------------ End of change #1, Section 9.2.1.16 ------------------------------------------------------------------

------------------------------ Change #2, Section 9.2.1.EE -------------------------------------------------------------------------

#### 9.2.1.EE	DAPS Response Information

#### The DAPS Response Indicator IE indicates that the response to a requested DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Response Indicator | M |  | ENUMERATED (DAPS HO accepted, …) | Indicates if the DAPS Handover is accepted |

------------------------------ End of change #2, Section 9.2.1.EE ----------------------------------------------------------------

# References

[1] Chairman’s notes, RAN3 #107-e.

[2] Chairman’s notes, RAN3 #107-bis-e.