3GPP TSG-RAN WG2 Meeting #108e                                 R3-203344

Online, June 1 – June 12, 2020

Agenda item:    13.2.1.2

Source: 		Qualcomm Incorporated

Title: 	(TP for NR_IAB BL CR TS 38300) IAB stage-2 clean up

Document for:	Other

# Introduction

- This paper includes a TP for BL CR to TS 38300. This TP aims to align the BL CR with RAN2’s present TS 38.300 v16.0.0 and to capture recent RAN3 agreements.

# Discussion

- Proposal: Include the TP below into BL CR to TS 38300.

# 3	Conclusion

This paper made to following proposal:

- Proposal: Include the TP below into BL CR to TS 38300.

# 4 TP for NR_IAB BL CR TS 38300

-------------------------------------------Change 1-------------------------------------------

### 6.11.2	Traffic Mapping from Upper Layers to Layer-2

In upstream direction, the IAB-donor-CU configures the IAB-node with mappings between upstream F1- and non-F1-traffic originated at the IAB-node, and the appropriate BAP routing ID, next-hop BAP address and Backhaul RLC channel. A specific mapping is configured:

-  for each F1-U GTP-U tunnel,

-  for non-UE associated F1AP messages,

-  for UE-associated F1AP messages,

-  for non-F1 traffic.

Multiple mappings can contain the same Backhaul RLC channel and/or next-hop BAP address and/or BAP routing ID.

In case the IAB-MT is NR-dual-connected (SA-mode only), the mapping may include a separate BH RLC channel for each of the egress links. The egress link is selected based on routing as described in Section 6.11.3.

In case the IAB-node is configured with multiple IP addresses that are anchored at different IAB-donor-DUs (SA-mode only), a separate mapping can be configured for F1AP messages with respect to each of these IAB-donor-DUs. The appropriate mapping is selected based on the UL packet’s designated path on IP layer.

-------------------------------------------End of changes ------------------------------------------