3GPP TSG-RAN WG#108-e	R3-203355

# 1 - 12 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.443 | CR | 0129 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the configuration of subframe #0 and #5 for MCH in MBMS dedicated cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast-Core |  |  |  |  |  | Date: |  |  | 2020-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In dedicated MBMS cell, it should be possible to configure all subframes including subframe #0 and #5 as MBSFN subframe(s). However, (P)MCH cannot be scheduled in subframe #0 and #5. In particular with the introduction of 3ms MBMS slot length (i.e., 0.37kHz subcarrier spacing) in Rel-16, it is necessary that subframe #0 and #5 can be used for (P)MCH to improve network resource utilization as shown in figure below. Therefore, it is important that MBMS area configuration can schedule the (P)MCH in subframe#0 and #5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new extension is added to the MBSFN Subframe Configuration IE, which enables subframes #0 and #5 to be scheduled in an MBSFN area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network cannot use subframe #0 and #5 to schedule (P)MCH, thereby limiting the resource utilization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.17, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CR#4259 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.1.17	MBSFN Subframe Configuration

This IE indicates the MBSFN Subframe Configuration, as defined in TS 36.331 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Radio Frame Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32) | Semantics along the IE definition in TS 36.331 [11]. |
| Radio Frame Allocation Offset | M |  | INTEGER (0..7) | Semantics along the IE definition in TS 36.331 [11]. |
| CHOICE Subframe Allocation | M |  |  |  |
| >One Frame |  |  |  |  |
| >>One Frame Item | M |  | BIT STRING (SIZE(6)) | Semantics along the IE definition in TS 36.331 [11]. |
| >Four Frames |  |  |  |  |
| >>Four Frame Item | M |  | BIT STRING (SIZE(24)) | Semantics along the IE definition in TS 36.331 [11]. |
| CHOICE Subframe Allocation Extended | O |  |  |  |
| >One Frame Extension |  |  |  |  |
| >>One Frame Extension Item | M |  | BIT STRING (SIZE(2)) | Semantics along the definition of the MBSFN-SubframeConfig-v1430 IE as specified in TS 36.331 [11]. |
| >Four Frames Extension |  |  |  |  |
| >>Four Frame Extension Item | M |  | BIT STRING (SIZE(8)) | Semantics along the definition of the MBSFN-SubframeConfig-v1430 IE as specified in TS 36.331 [11]. |
| CHOICE Subframe Allocation Further Extension | O |  |  |  |
| >One Frame Further Extension |  |  |  |  |
| >>One Frame Further Extension Item | M |  | BIT STRING (SIZE(2)) | Semantics along the definition of the MBSFN-SubframeConfig-v16xy IE as specified in TS 36.331 [11]. |
| >Four Frames Further Extension |  |  |  |  |
| >>Four Frame Further Extension Item | M |  | BIT STRING (SIZE(8)) | Semantics along the definition of the MBSFN-SubframeConfig-v16xy IE as specified in TS 36.331 [11]. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

**  skip unchanged asn  **

MBSFN-SynchronisationArea-ID ::= INTEGER (0..65535)

MBSFN-Subframe-Configuration ::= SEQUENCE {

radioframeAllocationPeriod		ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset		INTEGER (0..7),

subframeAllocation				CHOICE {

oneFrame						BIT STRING (SIZE (6)  ),

fourFrames						BIT STRING (SIZE (24) ) },

iE-Extensions					ProtocolExtensionContainer { { MBSFN-Subframe-Configuration-ExtIEs} } OPTIONAL,

...

}

MBSFN-Subframe-Configuration-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

{ID id-SubframeAllocationExtended		CRITICALITY reject	EXTENSION SubframeAllocationExtended	PRESENCE optional} |

{ID id-SubframeAllocationFurtherExtension	CRITICALITY reject	EXTENSION SubframeAllocationFurtherExtension		PRESENCE optional},

...

}

MCCH-Update-Time ::= INTEGER (0..255)

**  skip unchanged asn  **

#### -- S

SC-PTM-Information ::= SEQUENCE {

mbmsCellList					MBMS-Cell-List,

mbms-E-RAB-QoS-Parameters		MBMS-E-RAB-QoS-Parameters,

iE-Extensions					ProtocolExtensionContainer { {SC-PTM-Information-ExtIEs} } OPTIONAL,

...

}

SC-PTM-Information-ExtIEs M2AP-PROTOCOL-EXTENSION ::= {

...

}

SFN ::= INTEGER (0..1023)

Subcarrier-SpacingMBMS ::= ENUMERATED {khz-7dot5, khz-1dot25, ...}

SubframeAllocationExtended	::=	CHOICE {

oneFrameExtension				BIT STRING (SIZE(2)),

fourFrameExtension				BIT STRING (SIZE(8)),

choice-extension				ProtocolIE-Single-Container { { SubframeAllocationExtended-ExtIEs} },

...

}

SubframeAllocationExtended-ExtIEs M2AP-PROTOCOL-IES ::= {	...

}

SubframeAllocationFurtherExtension	::=	CHOICE {

oneFrameFurtherExtension				BIT STRING (SIZE(2)),

fourFrameFurtherExtension				BIT STRING (SIZE(8)),

choice-extension				ProtocolIE-Single-Container { { SubframeAllocationFurtherExtension-ExtIEs} },

...

}

SubframeAllocationFurtherExtension-ExtIEs M2AP-PROTOCOL-IES ::= {	...

}

#### -- T

NEXT CHANGE

### 9.3.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

**  skip unchanged asn  **

id-MCH-Scheduling-PeriodExtended2											ProtocolIE-ID ::= 48

id-Subcarrier-SpacingMBMS													ProtocolIE-ID ::= 49

id-SubframeAllocationExtended												ProtocolIE-ID ::= 50

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea-Item 								ProtocolIE-ID ::= 51

id-MCCHrelatedBCCH-ExtConfigPerMBSFNArea									ProtocolIE-ID ::= 52

id-SubframeAllocationFurtherExtension										ProtocolIE-ID ::= XX