3GPP TSG-RAN3 #108-e	R3-203366

1st June – 12th June 2020

Online

Agenda Item:	08.1

Source:	Ericsson

Title:	Discussion on PWS Concurrent Warning Message Indication usage

Document for:	Discussion, Decision

# Introduction

During RAN3-105bis and RAN3-106 a discussion on the usage of the PWS Concurrent Warning Message Indication (CWMI) has been carried out, producing the LS in [1]. In [1] the following question was asked to CT1:

- Question: Is there any identified use case when it is beneficial for the CBC to not set the Concurrent Warning Message indicator for a CMAS type of system?

Together with the LS in [1] the Summary of offline discussions in [2] was captured. In [2] the following was captured, which agreed the consensus within companies involved in the discussion at RAN3-106

With the above discussion, we propose the following way-forwards:

- Send LS to CT1 for clarification: is there any identified use case when it is beneficial for the CBC to not set the Concurrent Warning Message indicator for a CMAS type of system?

- Depending on the response, RAN3 has following actions:

  - Regardless of the answer, RAN3 will not make any modification to Rel-15 NGAP/S1AP specifications w.r.t to concurrent broadcasting for CMAS

  - If CT1’s response is “yes”, RAN3 reuses the Notification Information IE to support concurrent broadcasting for CMAS, and some clarification text may be added to TS38.473

  - If CT1’s response is “no”, RAN3 considers to introduce Concurrent Warning Message Indicator IE over F1 interface as conditional mandatory for sibType8 (i.e. CMAS). Whether the revision to CT1 and RAN3 specs is needed in Rel-16 or not needs further discussion

In this paper the reply LS from CT1 in [3] is taken as an input to derive changes to the specifications in line with the consensus reached at RAN3-106.

# Discussion

The LS in [3] spells out that

CT1 answer: There is no use case when the Concurrent Warning Message indicator is not set for a CMAS type of system.

Additionally, CT1 agreed a note for clarification on CWMI inclusion to 3GPP TS 23.041 as attached to this LS. The note is added for the open release only as it is not considered a FASMO CR, but the information is applicable also for older releases.

The note referred to by the LS in [1] states the following:

NOTE:	The Concurrent Warning Message Indicator IE is always present for non-ETWS (eg. CMAS) messages. The Concurrent Warning Message Indicator is not applicable for ETWS.

With the above inputs from CT1, RAN3 can conclude that the additions made in Rel15 for PWS are sufficient to enable the system to work properly. However, there is still ambiguity with regards to when a gNB-DU shall broadcast a message concurrently.

The reason for such ambiguity is the same that sparked the discussion in RAN3-105bis. Namely, the Concurrent Warning Message Indicator is received by the RAN over the NG interface, and it is included in the NG: WRITE-REPLACE WARNING REQUEST.

Upon receiving this message, and knowing that a concurrent message can only be possible in CMAS broadcast, the gNB-CU signals to the gNB-DU an F1: WRITE-REPLACE WARNING REQUEST which includes the Notification Information IE with Message Identifier IE and/or Serial Number IE identifying a new message not currently broadcast and the gNB-CU includes in the Notification Information IE the SIB Type IE is set to “8”.

As per TS38.331 “SIB8 contains a CMAS notification.”. Hence the gNB-DU deduces that the new message to be broadcast needs to be sent concurrently with other ongoing broadcasts.

The procedure description above needs to be clarified in order to remove the ambiguity that sparked this discussion. For this reason, the summary of offline discussinos in [2] mentioned that

If CT1’s response is “yes”, RAN3 reuses the Notification Information IE to support concurrent broadcasting for CMAS, and some clarification text may be added to TS38.473

Accordingly, a CR including clarifications for the PWS procedures over F1 is proposed in [4].

# Conclusion

The paper is based on the reply LS from CT1 in [3]. The paper resumes the discussion on PWS concurrent warning message indicator usage and it concludes that, in order to clarify the RAN3 specifications in a way that they reflect the CT1 response, the CR in [4] should be agreed.

Conclusion: In order to reflect in the RAN3 specifications the LS from CT1 in C1-202116, it is proposed to agree to the CR in R3-203367

# References

[1] R3-197749, LS on Concurrent Broadcasting for CMAS, RAN3-106

[2] R3-197547, Summary of offline discussion on PWS concurrent broadcasting, Samsung

[3] C1-202116, LS on Concurrent Broadcasting for CMAS, CT1-123-e

[4 ] R3-203367, “Clarifications on Concurrent Warning Message broadcast”, Ericsson