3GPP TSG-RAN WG3 Meeting #108-e	R3-203370

1st to 12th June

Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0581 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of Inactive UE Context stored at gNB-DU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has agreed that “The sPCellConfigCommon for the PSCell is saved as part of the UE AS Inactive AS context.”. This was communicated to RAN3 in R2-2004242. TS38.473 needst to be updated in order to take such agreement into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Amend the semantics decriptino of the Lower Layer Presence Status Change IE to reflect the RAN2 agreements. Impact Analysis: This CR has functional impact on the specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is a mismatch between the context stored at the UE when in RRC_INACTIVE and the context stored at the gNB-DU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.94 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.94	Lower Layer Presence Status Change

This IE indicates lower layer resources’ presence status shall be changed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Lower Layer Presence Status Change | M |  | ENUMERATED (suspend lower layers, resume lower layers ...) | “suspend lower layers” will store CellGroupConfig; from the parameters received within ReconfigurationWithSync, only the sPCellConfigCommon is stored.. “resume lower layers” shall restore SCG and it is set only after "suspend lower layers" has been indicated |

END CHANGES