3GPP TSG-RAN3 #108-e	R3-203383

1st June – 12th June 2020

Online

Agenda Item:	09.3.8.1

Source:	Ericsson

Title:	Measurement gap deactivation over F1AP.

Document for:	Discussion, Decision

# Introduction

In the previous meeting the issue of Measurement Gap deactivation was brought up in [1] and [2]. In this contribution we will further elaborate on this issue and provide our thoughts on the matter as well as our proposals.

# Discussion

- The use case we would like to analyse is the following. The gNB-DU has some specific measurement gaps configured. At some point though gNB-CU realizes some of those measurement gaps are no longer needed and wants them to be deactivated. The question we are trying to answer is how the gNB-CU informs the gNB-DU about its decision.

- We start by looking in TS 38.473 at the relevant information element namely CU to DU RRC Information IE where we find the following:

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | CG-ConfigInfo, as defined in TS 38.331 [8]. | - |  |
| UE-CapabilityRAT-ContainerList | O |  | OCTET STRING | This IE is used in the NG-RAN and it consists of the UE-CapabilityRAT-ContainerList, as defined in TS 38.331 [8]. | - |  |
| MeasConfig | O |  | OCTET STRING | MeasConfig, as defined in TS 38.331 [8] (without MeasGapConfig). For EN-DC/NGEN-DC operation, includes the list of FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps. For NG-RAN,NE-DC and MN for NR-NR DC, includes the list of FR1 and/or FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps and the gap type (per-UE or per-FR). | - |  |
| Handover Preparation Information | O |  | OCTET STRING | HandoverPreparationInformation, as defined in TS 38.331 [8]. | YES | ignore |
| CellGroupConfig | O |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. | YES | ignore |
| Measurement Timing Configuration | O |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. In EN-DC/NGEN-DC, it is included when the gaps for FR2 are requested to be configured by the MeNB. For MN in NR-NR DC,it is included when the gaps for FR2 and/or FR1 are requested by the SgNB | YES | ignore |
| UEAssistanceInformation | O |  | OCTET STRING | UEAssistanceInformation, as defined in TS 38.331 [8]. | YES | ignore |
| CG-Config | O |  | OCTET STRING | CG-Config, as defined in TS 38.331 [8]. | YES | ignore |

- The IE of interest is MeasConfig. Searching further in TS 38.331 we see the following

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

What we can conclude is that MeasConfig IE consists of two types of information, the MeasObjectToAddModList IE, which specifies the list of frequencies for which the gNB-CU requests the gNB-DU to generate gaps and measObjectToRemoveList IE, which specifies a list of Measurement Object IDs for which measurement gaps shall be deactivated.

So, to summarize when gNB-CU wants specific gaps to be configured, it informs gNB-DU via MeasConfig IE and more specifically via MeasObjectToAddModList IE.

If gNB-CU wants to deactivate a number of measurement gaps, TS 38.473 is not providing clear guidelines on how to carry out such action. We believe that the best way is that the gNB-CU, via MeasConfig IE, informs the gNB-DU about all the frequencies that need to be configured with a measurement gap and correspondingly the frequencies not included in MeasConfig IE are to be removed. So, we propose the following, as also presented in [1].

• gNB-CU indicates to gNB-DU that measurements gaps are to be activated or deactivated via including MeasConfig IE in CU to DU RRC Information IE

• Measurement gaps for frequencies included in MeasObjectToAddModList IE are to be activated

• Measurement gaps for frequencies previously configured at gNB-DU, if not included in MeasObjectToAddModList IE, are to be deactivated

• If gNB-CU includes MeasConfig IE, but it does not contain MeasObjectToAddModList IE, this is understood by the gNB-DU as a request to deactivate all the existing configured measurement gaps

• If the gNB-DU receives CU to DU RRC Information IE without the MeasConfig IE, it does not apply any change to the existing configured measurement gaps

We understand that this results in a change of RRC behavior in the internode message compared to the use of the MeasGap IE in RRC towards the UE. However, in the case of RRC towards the UE, delta configurations are used, and the main objective is that of minimizing over the air signaling. For this reason, the UE needs to store the full measurement configuration context, namely for each measurement gap it needs to store e.g. the frequency and MeasurementObject ID for the measurement gap. In this way we save resources during RRC signaling because e.g. for removal of a measurement gap, the UE can be purely signaled to remove a Measurement Object ID. However, in the gNB-CU to gNB-DU case the main priority is not saving some bits over the F1 interface, but to allow for an efficient and cost-effective design. If the gNB-DU had to follow the same procedure as the UE, the gNB-DU shall also store the MeasObjectIDs for each measurement gap and the associated mapping to a given frequency in order to be able to identify the gaps that the gNB-CU wants to remove.

The advantage with the solution proposed above is that the gNB-DU does not need to store a mapping between measurement gap, Measurement Object ID and any other parameter associated to the gap, e.g. frequency. Therefore, the solution proposed above allows the gNB-DU to be stateless with respect to measurement gap configuration. With the proposed solution gNB-DU relies on the gNB-CU to provide the information about the measurement gaps that are needed or not.

In conclusion we believe the proposed solution is the most advantageous way forward.

- In light of the above the following is proposed:

Proposal 1: the gNB-CU via MeasConfig IE informs the gNB-DU about all the measurement gaps that need to be configured and correspondingly the gaps not included in MeasConfig IE are to be removed by the gNB-DU

# Conclusion

### In this contribution the issue of Measurement Gap deactivation over F1 AP has been discussed and the following proposal was made:

Proposal 1: the gNB-CU via MeasConfig IE informs the gNB-DU about all the measurement gaps that need to be configured and correspondingly the gaps not included in MeasConfig IE are to be removed by the gNB-DU

- CRs reflecting the proposal above are available in R3-203384 and in R3-203385.

# References

[1] R3-201909, “Discussion on Measurement Gap Deactivation”, Nokia , Nokia Shanghai Bell

[2] R3-201912, “Measurement Gap Deactivation”,  Nokia , Nokia Shanghai Bell