3GPP TSG-RAN WG3 Meeting #108_e	R3-203415

# Online Meeting, 1st – 12th June 2020

Agenda Item:	17.2.4.2

Source:	Ericsson

Title: 	Remaining issues on Dynamic Control

Document for:	Discussion and Approval

# 1		Information

At the last RAN3 meeting, in the Summary of Offline [1], on Dynamic control, we listed:

Agreement:

Introduce the DL radio quality assistance information provided per RLC

Open Issue:

1.	Which node controls the UL duplication activation of RLC entity?

2.	Assistance information exchange between RLC entities for UL duplication

- UL PDCP duplication activation state

- UL Radio quality index

- Assistance information per RLC

- UL PDCP duplication activation suggestion

3.	Initial UL duplication configuration

- The number of allowed activated RLC entities from the hosting node to assisting node, fixed number or range?

- Which node determines the initial UL activation status?

4.	Introduce the DL activation suggestion

- per RLC

- per DRB

5.	How to implement the agreement Introduce the DL radio quality assistance information provided per RLC in spec

- introduce the flag for per RLC report

- Just update the text description without flag

- Whether we use one frame for one RLC report or combine all the report of RLC entities in one

# 2		Discussion

The TS 38.425 Baseline CR already include that the “assistance information” is sent per DRB or per RLC.

Note that for each RLC duplication copy, a user plane tunnel is setup. This as we discussed early, in total it is up to RLC copies.

We think the approach of updating the text ( which is already implemented) is enough to carry the DL radio quality assistance information per DRB/RLC over the user data.

Proposal 1: RAN3 to agree that the current baseline CR on TS 38.425 already implemented the DL radio quality assistance information provided per RLC

# 3		Proposal

Proposal 1: RAN3 to agree that the current baseline CR on TS 38.425 already implemented the DL radio quality assistance information provided per RLC

# 4		Reference

[1]		R3-202703	Summary of offline discussion on CB: # NRIIOT3_Email_NRIIoT_PDCPdup_ctr