3GPP TSG-RAN WG3 Meeting #108-e	R3-203421

# Online, 1st – 11th June 2020

Agenda Item:	17.4

Source:	Ericsson

Title:	EHC updates on E1AP TP

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we propose to update the EHC configuration IE in E1AP so that it is synched with RAN 2 specifications

# 2	Discussion

A number of EHC related parmeters have been changed in RAN2 [1] associated with the Ethernet Header Compression .configuration.

ethernetHeaderCompression-r16  CHOICE {

notUsed                 NULL,

ehc                     SEQUENCE {

ehc-Common              SEQUENCE {

ehc-CID-Length	          ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink            SEQUENCE {confb

drb-ContinueEHC-DL      ENUMERATED { true }                                     OPTIONAL,   -- Need N

...

}                                                                                   OPTIONAL,   -- Need M

ehc-Uplink              SEQUENCE {

drb-ContinueEHC-UL      ENUMERATED { true }                                     OPTIONAL,   -- Need N

...

}                                                                                   OPTIONAL,   -- Need M

...

},

...

}                                                                                           OPTIONAL    -- Cond DRB2

Among the changes applicable to the RAN3 specifications it can be seen that the “ehc-HeaderSize” has be to “ehc-CID-Length” while the its values set has been changed to ENUMERATED { bits7, bits15 }

These changes should be carried to the EHC BL CR for 38.463

- Update the EHC Parameters IE in 38.463 to sync them  to the latest RAN2 specification

# 3	Conclusion

Agree on the proposals below

- Update the EHC Parameters IE in 38463 to sync them to the latest RAN2 specification

# 4	References

- draft R2-2002703 Correction of NR IIoT 38331_v7

# TP for EHC BL CR for TS 38.463

#### <<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xx		EHC Parameters

This IE carries the EHC parameters for ethernet header compression.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| EHC Common | M |  |  |  |
| >EHC- CID-Length | M |  | ENUMERATED { bits7, bits15, … } | See description of ehc- CID-Length in TS 38.331 [10] |
| EHC Downlink | O |  |  |  |
| >drb-ContinueEHC-DL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-DL in TS 38.331 [10] |
| EHC Uplink | O |  |  |  |
| >drb-ContinueEHC-UL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-UL in TS 38.331 [10] |

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

### 9.4.5	Information Element Definitions

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

EHC-Common-Parameters ::= SEQUENCE {

ehc-CID-Length						ENUMERATED { bits7, bits15, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Common-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Common-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Downlink-Parameters ::= SEQUENCE {

drb-ContinueEHC-DL					ENUMERATED { true, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Downlink-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Downlink-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Uplink-Parameters ::= SEQUENCE {

drb-ContinueEHC-DL					ENUMERATED { true, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Uplink-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Uplink-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Parameters ::= SEQUENCE {

ehc-Common							EHC-Common-Parameters,

ehc-Downlink						EHC-DL-Parameters					OPTIONAL,

ehc-Uplink							EHC-UL-Parameters					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { EHC-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>