3GPP TSG-RAN WG3 #108-e 																R3-203497

01-12 June 2020 E-Meeting

Title: 	(TP for SON BL CR for TS 38.473): UE RACH Report for RACH Optimization

Source: 	Huawei

Agenda item:	10.2.3.2

Document Type:	other

1. Introduction

In the latest RAN2 meetings, RAN2 discussed the RACH report and achieved many conclusions. In this contribution, we mainly focus on the leftovers.

2. Discussion

In the last RAN3 meeting, RAN3 reached the following conclusions:

Define a new message “ACCESS AND MOBILITY INDICATION” over F1AP with “RACH Report” as OPTIONAL IE

“ACCESS AND MOBILITY INDICATION” message over F1AP is introduced for non-UE associated signaling, with FFS for UE-associated signaling

Define a new message “ACCESS AND MOBILITY INDICATION” over XnAP with “RACH Report” as OPTIONAL IE

Reuse RAN2 IE RACH -Report-r16 for RACH Report in RAN3 instead of identifying and defining new IEs for better reusability and forward compatibility

No stage2 details regarding filtering of RACH Report is identified yet and can be added later if needed

FFS whether to introduce a RACH indication message in F1AP from DU to CU

1864 rev in R3-202783 rev in R3-202828

- add FFS to ACCESS AND MOBILITY INDICATION message name

- MESSAGE ID is at beginning of message

- check against latest comments on drafts

rev in R3-202866 Agreed unseen

1865 rev in R3-202784 rev in R3-202829

- AP IDs is at beginning of message; add FFS in tabular

rev in R3-202867 Agreed unseen

Whether to support RACH indication from DU to CU to collect UE RACH report over Uu?

- Pros: i) RACH indication could be useful assistance information for CU to trigger RACH Report from UE in case of DU-only detected RACH accesses before the maximum storage of 8 RACH Reports in absence of RACH availability indication from UE, ii) Timely collection of RACH Report without need of obtaining through Xn or filtering the RACH Reports

- Cons: i) Not reliable as UE has complete knowledge of RACH accesses and DU cannot signal RACH indication in case of CU-only detected RACH attempts to trigger RACH Report, ii) Most RACH attempts are due to mobility and DU-only detected RACH attempts are rare

To be continued...

There are two approaches to get RACH reports by the CU: one is directly from the UE in the connected state, and the other is from the neighbor gNB. When getting the RACH reports via any approach, the CU will forward the RACH reports to the DU. One question is whether RACH report forward procedure in F1 interface is UE-associated signaling or non-UE-associated signaling. For the former approach, it can be UE-associated, but for the latter approach, it can only be non-UE-associated since there are no corresponding gNB-CU UE F1AP ID/ gNB-DU UE F1AP ID. We does not see the necessity to include the F1AP IDs in the RACH report forward procedure. If something is needed, the UE can report it in the RACH report via the Uu interface. Therefore, we suggest not to include the F1AP IDs in the RACH report forward procedure.

Proposal 1: The F1AP IDs in the RACH report forward procedure (i.e., ACCESS AND MOBILITY INICATION message) from CU to DU is not needed.

In some scenarios, the CU knows the RACH occurrence, but in the following scenarios, the CU does not know:

- DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised";

- UL data arrival during RRC_CONNECTED when there are no PUCCH resources for SR available;

- SR failure;

- To establish time alignment for a secondary TAG;

- Beam failure recovery.

One question is whether it is necessary that the DU notifies the CU the RACH occurrence to trigger the CU to retrieve the RACH report in the above scenarios. RAN2 has agreed that the UE can store records for multiple RACH procedures and RAN3 has agreed to support the RACH report transfer over Xn interface. Therefore, for the above scenarios (the CU does not know the RACH occurrence), the UE can store the records and report them to the network together with next RACH procedure where CU knows the RACH occurrence. If needed, the RACH reports are sent to the corresponding gNB via the Xn interface.

Therefore, we accept not to have the RACH event notification signalling on F1.

Proposal 2: The RACH event notification from DU to CU is not supported in rel-16.

3. Conclusion

- In this contribution, the RACH report is discussed and the following proposals are proposed:

Proposal 1: The F1AP IDs in the RACH report forward procedure (i.e., ACCESS AND MOBILITY INICATION message) from CU to DU is not needed.

Proposal 2: The RACH event notification from DU to CU is not supported in rel-16.

- The TPs to TS 38.473 BL CR are provided in the Annex.

- The TPs to TS 38.423 BLCR are provided in [1].

4. Reference

[1] R3-203496	(TP for SON BL CR for TS 38.423): UE RACH report for RACH optimization.

Annex –TP for SON BL CR for TS 38.473

Start of the first change

8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |

Start of the next change

8.2.z1	Access and Mobility Indication

8.2.z1.1	General

This procedure is initiated by the gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.2.z1.2	Successful Operation

Figure 8.2.z1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from the gNB-CU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

8.2.z1.3	Abnormal Conditions

Not applicable.

Start of the next change

9.2.1.X5	ACCESS AND MOBILITY INICATION

This message is sent by the gNB-CU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | M |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | The gNB-DU UE F1AP ID 9.3.1.5 |  |  |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | EACH | ignore |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [18]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |

End of changes