3GPP TSG-RAN WG3 #108-e 	R3-203499

01 - 11 June 2020 E-Meeting

Title: 	(TP for MDT BL CR for TS 38.413): Clean up FFSes in MDT BLCR

Source: 	Huawei

Agenda item:	10.3.1

Document Type:	discussion

1. Introduction

The MDT measurement was discussed during the last meetings. In this document we will continue to discuss the remaining FFSs for the MDT measurements in the BL CRs on top of the progress of last meeting.

Summary of offline discussion R3-202469 rev in R3-202638 noted

2379 rev in R3-202748 noted

1866 rev in R3-202785 noted

(TP for [NR_SON_MDT] BL CR for TS 38.463): Addition of MDT (ZTE) R3-202791 noted

2262 rev in R3-202830 noted

1783 rev in R3-202826 noted

To be continued on this basis...

2. Discussion

According to the latest TS 37.320, M8 is the RSSI measurement by UE for WLAN/Bluetooth measurement. M9 is the RTTI measurement by UE for WLAN measurement.

⁻ M8: RSSI measurement by UE (for WLAN/Bluetooth measurement) see TS 38.331 [15]. ⁻ M9: RTT Measurement by UE (for WLAN measurement) see TS 38.331 [15].

However, in the current BL CR, the M8 measurement only includes the Bluetooth measurement and the M9 measurement only includes the WLAN measurement. The BL CR needs to update to align with the stage 2 specification.

9.3.1.x8 	M8 Configuration

This IE defines the parameters for Bluetooth measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | 9.3.1.x2 |  |

9.3.1.x9	M9 Configuration

This IE defines the parameters for WLAN measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | 9.3.1.x3 |  |

In LTE, we usually configure WLAN measurement and or Bluetooth measurement for signalling based immediate MDT and logged MDT.  Therefore, it is propose to follow the same principle in NR.

Proposal 1: To align the M8 and M9 configurations for signalling based immediate MDT in the BLCR with the stage 2 specification.

In the last meeting RAN2#108, RAN2 has agreed to introduce the logged MDT measurement frequencies and cell IDs for neighbour cell measurement in the area configuration. UE only need to log and report measurement results for the configured frequencies if the results available. Also in the last meeting, RAN3 has agreed to add the area scope configuration for neighbour cells. The FFS is the max value of the number of frequencies and cell Ids. RAN2 are still discuss the max number. In our understanding, the max value of the number of frequencies can be same to the max value of the number of inter-frequencies in SIB4 (cell reselection for inter-frequencies), i.e. 8.

-- ASN1START -- TAG-SIB4-START SIB4 ::= SEQUENCE { interFreqCarrierFreqList InterFreqCarrierFreqList, lateNonCriticalExtension OCTET STRING OPTIONAL, ... } InterFreqCarrierFreqList ::= SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo ………. maxFreq INTEGER ::= 8 -- Max number of frequencies.

For the max value of the number of cells, we think we can it can be same to the existing maxnoofCellIDforMDT (i.e. 32) within Area Scope of MDT.

Proposal 2: The max value of the number of frequencies and the max value of the number of cells in the area scope of neighbour cells is 8 and 32 respectively.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: To align the M8 and M9 configurations for signalling based immediate MDT in the BLCR with the stage 2 specification.

Proposal 2: The max value of the number of frequencies and the max value of the number of cells in the area scope of neighbour cells is 8 and 32 respectively.

The TP for MDT BL CR for TS 38.413 is provided in the ANNEX.

The mirror changes to BL CRs for TS 38.423 are provided in [1].

4. Reference

[1] R3-203500	(TP for MDT BL CR for TS 38.423): Clean up FFSes in MDT BLCR

Annex – TP for MDT BL CR for TS 38.413

Start Change

Next Change

#### 9.3.1.xx4	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…) |  | - | - |
| CHOICE Area Scope of MDT | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  |  |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [x]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration. Value “1” indicates “activate” and value “0” indicates “do not activate”. |  |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.x1 |  |  |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.x4 |  |  |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.x5 |  |  |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.x6 |  |  |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.x7 |  |  |  |
| >> Bluetooth Measurement Configuration | O |  | 9.3.1.x11 |  |  |  |
| >> WLAN Measurement Configuration | O |  | 9.3.1.x12 |  |  |  |
| >>MDT Location Information | O |  | 9.3.1.x10 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x13 |  |  |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (320ms,640ms,1280ms, 2560ms, 5120ms, 10240ms, 20480s, 30720ms, 40960ms, 61440ms, infinity,…) | This IE is defined in TS 38.331 [18]. |  |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120) | This IE is defined in TS 38.331 [18]. Unit: [minute]. |  |  |
| >>CHOICE Report Type | M |  |  |  |  |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>> Logged Event Trigger Config | M |  | 9.3.1.x14 |  |  |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.x11 |  |  |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.x12 |  |  |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.x13 |  |  |  |
| Area Scope of Neighbour | O |  | 9.3.1.xxx2 |  |  |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.3.1.xx3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

Next Change

#### 9.3.1.xxx2	Area Scope of Neighbour Cells

This IE defines the area scope of neighbour cells for logged MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area Scope of Neighbour Cells | M | 1 .. <maxnoofFreqforMDT> |  |  |
| >NR Frequency Info | M |  | 9.3.1.xxx1 |  |
| >>Neighbour cell List |  | 0..1 |  |  |
| >>Neighbour cell List Item |  | 1 .. <maxnoofNeighPCIforMDT> |  |  |
| >>>NR PCI |  |  | INTEGER (0..1007, …) | NR Physical Cell ID |

| Range bound | Explanation |
| --- | --- |
| maxnoofFreqforMDT | Maximum no. of Frequency Information subject for MDT scope. Value is 8. |
| maxnoofNeighPCIforMDT | Maximum no. of Neighbour cells subject for MDT scope. Value is 32. |

Next Change

### 9.4.5	Information Element Definitions

------------------------------skip the unchanged text-----------------------

#### -- I

IMSVoiceSupportIndicator ::= ENUMERATED {

supported,

not-supported,

...

}

IndexToRFSP ::= INTEGER (1..256, ...)

InfoOnRecommendedCellsAndRANNodesForPaging ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

recommendRANNodesForPaging		RecommendedRANNodesForPaging,

iE-Extensions		ProtocolExtensionContainer { {InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

IntendedNumberOfPagingAttempts ::= INTEGER (1..16, ...)

InterfacesToTrace ::= BIT STRING (SIZE(8))

ImmediateMDTNr ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1Configuration				M1Configuration		OPTIONAL,

m4Configuration				M4Configuration		OPTIONAL,

m5Configuration				M5Configuration		OPTIONAL,

m6Configuration				M6Configuration		OPTIONAL,

m7Configuration				M7Configuration		OPTIONAL,

bluetoothMeasurementConfiguration				BluetoothMeasurementConfiguration						OPTIONAL,

wLANMeasurementConfiguration				WLANMeasurementConfiguration						OPTIONAL,

mDT-Location-Info 			MDT-Location-Info 	OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDTNr-ExtIEs} } OPTIONAL,

...

}

ImmediateMDTNr-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Next Change

#### -- M

------------------------------skip the unchanged text-----------------------

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

MDT-Location-Info ::= BIT STRING (SIZE (8))

Next Change

### 9.4.7	Constant Definitions

------------------------------skip the unchanged text-----------------------

maxnoofTAIforWarning				INTEGER ::= 65535

maxnoofTAIinAoI						INTEGER ::= 16

maxnoofTimePeriods					INTEGER ::= 2

maxnoofTNLAssociations				INTEGER ::= 32

maxnoofXnExtTLAs					INTEGER ::= 16

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofMDTPLMNs						INTEGER ::= 16

maxnoofCellIDforMDT					INTEGER ::= 32

maxnoofTAforMDT						INTEGER ::= 8

maxnoofBluetoothName				INTEGER ::= 4

maxnoofWLANName						INTEGER ::= 4

maxnoofSensorName					INTEGER ::= 3

maxnoofFreqforMDT					INTEGER ::= 8

maxnoofNeighPCIforMDT				INTEGER ::= 32

maxnoofNRCellBands					INTEGER ::= 32

maxNRARFCN							INTEGER ::= 3279165

End Change