3GPP TSG-RAN WG3 Meeting #108-e	R3-203546

# E-meeting, 1 – 11 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1767 | rev | 2 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of connected en-gNBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ASN.1 includes en-gNB ID instead of Global en-gNB ID in Connected en-gNB List in the S1 SETUP REQUEST message. Similar issues in the ENB CONFIGURATION UPDATE message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of S1 SETUP REQUEST and ENB CONFIGURATION UPDATE: - Aligning tabular on ASN.1. - Added clarification on how the MME derives the Global en-gNB ID based on signalled information. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because the changes are backwards compatible. This CR has an impact under functional point of view. The impact can be considered isolated because the change affects only EN-DC TNL address discovery function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MME can't route messages for TNL discovery for EN-DC towards the correct en-gNB because the Global en-gNB IDs of connected en-gNBs are not known. Discrepancy between tabular and ASN.1 will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.8.4, 9.1.8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.300 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 2: Solution without ASN.1 impact, submitted to RAN3#108-e. |  |  |  |  |  |  |  |  |

#### 9.1.8.4	S1 SETUP REQUEST

This message is sent by the eNB to transfer information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT-Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | 9.2.1.16 |  | YES | ignore |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGIds> | 9.2.1.62 |  |  |  |
| UE Retention Information | O |  | 9.2.1.112 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |
| Connected en-gNB List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list for the en-gNB. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported (EPS) TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 | This information is used as specified in TS 36.300 [14]. | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |
| maxnoofConnecteden-gNBs | Maximum no. of en-gNBs connected to the eNB. Value is 256. |

#### 9.1.8.7	ENB CONFIGURATION UPDATE

This message is sent by the eNB to transfer updated information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 0..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Default Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |
| Connected en-gNB To Be Added List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the added en-gNB. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Connected en-gNB To Be Removed List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the en-gNB to be removed. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |
| maxnoofConnecteden-gNBs | Maximum no. of en-gNBs connected to the eNB. Value is 256. |