3GPP TSG-RAN3 Meeting #108-e	R3-203583

E-Meeting, 1 – 11 June 2020

Title: 	(TP for NPN BL CR for TS 38.300): Paging Aspects for SNPN

Source: 	NEC

Agenda item:	16.2

Document for:	Agreement

# 1   Introduction

In RAN3#107bis-e, RAN3 discussed Paging Optimization for SNPN in the CB: # 33_Email_PRN_paging, and concluded in [1] on adding a section on paging for SNPN for 38.300, with the following Editor’s note [2]:

Editor’s note: The section may be updated or even removed if no strong motivation is found at next phase.

In this contribution, we first revisit the added text and propose to remove it based on the understanding that there is no need for Paging Optimization at the NG-RAN node in the case of SNPN deployment.

# 2   Discussion

In the summary of the CB: # 33_Email_PRN_paging (Proposal 1, [1]), RAN3 agreed that there is no need to include the serving SNPN ID in the NGAP paging message [1]:

Proposal 1:	Agree that there is no technical justification to include the Serving PLMN ID information in the NGAP PAGING message.

In this purpose, no change was agreed to include the serving SNPN ID in the NPN Paging Assistance Information IE, shown below (in NPN BL CR for 38.413) [3]:

#### 9.3.1.X1	NPN Paging Assistance Information

This IE contains NPN Paging Assistance Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Paging Assistance Information | M |  |  |  |
| >PNI-NPN Paging Assistance |  |  |  |  |
| >>PNI-NPN Paging Assistance | M |  | Allowed PNI-NPN List 9.3.3.Y4 |  |

Additionally, according to the following description on UE operation in SNPN access mode in TS 38.300 [3]:

## 16.x Stand-Alone NPN ### 16.x.1 General [text omitted] An SNPN-capable UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode, the UE only selects and registers with SNPNs. When the UE is not set to operate in SNPN access mode, the UE performs normal PLMN selection procedures. [text omitted] ### 16.x.2 Mobility The same principles as described in 9.2 apply to SNPN except for what is described below. UEs operating in SNPN access mode only (re)select cells within the selected/registered SNPN and a cell can only be considered as suitable if the PLMN and NID broadcast by the cell matches the selected/registered SNPN.

In our understanding, the UE performs registration and cell selection/re-selection in cells that belong to the UE’s registration area (i.e. TAIs), which only supports the selected/registered SNPN. Hence, when receiving the NG paging message for the UE, operating in SNPN access mode, the NG-RAN node will only page the UE in cells that belong to the UE’s registration area that supports the SNPN ID. That is, there is no need for any paging optimization at the NG-RAN node, since the NG-RAN node will not page the UE in cells of TAIs that do not support the selected/registered SNPN ID.

Observation 1: no paging optimization is needed at the NG-RAN node in the case of SNPN.

Proposal 1: it is proposed to remove the section “16.x.x3 Paging” in NPN BL CR for TS 38.300.

# 3   References

[1] R3-202514, CB: # 33_Email_PRN_paging

[2] R3-202681, (TP for NPN BL CR for TS 38.300): Paging for NPN

[3] R3-202890, (NPN BL CR for TS 38.413) Introduction of Non-Public Networks

# 4   Annex TP for NPN BL CR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 16.x	Stand-Alone NPN

<Unchanged Text Omitted>

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>