3GPP TSG-RAN WG3 #108-e						R3-203588

E-meeting, 1 – 11 June 2020

Agenda Item:  15.4.3

Source:       NEC

Title:         Autonomous report for Discarding DL COUNT

Document for:	Discussion and decision

- 1. Introduction

Last meeting RAN3#107bis-e discussed in [# 25_Email_Mob_Enh_Data_fwd_E1] for the support of Early Data forwarding over E1 interface. Summary in [R3-202507].

We give a discussion and proposal for the “FFS whether autonomous report for Discarding DL COUNT from the source CU-UP is needed”.

- 2. Discussion

If the early data forwarding is applied, it is needed for the source to tell the target to discard the early forwarded data which have been successfully delivered to the UE in source side. For the CP/UP split architecture, furthermore for optimization, the intermediate status of the delivery status in the source gNB-CU-UP can be informed so can optimize the buffer in the target side.

One way has been suggested is to introduce a new class 2 procedure so that the source gNB-CU-CP request to have periodically reporting of the delivery status in the source gNB-CU-UP, the source gNB-CU-UP can e.g. periodically give the latest data delivery status (i.e. the COUNT value) to the source gNB-CU-CP, and then that is given to the target gNB-CU-UP [R3-202373, R3-202377].

The intermediate delivery status information may be beneficial but no need to have periodically reporting. If there is no data sending at the period of time then then the information will be useless. Therefore, if the new class 2 procedure will be introduced, it is up to the gNB-CU-UP to inform the gNB-CU-CP whenever the delivery status is available.

Since this is an optimization, it is also worth to consider and discuss if this can postpone to later release.

- 3. Conclusion and proposal

Proposal: if needed, the to be introduced new class 2 procedure to inform from source gNB-CU-UP to gNB-CU-UP the data delivery status, without need to be requested for periodically informing.

Since this is an optimization, it is also worth to consider and discuss if this can postpone to later release.

Ref:

R3-202507