3GPP TSG-RAN3 Meeting #108-e	R3-203665

E-Meeting, 1 – 11 June, 2020

Title: 	(TP for RACS BL CR for TS 36.423): Signalling UE capability identity

Source: 	Huawei

Agenda item:	18.2

Document Type:	 Other

# Annex – TP

CHANGES START

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Addition Preparation | SGNB ADDITION REQUEST | SGNB ADDITION REQUEST ACKNOWLEDGE | SGNB ADDITION REQUEST REJECT |
| MeNB initiated SgNB Modification Preparation | SGNB MODIFICATION REQUEST | SGNB MODIFICATION REQUEST ACKNOWLEDGE | SGNB MODIFICATION REQUEST REJECT |
| SgNB initiated SgNB Modification | SGNB MODIFICATION REQUIRED | SGNB MODIFICATION CONFIRM | SGNB MODIFICATION REFUSE |
| SgNB change | SGNB CHANGE REQUIRED | SGNB CHANGE CONFIRM | SGNB CHANGE REFUSE |
| MeNB initiated SgNB Release | SGNB RELEASE REQUEST | SGNB RELEASE REQUEST ACKNOWLEDGE | SGNB RELEASE REQUEST REJECT |
| SgNB initiated SgNB Release | SGNB RELEASE REQUIRED | SGNB RELEASE CONFIRM |  |
| EN-DC X2 Setup | EN-DC X2 SETUP REQUEST | EN-DC X2 SETUP RESPONSE | EN-DC X2 SETUP FAILURE |
| EN-DC Configuration Update | EN-DC CONFIGURATION UPDATE | EN-DC CONFIGURATION UPDATE ACKNOWLEDGE | EN-DC CONFIGURATION UPDATE FAILURE |
| EN-DC Cell Activation | EN-DC CELL ACTIVATION REQUEST | EN-DC CELL ACTIVATION RESPONSE | EN-DC CELL ACTIVATION FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| EN-DC X2 Removal | EN-DC X2 REMOVAL REQUEST | EN-DC X2 REMOVAL RESPONSE | EN-DC X2 REMOVAL FAILURE |
| UE Radio Capability ID Mapping Request | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |

NEXT CHANGE

### 8.7.a	UE Radio Capability ID Mapping Request

#### 8.7.a.1	General

The purpose of the UE Radio Capability ID Mapping Request procedure is to enable the en-gNB to request the MeNB to provide the UE Radio Capability information that maps to a specific UE Radio Capability ID. The procedure uses non UE-associated signalling.

#### 8.7.a.2	Successful Operation

Figure 8.7.a.2-1: UE Radio Capability ID Mapping Request procedure, Successful operation.

The en-gNB initiates the procedure by sending a UE RADIO CAPABILITY ID MAPPING REQUEST message to the MeNB with the UE Radio Capability ID IE.

Upon receipt of the UE RADIO CAPABILITY ID MAPPING REQUEST message, the MeNB shall include the UE Radio Capability information that maps to the UE Radio Capability ID in the UE RADIO CAPABILITY ID MAPPING RESPONSE message.

#### 8.7.a.3	Unsuccessful Operation

Not applicable.

#### 8.7.a.4	Abnormal Conditions

Void.

NEXT CHANGE

#### 9.1.4.b	UE RADIO CAPABILITY ID MAPPING REQUEST

This message is sent by the en-gNB to the MeNB to require the UE Radio Capability information that maps to a specific UE Radio Capability ID.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| UE Radio Capability ID | M |  | 9.2.x |  | YES | reject |

#### 9.1.4.c	UE RADIO CAPABILITY ID MAPPING RESPONSE

This message is sent by the MeNB to the en-gNB to provide the UE Radio Capability information that maps to a specific UE Radio Capability ID indicated in the UE RADIO CAPABILITY ID MAPPING REQUEST message.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| UE Radio Capability ID | M |  | 9.2.x |  | YES | reject |
| UE Radio Capability | M |  | 9.2.d |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |

NEXT CHANGE

### 9.2.d	UE Radio Capability

This IE contains UE Radio Capability information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability | M |  | OCTET STRING | Includes the RRC UERadioAccessCapabilityInformation message as defined in TS 38.331 [31]. |

NEXT CHANGE

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

X2AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM X2AP-CommonDataTypes

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

DeactivateTrace,

TraceStart,

UERadioCapabilityIDMappingRequest,

UERadioCapabilityIDMappingResponse

FROM X2AP-PDU-Contents

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

id-deactivateTrace,

id-traceStart,

id-UERadioCapabilityIDMappingRequest

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

#### -- Interface Elementary Procedure List

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURES X2AP-ELEMENTARY-PROCEDURE ::= {

X2AP-ELEMENTARY-PROCEDURES-CLASS-1			|

X2AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

X2AP-ELEMENTARY-PROCEDURES-CLASS-1 X2AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

reset													|

x2Setup													|

resourceStatusReportingInitiation						|

eNBConfigurationUpdate									|

mobilitySettingsChange									|

cellActivation											|

seNBAdditionPreparation									|

meNBinitiatedSeNBModificationPreparation				|

seNBinitiatedSeNBModification							|

seNBinitiatedSeNBRelease								|

x2Removal												|

retrieveUEContext										|

sgNBAdditionPreparation									|

meNBinitiatedSgNBModificationPreparation					|

sgNBinitiatedSgNBModification							|

meNBinitiatedSgNBRelease									|

sgNBinitiatedSgNBRelease									|

sgNBChange												|

endcX2Setup												|

endcConfigurationUpdate									|

endcCellActivation										|

endcPartialReset											|

eUTRANRCellResourceCoordination							|

endcX2Removal								|

uERadioCapabilityIDMappingRequest,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

#### -- Interface Elementary Procedures

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

uERadioCapabilityIDMappingRequest X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityIDMappingRequest

SUCCESSFUL OUTCOME		UERadioCapabilityIDMappingResponse

PROCEDURE CODE			id-UERadioCapabilityIDMappingRequest

CRITICALITY				reject

}

NEXT CHANGE

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

TNLA-Failed-To-Setup-List,

RAN-UE-NGAP-I,

UERadioCapability

FROM X2AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

X2AP-PRIVATE-IES,

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES,

X2AP-PROTOCOL-IES-PAIR

FROM X2AP-Containers

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-UEContextReferenceatSourceNGRAN,

id-UERadioCapability,

NEXT CHANGE

##### -- UE RADIO CAPABILITY ID MAPPING REQUEST

--

-- **************************************************************

UERadioCapabilityIDMappingRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UERadioCapabilityIDMappingRequestIEs} },

...

}

UERadioCapabilityIDMappingRequestIEs X2AP-PROTOCOL-IES ::= {

{ ID id-UERadioCapability-ID		CRITICALITY reject	TYPE UERadioCapability-ID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UE RADIO CAPABILITY ID MAPPING RESPONSE

--

-- **************************************************************

UERadioCapabilityIDMappingResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UERadioCapabilityIDMappingResponseIEs} },

...

}

UERadioCapabilityIDMappingResponseIEs X2AP-PROTOCOL-IES ::= {

{ ID id-UERadioCapability-ID			CRITICALITY reject	TYPE UERadioCapability-ID			PRESENCE mandatory	}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

NEXT CHANGE

### 9.3.5	Information Element definitions

UERadioCapability ::= OCTET STRING

NEXT CHANGE

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

id-UERadioCapabilityIDMappingRequest		ProcedureCode ::= d1

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

#### -- IEs

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

id-UERadioCapability													ProtocolIE-ID ::=  d2

END OF CHANGE