3GPP TSG-RAN3 Meeting #108-e	R3-203666

E-Meeting, 1 – 11 June, 2020

Title: 	(TP for RACS BL CR for TS 36.413): Signalling UE capability identity

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

# Annex – TP

CHANGES START

## 8.14	UE Radio Capability Management Procedures

### 8.14.1	UE Radio Capability Info Indication

#### 8.14.1.1	General

The purpose of the UE Radio Capability Info Indication procedure is to enable the NG-RAN node to provide to the AMF UE radio capability-related information. The procedure uses UE-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: UE radio capability info indication

The NG-RAN node controlling a UE-associated logical NG connection initiates the procedure by sending a UE RADIO CAPABILITY INFO INDICATION message to the AMF including the UE radio capability information.

The UE RADIO CAPABILITY INFO INDICATION message may also include paging specific UE radio capability information within the UE Radio Capability for Paging IE.

The UE radio capability information received by the AMF shall replace previously stored corresponding UE radio capability information in the AMF for the UE, as described in TS 23.501 [9].

If UE RADIO CAPABILITY INFO INDICATION message contains the UE Radio Capability E-UTRA IE, the AMF shall, if supported, use it according to TS 23.501 [9].

NEXT CHANGE

#### 9.2.13.1	UE RADIO CAPABILITY INFO INDICATION

This message is sent by the NG-RAN node to provide UE radio capability related information to the AMF.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| UE Radio Capability | M |  | 9.3.1.74 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| UE Radio Capability E-UTRA | O |  | 9.3.1.a |  | YES | ignore |

EXT CHANGE

#### 9.3.1.a	UE Radio Capability E-UTRA

This IE contains UE Radio Capability information formats specified in TS 36.331 [21].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability E-UTRA | M |  | OCTET STRING | Includes UERadioAccessCapabilityInformation message as defined in TS 36.331 [21]. |

NEXT CHANGE

### 9.4.4	PDU Definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-RIMInformationTransfer,

id-UERadioCapabilityE-UTRA

NEXT CHANGE

##### -- UE RADIO CAPABILITY INFO INDICATION

--

-- **************************************************************

UERadioCapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {UERadioCapabilityInfoIndicationIEs} },

...

}

UERadioCapabilityInfoIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional		}|

{ ID id-UERadioCapabilityE-UTRA			CRITICALITY ignore	TYPE UERadioCapabilityE-UTRA		PRESENCE optional		},

...

}

NEXT CHANGE

### 9.4.5	Information Element Definitions

<<<<<<<<<<<<<<<<<<<< Unmodified Text omitted >>>>>>>>>>>>>>>>>>>>

UERadioCapabilityE-UTRA ::= OCTET STRING

NEXT CHANGE

### 9.4.7	Constant Definitions

#### -- IEs

id-UERadioCapabilityE-UTRA									ProtocolIE-ID ::= b

END OF CHANGE