3GPP TSG-RAN WG3 #108-e		                                   R3-203684

1 – 11 June 2020 E-Meeting

Source:	CATT

Title:	Discussion on RAN sharing in XN interface

Agenda Item:	16.4

Document for:	Discussion and Decision

Introduction

At the RAN3 107bis-e meeting, due to collision of CB#29 and CB#36 on NPN RAN sharing, how to translate PLMN and NPN related information in XN interface has not be discussed. It is an important issue and need continue discussion.

Discussion

When serving cell NR IE was introduced in Rel-15 in XN interface, only public network is supported and PLMN Identity IE is Mandatory. Now, with the introduction of NPN network, one logical cell could belong to either PLMN or NPN. Since PLMN Identity IE is also included in NPN Broadcast Information IE, there would be duplicated PLMN related information for one logical cell. In this case, it is unclear to the peer node which IE represent the PLMN information for the logical node, PLMN Identity IE within Broadcast PLMN Identity Info List NR IE or PLMN Identity IE within NPN Broadcast Information IE?

To resolve the ambiguity, there are three solutions submitted by companies at the last meeting:

Solution 1:

Currently, there are two lists for PLMN and NPN separately broadcasted in air interface. while there is only one Broadcast PLMN Identity Info List in XN interface. To be aligned with RAN2, it is proposed to expand NPN Support Information IE to a separate list to contain all of the NPN configuration information. In this case, NPN Support Information list IE and Broadcast PLMN Identity Info List NR IE is used for public network and NPN separately.

The NPN configuration information may include cell ID for different network types due to Logical cell would be associated with a single access mode. NPN Support Information list IE is depicted as below.

| IE name | description |
| --- | --- |
| NPN Support Information list | All of the NPN configuration information list, maximum is 12 |
| >CHOICE Broadcast NPN per PLMN |  |
| >> SNPN Information |  |
| >>> Broadcast SNPN ID List | SNPN ID list |
| >>> NR Cell Identity | cell ID for SNPN list |
| >> PNI-NPN Information |  |
| >>> Broadcast NPN-NPN ID List | NPN-NPN ID List |
| >>> NR Cell Identity | cell ID for NPN-NPN list |
| >TAC |  |
| >RANAC | RAN Area Code |

Solution 2:

With one list for all of the PLMN and NPN information, PLMN information is duplicated since there is PLMN Identity IE in Broadcast SNPN ID List IE/Broadcast PNI-NPN ID Information IE as below and the same one in Broadcast PLMN Identity Info List NR IE. For compatibility with old version, it is not suitable to change the existing PLMN Identity from Mandatory to optional. it is proposed to remove the PLMN Identity IE in Broadcast SNPN ID List IE and Broadcast PNI-NPN ID Information IE.

9.2.2.x3 Broadcast SNPN ID List This IE contains a list of SNPN IDs. IE/Group Name; Presence. Broadcast SNPN ID List. >PLMN Identity; M. >NID; M. 9.2.2.x5 Broadcast PNI-NPN ID Information This IE contains a list of PNI-NPN IDs. IE/Group Name; Presence. Broadcast PNI-NPN ID Information. >PLMN Identity; M. >Broadcast CAG-Identifier List; M.

Solution 3:

Add semantics descriptions that for the NPN Broadcast Information IE in the Served Cell Information NR IE that the information contained in the Broadcast PLMNs IE shall be ignored, as proposed in [1].

| >NPN Broadcast Information | O |  | 9.2.2.x6 | If this IE is included the content of the Broadcast PLMNs IE in the Broadcast PLMN Identity Info List NR IE is ignored. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| NPN Broadcast Information | O |  | 9.2.2.x6 | If this IE is included the content of the Broadcast PLMNs IE in the Served Cell Information NR IE is ignored. | YES | reject |

For above three solutions, we have a preference on solution 1 since it is aligned with RAN2 and RAN3 can make similar changes according to RAN2’s modification. For solution 2, the structure of Broadcast PLMN Identity Info List NR may not be suitable for future change.

So, solution 1 is recommended.

Proposal 1: It is proposed to adopt solution 1 and add a separate list to contain all of the NPN configuration information in Broadcast PLMN Identity Info List NR IE in Xn interface.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: It is proposed to adopt solution 1 and add a separate list to contain all of the NPN configuration information in Broadcast PLMN Identity Info List NR IE in Xn interface.

References

[1] R3-203685 "(TP on NPN BLCR for 38.423) Correction on RAN sharing in XN interface" RAN3#108-e