3GPP TSG-RAN3 Meeting #108-e	R3-203718

E-meeting, 01 - 11 June 2020

Title: 	(TP for NR_IIOT BL CR for TS 38.473): Proposed rapporteur changes to the BL CR

Source: 	Huawei

Agenda item:	17.1

Document for:	Other

1. Introduction

This contribution intends to propose to update the Periodic IE in the BL CR for TS 38.473 [1].

As shown in section 9.3.1.XX, the Periodic IE is provided to request the report periodicity value under the periodic event. The periodic IE is associated with the Report Periodicity Value IE with integer value. But this is not needed since the Periodic IE can be set as integer value directly, without the definition of the Report Periodicity Value IE.

9.3.1.XX	Reporting Request Type

This IE indicates the type of accurate reference time information reporting to be handled by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (on demand, periodic, stop, …) |  |
| Periodic | C-ifEventTypeisPeriodic |  |  |  |
| >Report Periodicity Value | M |  | INTEGER (0..512, …) | Indicates the periodicity of accurate reference time information report, Unit in radio frame. |

Hence it is proposed to remove the Report Periodicity Value IE, and set the Periodic IE as integer directly. And there is no change to ASN.1.

Remove the Report Periodicity Value IE, and set the Periodic IE as integer type directly in Section 9.3.1.xx.

2. Reference

- R3-203077	Introduction of NR_IIOT support to TS 38.473,	Huawei

3. Annex – TP

Change Begins

9.3.1.XX	Reporting Request Type

This IE indicates the type of accurate reference time information reporting to be handled by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (on demand, periodic, stop, …) |  |
| Periodic | C-ifEventTypeisPeriodic |  | INTEGER (0..512, …) | Indicates the periodicity of accurate reference time information report, Unit in radio frame. |

| C-ifEventYpeisStop | Explanation |
| --- | --- |
| ifEventTypeisPeriodic | This IE shall be present if the Event Type IE is set to "periodic". |

End Change