# 3GPP TSG-RAN3 Meeting #108e																	R3-203750

# Online, 1st – 11th June

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.455 | CR | 0109 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting of per beam measurements and SFTD inter-RAT results in LPPa |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DOCOMO Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding per SS beam index measurement and SFTD measurements in the E-CID measurement results |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding per SS beam index measurement in the inter-RAT measurement results for NR Adding SFTD measurements with SFN offset and frame boundary values in the E-CID measurement result |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | insufficient measurements reports in E-CID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.13, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | ECGI 9.2.6 | E-UTRAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | OCTET STRING(2) | Tracking Area Code of the serving cell |
| E-UTRAN Access Point Position | O |  | 9.2.8 | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 1 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 2 | M |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Result RSRP |  | 1 .. <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRP | M |  | INTEGER(0..97, …) |  |
| >>Result RSRQ |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRQ | M |  | INTEGER(0..34, …) |  |
| >>Result SFTD |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>SFN-Offset | M |  | INTEGER (0..1023) | The observed SFN and frame timing difference that a UE measured between an E-UTRA PCell and an NR Cell, |
| >>>MeanFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Average value of the frame boundary offset observed between PCell and the measured cell. |
| >>>MinFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Minimum value of the frame boundary offset observed between PCell and the measured cell. |
| >>>MaxFrameBoundary-Offset | M |  | INTEGER (-30720..30719) | Maximum value of the frame boundary offset |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |

NEXT CHANGE

### 9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. |
| >>Result NR | M | 1..<maxNRMeas> |  |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) |  |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) |  |
| >>>SFN-Offset | O |  | INTEGER (0..1023) | The observed SFN and frame timing difference that a UE measured between an NR PCell and the measured Cell, |
| >>>MeanFrameBoundaryOffset | O |  | INTEGER (-30720..30719) | Average value of the frame boundary offset observed between NR PCell and the measured cell. |
| >>>MinFrameBoundary-Offset | O |  | INTEGER (-30720..30719) | Minimum value of the frame boundary offset observed between NR PCell and the measured cell. |
| >>>MaxFrameBoundary-Offset | O |  | INTEGER (-30720..30719) | Maximum value of the frame boundary offset |
| >>>SSB Index List | O | 1..<maxResultsPerSSBIndex> |  |  |
| >>>>SSB Index | O |  | INTEGER (0..63) | Index of SSB in results NR |
| >>>>NR SS-RSRP beam value | O |  | INTEGER (0..127) | beam level measurement result of NR SS-RSRP |
| >>>>NR SS-RSRQ beam value | O |  | INTEGER (0..127) | beam level measurement result of NR SS-RSRQ |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 32. |
| maxResultsPerSSBIndex | Maximum no. of SSB indices that can be reported with in ine messafe. Value is 64 |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxResultsPerSSBIndex,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs

FROM LPPA-Constants

NEXT CHANGE

#### -- M

Measurement-ID ::= INTEGER (1..15, ...)

MeasurementPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

ms2048,

ms5120,

ms10240,

min1,

min6,

min12,

min30,

min60,

...

}

MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF ProtocolIE-Single-Container { {MeasurementQuantities-ItemIEs} }

MeasurementQuantities-ItemIEs LPPA-PROTOCOL-IES ::= {

{ ID id-MeasurementQuantities-Item	CRITICALITY reject	TYPE MeasurementQuantities-Item		PRESENCE mandatory}

}

MeasurementQuantities-Item ::= SEQUENCE {

measurementQuantitiesValue							MeasurementQuantitiesValue,

iE-Extensions										ProtocolExtensionContainer { { MeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

MeasurementQuantitiesValue-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

...

}

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival				INTEGER (0..719),

valueTimingAdvanceType1			INTEGER (0..7690),

valueTimingAdvanceType2			INTEGER (0..7690),

resultRSRP						ResultRSRP,

resultRSRQ						ResultRSRQ,

resultSFTD						ResultSFTD,

...

}

UNCHANGED TEXT OMITTED

#### -- R

RepetitionNumberofSIB1-NB ::= ENUMERATED {

r4,

r8,

r16,

...

}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...),

bandwidth				INTEGER (1..100, ...),

...

}

ResultRSRP ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-Item

ResultRSRP-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRP		ValueRSRP,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRP-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-Item

ResultRSRQ-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRQ		ValueRSRQ,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRQ-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSFTD ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultSFTD-Item

ResultSFTD-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

SFN-Offset			INTEGER (0..1023),

FrameBoundary-Offset		INTEGER (-30720..30719),

MinFrameBoundary-Offset		INTEGER (-30720..30719),

MaxFrameBoundary-Offset		INTEGER (-30720..30719),

iE-Extensions	ProtocolExtensionContainer { { ResultSFTD-Item-ExtIEs} } OPTIONAL,

...

}

ResultSFTD-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultNR ::= SEQUENCE (SIZE (1.. maxNRmeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nRARFCN				NRARFCN,

nRPCI				NRPCI,

sS-NRRSRP			SS-NRRSRP	OPTIONAL,

sS-NRRSRQ			SS-NRRSRQ	OPTIONAL,

SFN-Offset			INTEGER (0..1023)	OPTIONAL,

FrameBoundary-Offset		INTEGER (-30720..30719)	OPTIONAL,

MinFrameBoundary-Offset		INTEGER (-30720..30719)		OPTIONAL,

MaxFrameBoundary-Offset		INTEGER (-30720..30719)		OPTIONAL,

sSB-Index-list		SSB-Index-list	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} } OPTIONAL,

...

}

ResultNR-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

#### -- S

SFNInitialisationTime ::= BIT STRING (SIZE (64))

SRSConfigurationForAllCells ::= SEQUENCE (SIZE (1.. maxServCell)) OF SRSConfigurationForOneCell

SRSConfigurationForOneCell ::= SEQUENCE {

pci							PCI,

ul-earfcn					EARFCN,

ul-bandwidth				ENUMERATED {n6, n15, n25, n50, n75, n100},

ul-cyclicPrefixLength		CPLength,

srs-BandwidthConfig			ENUMERATED {bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7},

srs-Bandwidth				ENUMERATED {bw0, bw1, bw2, bw3},

srs-AntennaPort				ENUMERATED {an1, an2, an4, ...},

srs-HoppingBandwidth		ENUMERATED {hbw0, hbw1, hbw2, hbw3},

srs-cyclicShift				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7},

srs-ConfigIndex				INTEGER (0..1023),

maxUpPts					ENUMERATED {true}			OPTIONAL,	-- Cond ifTDD

transmissionComb			INTEGER (0..1),

freqDomainPosition			INTEGER (0..23),

groupHoppingEnabled			BOOLEAN,

deltaSS						INTEGER (0..29)				OPTIONAL,

sfnInitialisationTime		SFNInitialisationTime,

...

}

Subframeallocation ::= CHOICE {

oneFrame						BIT STRING (SIZE(6)),

fourFrames						BIT STRING (SIZE(24))

}

SS-NRRSRP ::= INTEGER (0..127)

SS-NRRSRQ ::= INTEGER (0..127)

SSB-Index ::= INTEGER (0..63)

SSB-Index-list ::= SEQUENCE (SIZE (1.. maxResultsPerSSBIndex)) OF SEQUENCE {

sSB-Index						SSB-Index,

sS-NRRSRPBeamValue				SS-NRRSRP,

sS-NRRSRQBeamValue				SS-NRRSRQ,

...

}

SSID ::= OCTET STRING (SIZE(1..32))

SystemInformation ::= SEQUENCE (SIZE (1.. maxNrOfPosSImessage)) OF SEQUENCE {

broadcastPeriodicity				BroadcastPeriodicity,

posSIBs								PosSIBs,

iE-Extensions						ProtocolExtensionContainer { { SystemInformation-ExtIEs} }	OPTIONAL,

...

}

SystemInformation-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

NEXT CHANGE

### 9.3.7	Constant definitions

UNCHANGED TEXT OMITTED

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellineNB								INTEGER ::= 256

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxGERANMeas								INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxCellineNB-ext							INTEGER ::= 3840

maxMBSFN-Allocations						INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNRmeas									INTEGER ::= 32

maxResultsPerSSBIndex						INTEGER ::= 64

END OF CHANGES