3GPP TSG-RAN WG3 Meeting #108-e	R3-203818

# June 1st – 11th 2020, Online

Agenda Item:	13.2.1.2

Source:	Ericsson

Title:	(TP for NR-IAB BL CR for TS 38.473): Correction of BH RLC CH ID

Document for: 	Agreement

# Introduction

RAN2 and RAN3 have extensively discussed the IE design of Backhaul RLC Channel ID IE. In RAN2, the discussion has so far focused on two options: a CHOICE structure (between the legacy and extended LCID format) or an ENUMERATED structure. Meanwhile, the RAN3#107bis-e decision to mandate the 16-bit format has been taken without any technical discussion and based on incorrect information that RAN2 has already agreed to choose the ENUMERATED structure.

With respect to the above, this contribution analyzes the principles that should be followed when designing the BH RLC Channel ID IE. The TP for IAB BL CR for TS 38.473 is presented in the Annex.

# Discussion

The DL user plane L2 structure for IAB-donor nodes is shown in Figure 1. The BAP layer receives data from higher layers. The BAP layer delivers BAP PDUs to the RLC entity via Backhaul RLC Channels. RLC processes the RLC SDU and delivers RLC PDUs to the MAC via Backhaul Logical Channels. This latter procedure is similar to UEs i.e. RLC and MAC are connected via Logical Channels. It is important to note that one BH Logical Channel requires one BH RLC Channel.

Figure 1: DL L2-structure for user plane at IAB-donor

During the IAB SI, two distinct mechanisms to transmit data over the BH links in an IAB network were identified. One mechanism consists of carrying the data of multiple UEs over a single BH RLC Channel and BH Logical Channel, i.e. N-to-1 mapping. A second mechanism consists of carrying the data of each individual UE over its own BH RLC Channel and BH Logical Channel, 1-to-1 mapping.

The first mechanism, N-to-1 mapping, was based on the current parameter set for Logical Channels. This means that the same Logical Channel ID range as in legacy could be configured. This also implied that the range for the BH RLC Channels had to necessarily be equal to the range of the BH Logical Channels, i.e. 32.

The second mechanism, 1-to-1 mapping, required an extension of the currently defined Logical channel IDs. A 2-byte length Logical Channel ID extension was introduced during RAN2#107bis. As in the previous case, an equal number of BH RLC Channels had to be introduced.

The basic mandatory functionality that an IAB-MT and the network must support is the legacy ID range i.e. BH Logical Channel ID range from 1 to 32 and, therefore, an equal range of IDs for the BH RLC Channels.

The optional functionality that an IAB-MT and the network may support is the extended ID range i.e. BH Logical Channel ID range from 33 to 2^16 and, therefore, an equal range of IDs for the BH RLC Channels.

Observation 1: The basic mandatory functionality that an IAB-MT and the network must support is the legacy ID range, i.e. Logical Channel ID range from 1 to 32, and hence, an equal range of IDs for the BH RLC Channels.

Observation 2: The optional functionality that an IAB-MT and the network may support is the extended ID range, i.e. BH Logical Channel ID range from 33 to 2^16 and, therefore, an equal range of IDs for the BH RLC Channels.

To ensure that a network node or an IAB-node does not implement functionality not required or supported by the network, the ASN.1 structure for the BH Logical Channel ID range was defined as CHOICE (see Figure 2). A network not supporting or configuring the BH LCH ID extension would only use the first parameter “bh-LogicalChannelIdentity-r16”; while a network supporting and configuring a value defined in the extended range, it would use the second parameter “bh-LogicalChannelIdentityExt-r16”. This structure is also a reflection of IAB-MT capabilities. All IAB-MTs should be able to handle and deal with “bh-LogicalChannelIdentity-r16” IE. However, the IAB-MTs not supporting the BH Logical Channel ID extension should not receive or handle the “bh-LogicalChannelIdentityExt-r16”.

Observation 3: To ensure that a network node or an IAB-node does not implement functionality not required or supported by the network, the ASN.1 structure for the BH Logical Channel ID range was defined as CHOICE.

Given the direct relationship between BH logical channels and BH RLC channels, it becomes straightforward to apply the same reasoning as above for BH RLC Channel ID design. All IAB-MTs and network nodes must support a BH RLC Channel ID range up to 32. The 2-byte extended ID range is, therefore, optional as the BH Logical Channel ID extension.

A CHOICE structure ensures that IAB-MTs and network supporting the feature, implements handling of the IE and allows the network uses the appropriate IEs that correspond to the support of a specific feature. Further, it makes sure that a network or IAB-MT not supporting the feature does not implement any handling for it, and hence, reduces the overhead signalling.

On the other hand, the 16-bit structure agreed at RAN3#107bis-e forces both the IAB-MT and the network to support the full range of IDs, making it more complex to find the relationship between the support of a feature and the corresponding IEs to configure such feature. Furthermore, it increases unnecessarily the overhead of both user plane and control plane since it mandates the network and IAB-MTs to always use an additional 2-byte field in each MAC subPDUs.

Observation 4: The 16-bit structure agreed at RAN3#107bis-e forces both the IAB-MT and the network to support the full range of IDs, making it more complex to find the relationship between the support of a feature and the corresponding IEs to configure such feature.

Observation 5: A CHOICE structure between the legacy LCID format and extended format ensures that IAB-MTs and network supporting the feature implements handling of the IE and allows the network uses the appropriate IEs that correspond to the support of a specific feature.

The CHOICE structure has clear advantages over mandating the 16-bit format. It clearly defines an optional IE which is related to specific capability support and it allows minimizing the signalling overhead.

Proposal 1: The BH RLC Channel ID IE should be defined as a CHOICE structure that separates mandatory and optional ID ranges as follows:

BHRLCChannelID::= CHOICE {

lCID			LCID,

extendedLCID	ExtendedLCID,

...

}

Proposal 2: RAN3 to agree the TP to IAB BL CR for TS 38.473, presented in the Annex.

# Conclusions

In this paper we discuss the format of BH RLC Channel ID IE and observe the following:

Observation 1: The basic mandatory functionality that an IAB-MT and the network must support is the legacy ID range, i.e. Logical Channel ID range from 1 to 32, and hence, an equal range of IDs for the BH RLC Channels.

Observation 2: The optional functionality that an IAB-MT and the network may support is the extended ID range, i.e. BH Logical Channel ID range from 33 to 2^16 and, therefore, an equal range of IDs for the BH RLC Channels.

Observation 3: To ensure that a network node or an IAB-node does not implement functionality not required or supported by the network, the ASN.1 structure for the BH Logical Channel ID range was defined as CHOICE.

Observation 4: The 16-bit structure agreed at RAN3#107bis-e forces both the IAB-MT and the network to support the full range of BH RLC Channel IDs, making it more complex to find the relationship between the support of a feature and the corresponding IEs to configure such feature.

Observation 5: A CHOICE structure between the legacy LCID format and extended format ensures that IAB-MTs and network supporting the feature implements handling of the IE and allows the network uses the appropriate IEs that correspond to the support of a specific feature.

Based on the observations, the following is proposed:

Proposal 1: Both BH RLC Channel and BH Logical Channel IDs should be defined in ASN.1 with a CHOICE structure that separates mandatory and optional ID ranges as follows:

BHRLCChannelID::= CHOICE {

lCID			LCID,

extendedLCID	ExtendedLCID,

...

}

Proposal 2: RAN3 to agree the TP to IAB BL CR for TS 38.473, presented in the Annex.

# Annex: TP for NR-IAB BL CR for TS 38.473

-------------------------------------------Change 1-------------------------------------------

### 9.3.1.x	BH RLC Channel ID

This IE uniquely identifies a BH RLC channel for an IAB-node and IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE BH RLC CH ID Type | M |  |  |  |
| >LCID | M |  | 9.3.1.35 |  |
| >Extended BH RLC CH ID | M | INTEGER (33..65536) |  | Corresponds to the BH-LogicalChannelIdentity-r16, defined in subclause 6.3.2. of TS 38.331 [8]. |

-------------------------------------------Change 2-------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- B

BAPAddress ::= BIT STRING (SIZE(10))

BAPCtrlPDUChannel ::= ENUMERATED {true, ...}

BAPPathID ::= BIT STRING (SIZE(10))

BAPRoutingID ::= BIT STRING (SIZE(20))

BitRate ::= INTEGER (0..4000000000000,...)

BearerTypeChange ::= ENUMERATED {true, ...}

BHRLCChannelID::= CHOICE {

lCID			LCID,

extendedLCID	ExtendedLCID,

...

}

>>>>>>>>>>>>>>> Unchanged parts are skipped<<<<<<<<<<<<<<<<

#### -- E

EgressBHRLCCHList ::= SEQUENCE (SIZE(1..maxnoofEgressLinks)) OF EgressBHRLCCHItem

EgressBHRLCCHItem ::= SEQUENCE {

nextHopBAPAddress 		BAPAddress,

bHRLCChannelID			BHRLCChannelID,

iE-Extensions			ProtocolExtensionContainer {{EgressBHRLCCHItemExtIEs }}	 OPTIONAL

}

EgressBHRLCCHItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Endpoint-IP-address-and-port ::=SEQUENCE {

endpointIPAddress TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-portNumber	CRITICALITY reject	EXTENSION PortNumber		PRESENCE optional},

...

}

ExplicitFormat ::=	SEQUENCE {

permutation			Permutation,

noofDownlinkSymbols	NoofDownlinkSymbols,

noofUplinkSymbols	NoofUplinkSymbols,

iE-Extensions		ProtocolExtensionContainer { { ExplicitFormat-ExtIEs} } OPTIONAL

}

ExplicitFormat-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedLCID ::= INTEGER (33..65536)

-------------------------------------------End of changes-------------------------------------------