3GPP TSG-RAN WG3 Meeting #108-e	R3-203822

E-meeting, 1 – 12 June, 2020

Agenda item:	10.2.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.423) Introduction of RACH Assistance Information

Document for:	Text Proposal

# 1	Introduction

This TP includes updates to the SON BL CR for TS 38.423, by introducing support for transfer of RACH assistance information on the Xn interface, comprising NR UE RACH Reports and together with RACH Success and RACH Failure Information originating at a gNB-DU, according to the discussion in [1].

The TP is based on [2].

# References

[1] 	R3-203820 Remaining issues for RACH conflict resolution, Nokia, Nokia Shanghai Bell Nokia,

[2]	R3-201614, BL CR to TS 38.423

# Annex	- Text Proposals for SON BL CR for TS 38.423

<<< start of changes >>>

### 8.4.C	Access and Mobility Indication (message name is FFS)

#### 8.4.C.1	General

The purpose of the Access and Mobility Indication procedure is to transfer Access and Mobility related information between NG-RAN nodes.

#### 8.4.C.2	Successful Operation

Figure 8.2.x.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from the NG-RAN node1.

#### 8.4.C.3	Abnormal Conditions

Not applicable.

<<< next change >>>

#### 9.1.3.y1	ACCESS AND MOBILITY INDICATION

This message is sent by the NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| RACH Conflict Resolution Info | O |  |  |  |  |  |
| >CU-DU RACH Failure and Success Information | O |  | 9.2.2.KK |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

<<< next change >>>

#### 9.2.2.KK	CU-DU RACH Failure and Success Information

The RACH Failure and Success Information IE contains RACH failure and RACH success information calculated at cells served by the NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  |  |  |
| >>Reporting Cell ID | M |  | NR CGI 9.2.2.7 | The cell for which the RACH failure rate is calculated. |  |  |
| >>Cell in gNB-DU | M |  | NR CGI 9.2.2.7 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of RACH Failures on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Successes on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Failures on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |
| >>>Number of RACH Successes on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBIDs | Maximum no. SSB IDs within an NR cell. Value is 64. |
| maxnoofSSBIds-1 | Maximum no. of SSB IDs within an NR cell minus 1. Value is 63. |

<<< end of changes >>>