3GPP TSG-RAN WG3 Meeting #108-e	R3-203828

# E-Meeting, June 1-11, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0516 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Supported Slices on E1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split |  |  |  |  |  |  | Date: |  | 2020-05-22 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With the existing E1 structure for slice support, it is not possible to correctly signal configurations where some slices (e.g., URLLC, V2X) are supported only for certain cells in a gNB-CU-UP. Thus, the gNB-CU-UP is incorrectly forced to indicate its slice support as supported for all of the cells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce an Extended Slice Support IE to allow configuring the slices supported for a list of NR-CGIs. Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change is limited to slices supported. The CR is ASN.1 backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB-CU-UP is incorrectly forced to indicate its slice support as supported for all of the cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3, 8.2.4, 8.2.5, 9.2.1.4, 9.2.1.8, 9.2.1.10, 9.4 (ASN.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Beginning of Text Proposal to TS 38.463

### 8.2.3	gNB-CU-UP E1 Setup

#### 8.2.3.1	General

The purpose of the gNB-CU-UP E1 Setup procedure is to exchange application level data needed for the gNB-CU-UP and the gNB-CU-CP to correctly interoperate on the E1 interface. If the gNB-CU-UP initiates the first TNL association, it shall also initiate the gNB-CU-UP E1 Setup procedure. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: gNB-CU-UP E1 Setup procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-CP. The gNB-CU-CP responds with a GNB-CU-UP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-UP E1 SETUP REQUEST message contains the gNB-CU-UP Name IE the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP.

If the Slice Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NR CGI Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the QoS Parameters Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall take this IE into account.

If the GNB-CU-UP E1 SETUP REQUEST message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-UP E1 SETUP RESPONSE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the Extended Slice Support IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-UP shall store the corresponding information and may take it into account for bearer context establishment.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: gNB-CU-UP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the setup, it shall respond with a GNB-CU-UP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-UP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-UP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-CP.

#### 8.2.3.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-CP E1 SETUP REQUEST, GNB-CU-UP E1 SETUP RESPONSE, or GNB-CU-UP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-UP does not receive either GNB-CU-UP E1 SETUP RESPONSE message or GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP may reinitiate the gNB-CU-UP E1 Setup procedure towards the same gNB-CU-CP, provided that the content of the new GNB-CU-UP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-UP E1 SETUP REQUEST message.

If the gNB-CU-UP receives a GNB-CU-CP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.3.3.

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP FAILURE message and receives a subsequent GNB-CU-UP E1 SETUP RESPONSE message, the gNB-CU-UP shall ignore the GNB-CU-UP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

<< unchanged parts omitted >>

### 8.2.4	gNB-CU-CP E1 Setup

#### 8.2.4.1	General

The purpose of the gNB-CU-CP E1 Setup procedure is to exchange application level data needed for the gNB-CU-CP and the gNB-CU-UP to correctly interoperate on the E1 interface. If the gNB-CU-CP initiates the first TNL association, it shall also initiate the gNB-CU-CP E1 Setup procedure.The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-CU-CP E1 Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-UP. The gNB-CU-UP responds with a GNB-CU-CP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-CP E1 SETUP REQUEST message contains the gNB-CU-CP Name IE the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall take this IE into account.

If the GNB-CU-CP E1 SETUP REQUEST message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-CP E1 SETUP RESPONSE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the Extended Slice Support IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-CU-CP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot accept the setup, it shall respond with a GNB-CU-CP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-CP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-UP.

#### 8.2.4.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-UP E1 SETUP REQUEST, GNB-CU-CP E1 SETUP RESPONSE, or GNB-CU-CP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-CP does not receive either GNB-CU-CP E1 SETUP RESPONSE message or GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP may reinitiate the gNB-CU-CP E1 Setup procedure towards the same gNB-CU-UP, provided that the content of the new GNB-CU-CP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-CP E1 SETUP REQUEST message.

If the gNB-CU-CP receives a GNB-CU-UP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.4.3.

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP FAILURE message and receives a subsequent GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall ignore the GNB-CU-CP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

<< unchanged parts omitted >>

### 8.2.5	gNB-CU-UP Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU-UP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-UP and the gNB-CU-CP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU-UP Configuration Update procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP CONFIGURATION UPDATE message to the gNB-CU-CP including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU-CP responds with GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall interpret that the corresponding configuration data is not changed and shall continue to operate with the existing related configuration data.

If the Supported PLMNs IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall overwrite the whole list of information and store the corresponding information.

-	If the Slice Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the NR CGI Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the QoS Parameters Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the Extended Slice Support IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall take this IE into account.

If the gNB-CU-UP ID IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall associate the TNLA to the E1 interface instance using the gNB-CU-UP ID.

If the GNB-CU-UP CONFIGURATION UPDATE message includes gNB-CU-UP TNLA To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-UP TNLA To Remove List IE, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-CU-UP. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU-UP TNLA To Remove List IE in GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-CU-UP.

If the GNB-CU-UP CONFIGURATION UPDATE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU-UP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the update, it shall respond with a GNB-CU-UP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-UP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-UP shall wait at least for the indicated time before reinitiating the GNB-CU-UP CONFIGURATION UPDATE message towards the same gNB-CU-CP.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

<< unchanged parts omitted >>

#### 9.2.1.4	GNB-CU-UP E1 SETUP REQUEST

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >Extended Slice Support | O |  | 9.3.1.x |  | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

<< unchanged parts omitted >>

#### 9.2.1.8	GNB-CU-CP E1 SETUP RESPONSE

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >Extended Slice Support | O |  | 9.3.1.x |  | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

<< unchanged parts omitted >>

#### 9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | O |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >Extended Slice Support | O |  | 9.3.1.x |  | - | - |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |

<< unchanged parts omitted >>

#### 9.3.1.x	Extended Slice Support

This IE indicates the list of extended slices supported.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support List | M |  | 9.3.1.8 | Supported S-NSSAI. |
| NR CGI Support List | M |  | 9.3.1.36 | Supported cells. |

<< unchanged parts omitted >>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

ResetAcknowledge,

ErrorIndication,

GNB-CU-UP-E1SetupRequest,

GNB-CU-UP-E1SetupResponse,

GNB-CU-UP-E1SetupFailure,

GNB-CU-CP-E1SetupRequest,

GNB-CU-CP-E1SetupResponse,

GNB-CU-CP-E1SetupFailure,

GNB-CU-UP-ConfigurationUpdate,

GNB-CU-UP-ConfigurationUpdateAcknowledge,

GNB-CU-UP-ConfigurationUpdateFailure,

GNB-CU-CP-ConfigurationUpdate,

GNB-CU-CP-ConfigurationUpdateAcknowledge,

GNB-CU-CP-ConfigurationUpdateFailure,

BearerContextSetupRequest,

BearerContextSetupResponse,

BearerContextSetupFailure,

BearerContextModificationRequest,

BearerContextModificationResponse,

BearerContextModificationFailure,

BearerContextModificationRequired,

BearerContextModificationConfirm,

BearerContextReleaseCommand,

BearerContextReleaseComplete,

BearerContextReleaseRequest,

BearerContextInactivityNotification,

DLDataNotification,

ULDataNotification,

DataUsageReport,

E1ReleaseRequest,

E1ReleaseResponse,

GNB-CU-UP-CounterCheckRequest,

GNB-CU-UP-StatusIndication,

MRDC-DataUsageReport,

DeactivateTrace,

TraceStart,

PrivateMessage

FROM E1AP-PDU-Contents

id-reset,

id-errorIndication,

id-gNB-CU-UP-E1Setup,

id-gNB-CU-CP-E1Setup,

id-gNB-CU-UP-ConfigurationUpdate,

id-gNB-CU-CP-ConfigurationUpdate,

id-e1Release,

id-bearerContextSetup,

id-bearerContextModification,

id-bearerContextModificationRequired,

id-bearerContextRelease,

id-bearerContextReleaseRequest,

id-bearerContextInactivityNotification,

id-dLDataNotification,

id-uLDataNotification,

id-dataUsageReport,

id-gNB-CU-UP-CounterCheck,

id-gNB-CU-UP-StatusIndication,

id-mRDC-DataUsageReport,

id-DeactivateTrace,

id-TraceStart,

id-privateMessage

FROM E1AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

E1AP-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome		UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

gNB-CU-UP-E1Setup							|

gNB-CU-CP-E1Setup							|

gNB-CU-UP-ConfigurationUpdate				|

gNB-CU-CP-ConfigurationUpdate				|

e1Release									|

bearerContextSetup							|

bearerContextModification					|

bearerContextModificationRequired			|

bearerContextRelease						,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

bearerContextReleaseRequest				|

bearerContextInactivityNotification		|

dLDataNotification						|

uLDataNotification						|

dataUsageReport							|

gNB-CU-UP-CounterCheck					|

gNB-CU-UP-StatusIndication			|

mRDC-DataUsageReport					|

deactivateTrace							|

traceStart								|

privateMessage							,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

gNB-CU-UP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-UP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-UP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-UP-E1Setup

CRITICALITY				reject

}

gNB-CU-CP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-CP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-CP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-CP-E1Setup

CRITICALITY				reject

}

gNB-CU-UP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-UP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-UP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-UP-ConfigurationUpdate

CRITICALITY				reject

}

gNB-CU-CP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-CP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-CP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-CP-ConfigurationUpdate

CRITICALITY				reject

}

e1Release E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E1ReleaseRequest

SUCCESSFUL OUTCOME		E1ReleaseResponse

PROCEDURE CODE			id-e1Release

CRITICALITY				reject

}

bearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextSetupRequest

SUCCESSFUL OUTCOME		BearerContextSetupResponse

UNSUCCESSFUL OUTCOME	BearerContextSetupFailure

PROCEDURE CODE			id-bearerContextSetup

CRITICALITY				reject

}

bearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequest

SUCCESSFUL OUTCOME		BearerContextModificationResponse

UNSUCCESSFUL OUTCOME	BearerContextModificationFailure

PROCEDURE CODE			id-bearerContextModification

CRITICALITY				reject

}

bearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequired

SUCCESSFUL OUTCOME		BearerContextModificationConfirm

PROCEDURE CODE			id-bearerContextModificationRequired

CRITICALITY				reject

}

bearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseCommand

SUCCESSFUL OUTCOME		BearerContextReleaseComplete

PROCEDURE CODE			id-bearerContextRelease

CRITICALITY				reject

}

bearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseRequest

PROCEDURE CODE			id-bearerContextReleaseRequest

CRITICALITY				ignore

}

bearerContextInactivityNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextInactivityNotification

PROCEDURE CODE			id-bearerContextInactivityNotification

CRITICALITY				ignore

}

dLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLDataNotification

PROCEDURE CODE			id-dLDataNotification

CRITICALITY				ignore

}

uLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULDataNotification

PROCEDURE CODE			id-uLDataNotification

CRITICALITY				ignore

}

dataUsageReport E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataUsageReport

PROCEDURE CODE			id-dataUsageReport

CRITICALITY				ignore

}

gNB-CU-UP-CounterCheck E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-CounterCheckRequest

PROCEDURE CODE			id-gNB-CU-UP-CounterCheck

CRITICALITY				ignore

}

gNB-CU-UP-StatusIndication 	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-StatusIndication

PROCEDURE CODE			id-gNB-CU-UP-StatusIndication

CRITICALITY			ignore

}

privateMessage E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mRDC-DataUsageReport	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MRDC-DataUsageReport

PROCEDURE CODE			id-mRDC-DataUsageReport

CRITICALITY				ignore

}

deactivateTrace E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

traceStart E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

TraceID,

TraceActivation,

SubscriberProfileIDforRFP,

AdditionalRRMPriorityIndex,

RetainabilityMeasurementsInfo,

Transport-Layer-Address-Info,

ExtendedSliceSupport

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-TraceID,

id-TraceActivation,

id-SubscriberProfileIDforRFP,

id-AdditionalRRMPriorityIndex,

id-RetainabilityMeasurementsInfo,

id-Transport-Layer-Address-Info,

id-ExtendedSliceSupport,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset

FROM E1AP-Constants;

-- **************************************************************

--

#### -- RESET

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

e1-Interface					ResetAll,

partOfE1-Interface				UE-associatedLogicalE1-ConnectionListRes,

choice-extension				ProtocolIE-SingleContainer	{{ResetType-ExtIEs}}

}

ResetType-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalE1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualE1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalE1-ConnectionItemRes } }

UE-associatedLogicalE1-ConnectionItemRes E1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalE1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalE1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-UE-associatedLogicalE1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalE1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalE1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualE1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalE1-ConnectionItemResAck } }

UE-associatedLogicalE1-ConnectionItemResAck 	E1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalE1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalE1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY ignore	TYPE GNB-CU-CP-UE-E1AP-ID		PRESENCE optional}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID		PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- GNB-CU-UP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Request

--

-- **************************************************************

GNB-CU-UP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupRequestIEs} },

...

}

GNB-CU-UP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

SupportedPLMNs-List	::=	SEQUENCE (SIZE (1..maxnoofSPLMNs)) OF SupportedPLMNs-Item

SupportedPLMNs-Item ::= SEQUENCE {

pLMN-Identity					PLMN-Identity,

slice-Support-List				Slice-Support-List												OPTIONAL,

nR-CGI-Support-List				NR-CGI-Support-List												OPTIONAL,

qoS-Parameters-Support-List		QoS-Parameters-Support-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SupportedPLMNs-ExtIEs } }		OPTIONAL,

...

}

SupportedPLMNs-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-extendedSliceSupport	CRITICALITY ignore	EXTENSION	ExtendedSliceSupport	PRESENCE optional},

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Response

--

-- **************************************************************

GNB-CU-UP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupResponseIEs} },

...

}

GNB-CU-UP-E1SetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Failure

--

-- **************************************************************

GNB-CU-UP-E1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupFailureIEs} },

...

}

GNB-CU-UP-E1SetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-CP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Request

--

-- **************************************************************

GNB-CU-CP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupRequestIEs} },

...

}

GNB-CU-CP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Response

--

-- **************************************************************

GNB-CU-CP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupResponseIEs} },

...

}

GNB-CU-CP-E1SetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Failure

--

-- **************************************************************

GNB-CU-CP-E1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupFailureIEs} },

...

}

GNB-CU-CP-E1SetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-UP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP Configuration Update

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory }|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE optional	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-GNB-CU-UP-TNLA-To-Remove-List		CRITICALITY reject	TYPE GNB-CU-UP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-UP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-UP-TNLA-To-Remove-Item

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Failure

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateFailureIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-CP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP Configuration Update

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name						PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Add-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Add-List		PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Remove-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Update-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Update-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-To-Add-List       ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Add-Item

GNB-CU-CP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Remove-Item

GNB-CU-CP-TNLA-To-Update-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Update-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Setup-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Setup-List					PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Failed-To-Setup-List		PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-Setup-List				::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Setup-Item

GNB-CU-CP-TNLA-Failed-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Failed-To-Setup-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Failure

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateFailureIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- E1 RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- E1 Release Request

--

-- **************************************************************

E1ReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E1ReleaseRequestIEs} },

...

}

E1ReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- E1 Release Response

--

-- **************************************************************

E1ReleaseResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E1ReleaseResponseIEs} },

...

}

E1ReleaseResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Setup Request

--

-- **************************************************************

BearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupRequestIEs} },

...

}

BearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-SecurityInformation					CRITICALITY reject	TYPE SecurityInformation							PRESENCE mandatory }|

{ ID id-UEDLAggregateMaximumBitRate			CRITICALITY reject	TYPE BitRate										PRESENCE mandatory }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate	CRITICALITY reject		TYPE BitRate								PRESENCE optional	 }|

{ ID id-Serving-PLMN						CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE mandatory }|

{ ID id-ActivityNotificationLevel			CRITICALITY reject	TYPE ActivityNotificationLevel					PRESENCE mandatory }|

{ ID id-UE-Inactivity-Timer					CRITICALITY reject	TYPE Inactivity-Timer								PRESENCE optional  }|

{ ID id-BearerContextStatusChange			CRITICALITY reject	TYPE BearerContextStatusChange					PRESENCE optional  }|

{ ID id-System-BearerContextSetupRequest	CRITICALITY reject	TYPE System-BearerContextSetupRequest				PRESENCE mandatory }|

{ ID id-RANUEID								CRITICALITY ignore	TYPE RANUEID										PRESENCE optional }|

{ ID id-GNB-DU-ID							CRITICALITY ignore	TYPE GNB-DU-ID										PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional },

...

}

System-BearerContextSetupRequest	::=	CHOICE {

e-UTRAN-BearerContextSetupRequest		ProtocolIE-Container		 	{{EUTRAN-BearerContextSetupRequest}},

nG-RAN-BearerContextSetupRequest		ProtocolIE-Container		 	{{NG-RAN-BearerContextSetupRequest}},

choice-extension						ProtocolIE-SingleContainer		{{System-BearerContextSetupRequest-ExtIEs}}

}

System-BearerContextSetupRequest-ExtIEs E1AP-PROTOCOL-IES::= {

...

}

EUTRAN-BearerContextSetupRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-To-Setup-List-EUTRAN		CRITICALITY reject	 TYPE DRB-To-Setup-List-EUTRAN		PRESENCE mandatory }|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	 TYPE SubscriberProfileIDforRFP		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex		CRITICALITY ignore	 TYPE AdditionalRRMPriorityIndex	PRESENCE optional },

...

}

NG-RAN-BearerContextSetupRequest E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-To-Setup-List		CRITICALITY reject	 TYPE PDU-Session-Resource-To-Setup-List		PRESENCE mandatory },

...

}

-- **************************************************************

--

-- Bearer Context Setup Response

--

-- **************************************************************

BearerContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupResponseIEs} },

...

}

BearerContextSetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory	}|

{ ID id-System-BearerContextSetupResponse	CRITICALITY ignore	TYPE System-BearerContextSetupResponse				PRESENCE mandatory  },

...

}

System-BearerContextSetupResponse::=	CHOICE {

e-UTRAN-BearerContextSetupResponse		ProtocolIE-Container 			{{EUTRAN-BearerContextSetupResponse}},

nG-RAN-BearerContextSetupResponse		ProtocolIE-Container 			{{NG-RAN-BearerContextSetupResponse}},

choice-extension						ProtocolIE-SingleContainer		{{System-BearerContextSetupResponse-ExtIEs}}

}

System-BearerContextSetupResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextSetupResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-List-EUTRAN 		CRITICALITY ignore	 TYPE DRB-Setup-List-EUTRAN			PRESENCE mandatory }|

{ ID id-DRB-Failed-List-EUTRAN 		CRITICALITY ignore	 TYPE DRB-Failed-List-EUTRAN		PRESENCE optional  },

...

}

NG-RAN-BearerContextSetupResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-List 	CRITICALITY ignore	 TYPE PDU-Session-Resource-Setup-List		PRESENCE mandatory }|

{ ID id-PDU-Session-Resource-Failed-List 	CRITICALITY ignore	 TYPE PDU-Session-Resource-Failed-List		PRESENCE optional  },

...

}

-- **************************************************************

--

-- Bearer Context Setup Failure

--

-- **************************************************************

BearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupFailureIEs} },

...

}

BearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Modification Request

--

-- **************************************************************

BearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequestIEs} },

...

}

BearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-SecurityInformation						CRITICALITY reject	TYPE SecurityInformation									PRESENCE optional  }|

{ ID id-UEDLAggregateMaximumBitRate				CRITICALITY reject	TYPE BitRate												PRESENCE optional  }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate		CRITICALITY reject	TYPE BitRate											PRESENCE optional	 }|

{ ID id-BearerContextStatusChange				CRITICALITY reject	TYPE BearerContextStatusChange							PRESENCE optional  }|

{ ID id-New-UL-TNL-Information-Required			CRITICALITY reject	TYPE New-UL-TNL-Information-Required					PRESENCE optional  }|

{ ID id-UE-Inactivity-Timer						CRITICALITY reject	TYPE Inactivity-Timer										PRESENCE optional  }|

{ ID id-DataDiscardRequired					CRITICALITY ignore	TYPE DataDiscardRequired											PRESENCE optional  }|

{ ID id-System-BearerContextModificationRequest	CRITICALITY reject	TYPE System-BearerContextModificationRequest		PRESENCE optional  }|

{ ID id-RANUEID										CRITICALITY ignore	TYPE RANUEID												PRESENCE optional }|

{ ID id-GNB-DU-ID									CRITICALITY ignore	TYPE GNB-DU-ID												PRESENCE optional }|

{ ID id-ActivityNotificationLevel					CRITICALITY ignore	TYPE ActivityNotificationLevel							PRESENCE optional },

...

}

System-BearerContextModificationRequest	::=	CHOICE {

e-UTRAN-BearerContextModificationRequest		ProtocolIE-Container 			{{EUTRAN-BearerContextModificationRequest}},

nG-RAN-BearerContextModificationRequest			ProtocolIE-Container 			{{NG-RAN-BearerContextModificationRequest}},

choice-extension								ProtocolIE-SingleContainer		{{System-BearerContextModificationRequest-ExtIEs}}

}

System-BearerContextModificationRequest-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-To-Setup-Mod-List-EUTRAN		CRITICALITY reject	 TYPE DRB-To-Setup-Mod-List-EUTRAN		PRESENCE optional }|

{ ID id-DRB-To-Modify-List-EUTRAN			CRITICALITY reject	 TYPE DRB-To-Modify-List-EUTRAN			PRESENCE optional }|

{ ID id-DRB-To-Remove-List-EUTRAN			CRITICALITY reject	 TYPE DRB-To-Remove-List-EUTRAN			PRESENCE optional }|

{ ID id-SubscriberProfileIDforRFP			CRITICALITY ignore	 TYPE 	SubscriberProfileIDforRFP		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	 TYPE 	AdditionalRRMPriorityIndex		PRESENCE optional },

...

}

NG-RAN-BearerContextModificationRequest E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-To-Setup-Mod-List	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Setup-Mod-List	PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Modify-List 	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Modify-List		PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Remove-List		CRITICALITY reject	 TYPE PDU-Session-Resource-To-Remove-List		PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Response

--

-- **************************************************************

BearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationResponseIEs} },

...

}

BearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-System-BearerContextModificationResponse	CRITICALITY ignore	TYPE System-BearerContextModificationResponse		PRESENCE optional  },

...

}

System-BearerContextModificationResponse	::=	CHOICE {

e-UTRAN-BearerContextModificationResponse			ProtocolIE-Container {{EUTRAN-BearerContextModificationResponse}},

nG-RAN-BearerContextModificationResponse			ProtocolIE-Container {{NG-RAN-BearerContextModificationResponse}},

choice-extension									ProtocolIE-SingleContainer {{System-BearerContextModificationResponse-ExtIEs}}

}

System-BearerContextModificationResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Setup-Mod-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Failed-Mod-List-EUTRAN				PRESENCE optional }|

{ ID id-DRB-Modified-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Modified-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-To-Modify-List-EUTRAN		CRITICALITY ignore	TYPE DRB-Failed-To-Modify-List-EUTRAN		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo			CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo				PRESENCE optional },

...

}

NG-RAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-Mod-List 		CRITICALITY reject	TYPE PDU-Session-Resource-Setup-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-Mod-List		CRITICALITY reject	TYPE PDU-Session-Resource-Failed-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Modified-List			CRITICALITY reject	TYPE PDU-Session-Resource-Modified-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Failed-To-Modify-List		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo				CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo						PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Failure

--

-- **************************************************************

BearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationFailureIEs} },

...

}

BearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Modification Required

--

-- **************************************************************

BearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequiredIEs} },

...

}

BearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-System-BearerContextModificationRequired	CRITICALITY reject	TYPE System-BearerContextModificationRequired			PRESENCE mandatory },

...

}

System-BearerContextModificationRequired	::=	CHOICE {

e-UTRAN-BearerContextModificationRequired		ProtocolIE-Container {{EUTRAN-BearerContextModificationRequired}},

nG-RAN-BearerContextModificationRequired		ProtocolIE-Container {{NG-RAN-BearerContextModificationRequired}},

choice-extension								ProtocolIE-SingleContainer {{System-BearerContextModificationRequired-ExtIEs}}

}

System-BearerContextModificationRequired-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationRequired E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Required-To-Modify-List-EUTRAN	CRITICALITY reject	TYPE DRB-Required-To-Modify-List-EUTRAN	PRESENCE optional }|

{ ID id-DRB-Required-To-Remove-List-EUTRAN	CRITICALITY reject	TYPE DRB-Required-To-Remove-List-EUTRAN	PRESENCE optional },

...

}

NG-RAN-BearerContextModificationRequired E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Required-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Required-To-Modify-List	PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Remove-List	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Remove-List	PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Confirm

--

-- **************************************************************

BearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationConfirmIEs} },

...

}

BearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-System-BearerContextModificationConfirm		CRITICALITY ignore	TYPE System-BearerContextModificationConfirm		PRESENCE optional  },

...

}

System-BearerContextModificationConfirm	::=	CHOICE {

e-UTRAN-BearerContextModificationConfirm		ProtocolIE-Container {{EUTRAN-BearerContextModificationConfirm}},

nG-RAN-BearerContextModificationConfirm			ProtocolIE-Container {{NG-RAN-BearerContextModificationConfirm}},

choice-extension								ProtocolIE-SingleContainer {{System-BearerContextModificationConfirm-ExtIEs}}

}

System-BearerContextModificationConfirm-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationConfirm E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Confirm-Modified-List-EUTRAN	CRITICALITY ignore	 TYPE DRB-Confirm-Modified-List-EUTRAN	PRESENCE optional },

...

}

NG-RAN-BearerContextModificationConfirm E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Confirm-Modified-List	CRITICALITY ignore	 TYPE PDU-Session-Resource-Confirm-Modified-List	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Release Command

--

-- **************************************************************

BearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCommandIEs} },

...

}

BearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory },

...

}

-- **************************************************************

--

-- Bearer Context Release Complete

--

-- **************************************************************

BearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCompleteIEs} },

...

}

BearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	}|

{ ID id-RetainabilityMeasurementsInfo	CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Release Request

--

-- **************************************************************

BearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseRequestIEs} },

...

}

BearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-DRB-Status-List					CRITICALITY ignore	TYPE DRB-Status-List				PRESENCE optional  }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory },

...

}

DRB-Status-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Status-Item

-- **************************************************************

--

#### -- BEARER CONTEXT INACTIVITY NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Inactivity Notification

--

-- **************************************************************

BearerContextInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextInactivityNotificationIEs } },

...

}

BearerContextInactivityNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-ActivityInformation				CRITICALITY reject	TYPE ActivityInformation			PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DL DATA NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- DL Data Notification

--

-- **************************************************************

DLDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DLDataNotificationIEs } },

...

}

DLDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PPI							CRITICALITY ignore	TYPE PPI							PRESENCE optional },

...

}

-- **************************************************************

-- **************************************************************

--

-- UL Data Notification

--

-- **************************************************************

ULDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ULDataNotificationIEs } },

...

}

ULDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PDU-Session-To-Notify-List		CRITICALITY reject	 TYPE PDU-Session-To-Notify-List	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DATA USAGE REPORT

--

-- **************************************************************

-- **************************************************************

--

-- Data Usage Report

--

-- **************************************************************

DataUsageReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DataUsageReportIEs } },

...

}

DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-Data-Usage-Report-List			CRITICALITY ignore	TYPE Data-Usage-Report-List			PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- GNB-CU-UP COUNTER CHECK

--

-- **************************************************************

-- **************************************************************

--

-- gNB-CU-UP Counter Check Request

--

-- **************************************************************

GNB-CU-UP-CounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNB-CU-UP-CounterCheckRequestIEs } },

...

}

GNB-CU-UP-CounterCheckRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-System-GNB-CU-UP-CounterCheckRequest	CRITICALITY reject	TYPE System-GNB-CU-UP-CounterCheckRequest	PRESENCE mandatory },

...

}

System-GNB-CU-UP-CounterCheckRequest	::=	CHOICE {

e-UTRAN-GNB-CU-UP-CounterCheckRequest		ProtocolIE-Container		{{EUTRAN-GNB-CU-UP-CounterCheckRequest}},

nG-RAN-GNB-CU-UP-CounterCheckRequest		ProtocolIE-Container		{{NG-RAN-GNB-CU-UP-CounterCheckRequest}},

choice-extension							ProtocolIE-SingleContainer	{{System-GNB-CU-UP-CounterCheckRequest-ExtIEs}}

}

System-GNB-CU-UP-CounterCheckRequest-ExtIEs E1AP-PROTOCOL-IES::= {

...

}

EUTRAN-GNB-CU-UP-CounterCheckRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Subject-To-Counter-Check-List-EUTRAN	CRITICALITY ignore	 TYPE DRBs-Subject-To-Counter-Check-List-EUTRAN	PRESENCE mandatory },

...

}

NG-RAN-GNB-CU-UP-CounterCheckRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Subject-To-Counter-Check-List-NG-RAN	CRITICALITY ignore	 TYPE DRBs-Subject-To-Counter-Check-List-NG-RAN	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- gNB-CU-UP STATUS INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- gNB-CU-UP Status Indication

--

-- **************************************************************

GNB-CU-UP-StatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNB-CU-UP-StatusIndicationIEs} },

...

}

GNB-CU-UP-StatusIndicationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID				 			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-OverloadInformation	CRITICALITY reject	TYPE GNB-CU-UP-OverloadInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MR-DC DATA USAGE REPORT

--

-- **************************************************************

MRDC-DataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MRDC-DataUsageReportIEs } },

...

}

MRDC-DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-PDU-Session-Resource-Data-Usage-List	CRITICALITY ignore		TYPE PDU-Session-Resource-Data-Usage-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceID							CRITICALITY ignore	TYPE TraceID							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs E1AP-PRIVATE-IES ::= {

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AveragingWindow  ::= INTEGER (0..4095, ...)

#### -- B

BearerContextStatusChange	::= 	ENUMERATED {

suspend,

resume,

...

}

BitRate ::= INTEGER (0..4000000000000,...)

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer	{{Cause-ExtIEs}}

}

Cause-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

unknown-or-already-allocated-gnb-cu-cp-ue-e1ap-id,

unknown-or-already-allocated-gnb-cu-up-ue-e1ap-id,

unknown-or-inconsistent-pair-of-ue-e1ap-id,

interaction-with-other-procedure,

pPDCP-Count-wrap-around,

not-supported-QCI-value,

not-supported-5QI-value,

encryption-algorithms-not-supported,

integrity-protection-algorithms-not-supported,

uP-integrity-protection-not-possible,

uP-confidentiality-protection-not-possible,

multiple-PDU-Session-ID-Instances,

unknown-PDU-Session-ID,

multiple-QoS-Flow-ID-Instances,

unknown-QoS-Flow-ID,

multiple-DRB-ID-Instances,

unknown-DRB-ID,

invalid-QoS-combination,

procedure-cancelled,

normal-release,

no-radio-resources-available,

action-desirable-for-radio-reasons,

resources-not-available-for-the-slice,

pDCP-configuration-not-supported,

...,

ue-dl-max-IP-data-rate-reason,

uP-integrity-protection-failure,

release-due-to-pre-emption

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...

}

Cell-Group-Information	::= SEQUENCE (SIZE(1.. maxnoofCellGroups)) OF Cell-Group-Information-Item

Cell-Group-Information-Item	::=	SEQUENCE {

cell-Group-ID							Cell-Group-ID,

uL-Configuration						UL-Configuration		OPTIONAL,

dL-TX-Stop								DL-TX-Stop				OPTIONAL,

rAT-Type								RAT-Type				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Cell-Group-Information-Item-ExtIEs } }	OPTIONAL,

...

}

Cell-Group-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Group-ID	::=		INTEGER (0..3, ...)

CipheringAlgorithm	::=	ENUMERATED {

nEA0,

c-128-NEA1,

c-128-NEA2,

c-128-NEA3,

...

}

CNSupport ::= ENUMERATED {

c-epc,

c-5gc,

both,

...

}

CommonNetworkInstance ::= OCTET STRING

ConfidentialityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

CP-TNL-Information		::= 	CHOICE {

endpoint-IP-Address		TransportLayerAddress,

choice-extension		ProtocolIE-SingleContainer	{{CP-TNL-Information-ExtIEs}}

}

CP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

{ ID id-endpoint-IP-Address-and-Port	CRITICALITY reject	TYPE Endpoint-IP-address-and-port 	PRESENCE mandatory},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

transactionID					TransactionID					OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxnoofErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-QoS-Flow-List	::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF Data-Usage-per-QoS-Flow-Item

Data-Usage-per-QoS-Flow-Item ::= SEQUENCE {

qoS-Flow-Identifier		QoS-Flow-Identifier,

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

qoS-Flow-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-QoS-Flow-Item-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-QoS-Flow-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-Report-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF Data-Usage-Report-Item

Data-Usage-Report-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

rAT-Type					RAT-Type,

dRB-Usage-Report-List		DRB-Usage-Report-List,

iE-Extensions	ProtocolExtensionContainer { { Data-Usage-Report-ItemExtIEs } }	OPTIONAL,

...

}

Data-Usage-Report-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DefaultDRB	::=	ENUMERATED	{

true,

false,

...

}

DiscardTimer	::=	ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity}

DL-TX-Stop	::=	ENUMERATED	{

stop,

resume,

...

}

DRB-Activity	::= ENUMERATED {

active,

not-active,

...

}

DRB-Activity-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Activity-Item

DRB-Activity-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

dRB-Activity				DRB-Activity,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-EUTRAN

DRB-Confirm-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-NG-RAN

DRB-Confirm-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-EUTRAN

DRB-Failed-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-EUTRAN

DRB-Failed-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-NG-RAN

DRB-Failed-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-NG-RAN

DRB-Failed-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-EUTRAN

DRB-Failed-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-NG-RAN

DRB-Failed-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-ID	::=	INTEGER (1..32, ...)

DRB-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-EUTRAN

DRB-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-NG-RAN

DRB-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-UP-Transport-Parameters					UP-Parameters							OPTIONAL,

pDCP-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

flow-Setup-List								QoS-Flow-List							OPTIONAL,

flow-Failed-List							QoS-Flow-Failed-List					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Removed-Item	::=	SEQUENCE {

dRB-ID									DRB-ID,

dRB-Released-In-Session					ENUMERATED {released-in-session, not-released-in-session, ...}		OPTIONAL,

dRB-Accumulated-Session-Time			OCTET STRING (SIZE(5))													OPTIONAL,

qoS-Flow-Removed-List					SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Removed-Item		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Removed-Item-ExtIEs } }			OPTIONAL,

...

}

DRB-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-EUTRAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-EUTRAN

DRB-Required-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information							OPTIONAL,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-NG-RAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-NG-RAN

DRB-Required-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

flow-To-Remove							QoS-Flow-List								OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-EUTRAN

DRB-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

s1-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-EUTRAN

DRB-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-NG-RAN

DRB-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-NG-RAN

DRB-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Status-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

pDCP-DL-Count				PDCP-Count		OPTIONAL,

pDCP-UL-Count				PDCP-Count		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Status-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Status-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-EUTRAN

DRBs-Subject-To-Counter-Check-Item-EUTRAN	::=	SEQUENCE {

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions		ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-NG-RAN

DRBs-Subject-To-Counter-Check-Item-NG-RAN	::=	SEQUENCE {

pDU-Session-ID				PDU-Session-ID,

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions			ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-EUTRAN

DRB-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration						OPTIONAL,

eUTRAN-QoS								EUTRAN-QoS								OPTIONAL,

s1-UL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request					PDCP-SN-Status-Request						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

cell-Group-To-Add						Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify					Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove					Cell-Group-Information					OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-NG-RAN

DRB-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration						OPTIONAL,

pDCP-Configuration							PDCP-Configuration						OPTIONAL,

dRB-Data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request							PDCP-SN-Status-Request						OPTIONAL,

pdcp-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

cell-Group-To-Add							Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify						Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove						Cell-Group-Information					OPTIONAL,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List				OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject	EXTENSION QoS-Flow-List	PRESENCE optional}|

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-EUTRAN

DRB-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-EUTRAN

DRB-Required-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-NG-RAN

DRB-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-NG-RAN

DRB-Required-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-EUTRAN

DRB-To-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration,

eUTRAN-QoS								EUTRAN-QoS,

s1-UL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information					Cell-Group-Information,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

existing-Allocated-S1-DL-UP-TNL-Info	UP-TNL-Information						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-EUTRAN

DRB-To-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

pDCP-Configuration							PDCP-Configuration,

eUTRAN-QoS									EUTRAN-QoS,

s1-UL-UP-TNL-Information					UP-TNL-Information,

data-Forwarding-Information-Request			Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information						Cell-Group-Information,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-To-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-NG-RAN

DRB-To-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer					OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-Usage-Report-List ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF DRB-Usage-Report-Item

DRB-Usage-Report-Item	::= SEQUENCE {

startTimeStamp					OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions					ProtocolExtensionContainer { { DRB-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

DRB-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Duplication-Activation	::=		ENUMERATED {

active,

inactive,

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel					QoSPriorityLevel,

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

fiveQI								INTEGER (0..255, ...)								OPTIONAL,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

averagingWindow 					AveragingWindow										OPTIONAL,

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DataDiscardRequired	::= 	ENUMERATED {

required,

...

}

#### -- E

EncryptionKey	::=	OCTET STRING

Endpoint-IP-address-and-port::= SEQUENCE {

endpoint-IP-Address			TransportLayerAddress,

portNumber					PortNumber,

iE-Extensions							ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {EUTRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

EUTRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofEUTRANQOSParameters)) OF EUTRAN-QoS-Support-Item

EUTRAN-QoS-Support-Item ::= SEQUENCE {

eUTRAN-QoS	EUTRAN-QoS,

iE-Extensions			ProtocolExtensionContainer { { EUTRAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

EUTRAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS	::= SEQUENCE {

qCI										QCI,

eUTRANallocationAndRetentionPriority	EUTRANAllocationAndRetentionPriority,

gbrQosInformation						GBR-QosInformation										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { EUTRAN-QoS-ExtIEs } }	OPTIONAL,

...

}

EUTRAN-QoS-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedSliceSupport ::= SEQUENCE {

slice-Support-List				Slice-Support-List,

nR-CGI-Support-List				NR-CGI-Support-List,

iE-Extensions					ProtocolExtensionContainer { { ExtendedSliceSupport-ExtIEs } }	OPTIONAL,

...

}

ExtendedSliceSupport-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- F

#### -- G

GNB-CU-CP-Name 				::= PrintableString(SIZE(1..150,...))

GNB-CU-CP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-UP-Capacity				::= INTEGER (0..255)

GNB-CU-UP-CellGroupRelatedConfiguration	 ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF GNB-CU-UP-CellGroupRelatedConfiguration-Item

GNB-CU-UP-CellGroupRelatedConfiguration-Item ::= SEQUENCE {

cell-Group-ID				Cell-Group-ID,

uP-TNL-Information			UP-TNL-Information,

uL-Configuration			UL-Configuration		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs } }	OPTIONAL

}

GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-ID				::= INTEGER (0..68719476735)

GNB-CU-UP-Name 				::= PrintableString(SIZE(1..150,...))

GNB-CU-UP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-CP-TNLA-Setup-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

GNB-CU-CP-TNLA-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Add-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressgNBCUUP	CRITICALITY reject	EXTENSION CP-TNL-Information	PRESENCE optional},

...

}

GNB-CU-CP-TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Update-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress			CP-TNL-Information,

tNLAssociationTransportLayerAddressgNBCUCP	CP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }         OPTIONAL,

...

}

GTPTLA-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTPTunnel				::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { GTPTunnel-ExtIEs} } OPTIONAL,

...

}

GTPTunnel-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-OverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GNB-DU-ID	::= INTEGER (0..68719476735)

#### -- H

HFN		::=		INTEGER	(0..4294967295)

#### -- I

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionAlgorithm	::=	ENUMERATED {

nIA0,

i-128-NIA1,

i-128-NIA2,

i-128-NIA3,

...

}

IntegrityProtectionKey	::= OCTET STRING

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

Inactivity-Timer	::=	INTEGER (1..7200, ...)

InterfacesToTrace ::= BIT STRING (SIZE(8))

#### -- J

#### -- K

#### -- L

#### -- M

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MaxPacketLossRate ::= INTEGER (0..1000, ...)

MRDC-Data-Usage-Report-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp				OCTET STRING (SIZE(4)),

usageCountUL				INTEGER (0..18446744073709551615),

usageCountDL				INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { { MRDC-Data-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

MRDC-Data-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRDC-Usage-Information ::= SEQUENCE {

data-Usage-per-PDU-Session-Report		Data-Usage-per-PDU-Session-Report			OPTIONAL,

data-Usage-per-QoS-Flow-List			Data-Usage-per-QoS-Flow-List				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MRDC-Usage-Information-ExtIEs} } OPTIONAL,

...

}

MRDC-Usage-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

NetworkInstance ::= INTEGER (1..256, ...)

New-UL-TNL-Information-Required::= 	ENUMERATED {

required,

...

}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNGRANQOSParameters)) OF NG-RAN-QoS-Support-Item

NG-RAN-QoS-Support-Item ::= SEQUENCE {

non-Dynamic5QIDescriptor	Non-Dynamic5QIDescriptor,

iE-Extensions				ProtocolExtensionContainer { { NG-RAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

NG-RAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

Non-Dynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			QoSPriorityLevel				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Non-Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Non-Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-Cell-Identity	::=		BIT STRING (SIZE(36))

NR-CGI ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

nR-Cell-Identity		NR-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { { NR-CGI-ExtIEs } }	OPTIONAL

}

NR-CGI-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-CGI-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNRCGI)) OF NR-CGI-Support-Item

NR-CGI-Support-Item ::= SEQUENCE {

nR-CGI	NR-CGI,

iE-Extensions				ProtocolExtensionContainer { { NR-CGI-Support-Item-ExtIEs } }	OPTIONAL

}

NR-CGI-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- O

OutOfOrderDelivery	::=	ENUMERATED	{

true,

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PDCP-Configuration	::=	SEQUENCE {

pDCP-SN-Size-UL							PDCP-SN-Size,

pDCP-SN-Size-DL							PDCP-SN-Size,

rLC-Mode								RLC-Mode,

rOHC-Parameters							ROHC-Parameters			OPTIONAL,

t-ReorderingTimer						T-ReorderingTimer		OPTIONAL,

discardTimer							DiscardTimer			OPTIONAL,

uLDataSplitThreshold					ULDataSplitThreshold	OPTIONAL,

pDCP-Duplication						PDCP-Duplication		OPTIONAL,

pDCP-Reestablishment					PDCP-Reestablishment	OPTIONAL,

pDCP-DataRecovery						PDCP-DataRecovery		OPTIONAL,

duplication-Activation				Duplication-Activation		OPTIONAL,

outOfOrderDelivery						OutOfOrderDelivery		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Configuration-ExtIEs } }	OPTIONAL,

...

}

PDCP-Configuration-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-Count	::=	SEQUENCE {

pDCP-SN				PDCP-SN,

hFN					HFN,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Count-ExtIEs } }	OPTIONAL,

...

}

PDCP-Count-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN-Status-Request	::=		ENUMERATED {

requested,

...

}

PDCP-DataRecovery	::=	ENUMERATED	{

true,

...

}

PDCP-Duplication	::=	ENUMERATED	{

true,

...

}

PDCP-Reestablishment	::=	ENUMERATED	{

true,

...

}

PDU-Session-Resource-Data-Usage-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Data-Usage-Item

PDU-Session-Resource-Data-Usage-Item	::= SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

mRDC-Usage-Information						MRDC-Usage-Information,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Data-Usage-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Data-Usage-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN		::=		INTEGER	(0..262143)

PDCP-SN-Size	::=	ENUMERATED	{

s-12,

s-18,

...

}

PDCP-SN-Status-Information ::= SEQUENCE {

pdcpStatusTransfer-UL	DRBBStatusTransfer,

pdcpStatusTransfer-DL	PDCP-Count,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransfer-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

countValue				PDCP-Count,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-ID	::=	INTEGER (0..255)

PDU-Session-Resource-Activity	::= ENUMERATED {

active,

not-active,

...

}

PDU-Session-Resource-Activity-List ::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Activity-Item

PDU-Session-Resource-Activity-Item	::= SEQUENCE {

pDU-Session-ID									PDU-Session-ID,

pDU-Session-Resource-Activity					PDU-Session-Resource-Activity,

iE-Extensions	ProtocolExtensionContainer 		{ { PDU-Session-Resource-Activity-ItemExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Confirm-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Confirm-Modified-Item

PDU-Session-Resource-Confirm-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

dRB-Confirm-Modified-List-NG-RAN		DRB-Confirm-Modified-List-NG-RAN	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Confirm-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Confirm-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Item

PDU-Session-Resource-Failed-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Mod-Item

PDU-Session-Resource-Failed-Mod-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-To-Modify-Item

PDU-Session-Resource-Failed-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Modified-Item

PDU-Session-Resource-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

securityResult							SecurityResult							OPTIONAL,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN					OPTIONAL,

dRB-Failed-List-NG-RAN				DRB-Failed-List-NG-RAN				OPTIONAL,

dRB-Modified-List-NG-RAN				DRB-Modified-List-NG-RAN				OPTIONAL,

dRB-Failed-To-Modify-List-NG-RAN		DRB-Failed-To-Modify-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Required-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Required-To-Modify-Item

PDU-Session-Resource-Required-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

dRB-Required-To-Modify-List-NG-RAN		DRB-Required-To-Modify-List-NG-RAN		OPTIONAL,

dRB-Required-To-Remove-List-NG-RAN				DRB-Required-To-Remove-List-NG-RAN				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Required-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Required-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Item

PDU-Session-Resource-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

securityResult							SecurityResult				OPTIONAL,

nG-DL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

nG-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN,

dRB-Failed-List-NG-RAN					DRB-Failed-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Mod-Item

PDU-Session-Resource-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID											PDU-Session-ID,

securityResult											SecurityResult								OPTIONAL,

nG-DL-UP-TNL-Information								UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-Mod-List-NG-RAN								DRB-Setup-Mod-List-NG-RAN,

dRB-Failed-Mod-List-NG-RAN								DRB-Failed-Mod-List-NG-RAN					OPTIONAL,

iE-Extensions											ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Modify-Item

PDU-Session-Resource-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

securityIndication									SecurityIndication						OPTIONAL,

pDU-Session-Resource-DL-AMBR						BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information						OPTIONAL,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Data-Forwarding-Information	Data-Forwarding-Information	OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

networkInstance										NetworkInstance							OPTIONAL,

dRB-To-Setup-List-NG-RAN							DRB-To-Setup-List-NG-RAN				OPTIONAL,

dRB-To-Modify-List-NG-RAN							DRB-To-Modify-List-NG-RAN				OPTIONAL,

dRB-To-Remove-List-NG-RAN					DRB-To-Remove-List-NG-RAN		OPTIONAL,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SNSSAI		CRITICALITY reject	EXTENSION SNSSAI		PRESENCE optional}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	},

...

}

PDU-Session-Resource-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Remove-Item

PDU-Session-Resource-To-Remove-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Remove-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Remove-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Cause		CRITICALITY ignore	EXTENSION Cause		PRESENCE optional},

...

}

PDU-Session-Resource-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Item

PDU-Session-Resource-To-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

pDU-Session-Type						PDU-Session-Type,

sNSSAI									SNSSAI,

securityIndication						SecurityIndication,

pDU-Session-Resource-DL-AMBR			BitRate						OPTIONAL,

nG-UL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer			Inactivity-Timer	OPTIONAL,

existing-Allocated-NG-DL-UP-TNL-Info	UP-TNL-Information		OPTIONAL,

networkInstance							NetworkInstance		OPTIONAL,

dRB-To-Setup-List-NG-RAN				DRB-To-Setup-List-NG-RAN,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	},

...

}

PDU-Session-Resource-To-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Mod-Item

PDU-Session-Resource-To-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

pDU-Session-Type									PDU-Session-Type,

sNSSAI												SNSSAI,

securityIndication									SecurityIndication,

pDU-Session-Resource-AMBR							BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

dRB-To-Setup-Mod-List-NG-RAN						DRB-To-Setup-Mod-List-NG-RAN,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-NetworkInstance		CRITICALITY ignore	EXTENSION NetworkInstance		PRESENCE optional}|

{ID id-CommonNetworkInstance	CRITICALITY ignore	EXTENSION CommonNetworkInstance	PRESENCE optional},

...

}

PDU-Session-To-Notify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-To-Notify-Item

PDU-Session-To-Notify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

qoS-Flow-List							QoS-Flow-List,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-To-Notify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-To-Notify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Type ::= ENUMERATED {

ipv4,

ipv6,

ipv4v6,

ethernet,

unstructured,

...

}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PortNumber ::= BIT STRING (SIZE(16))

PPI ::= INTEGER (0..7, ...)

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

#### -- Q

QCI ::= INTEGER (0..255)

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				Non-Dynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer	{{QoS-Characteristics-ExtIEs}}

}

QoS-Characteristics-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

QoS-Flow-Identifier	::=	INTEGER (0..63)

QoS-Flow-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Item

QoS-Flow-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoS-Flow-Mapping-Indication		PRESENCE optional},

...

}

QoS-Flow-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Failed-Item

QoS-Flow-Failed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Mapping-Item

QoS-Flow-Mapping-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowMappingIndication						QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Mapping-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Mapping-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-Indication ::= ENUMERATED {ul, dl, ...}

QoS-Parameters-Support-List	::= SEQUENCE {

eUTRAN-QoS-Support-List			EUTRAN-QoS-Support-List			OPTIONAL,

nG-RAN-QoS-Support-List			NG-RAN-QoS-Support-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { QoS-Parameters-Support-List-ItemExtIEs} } OPTIONAL,

...

}

QoS-Parameters-Support-List-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSPriorityLevel ::= INTEGER (0..127, ...)

QoS-Flow-QoS-Parameter-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-QoS-Parameter-Item

QoS-Flow-QoS-Parameter-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-QoS-Parameter-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-QoS-Parameter-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest	CRITICALITY ignore	EXTENSION QosMonitoringRequest	PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QoS-Flow-Removed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoS-Flow-Released-In-Session			ENUMERATED {released-in-session, not-released-in-session, ...}			OPTIONAL,

qoS-Flow-Accumulated-Session-Time		OCTET STRING (SIZE(5))														OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flow-Removed-Item-ExtIEs } }		OPTIONAL,

...

}

QoS-Flow-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- R

RANUEID ::= OCTET STRING (SIZE (8))

RAT-Type	::=	ENUMERATED	{

e-UTRA,

nR,

...

}

RetainabilityMeasurementsInfo	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Removed-Item

RLC-Mode	::=	ENUMERATED	{

rlc-tm,

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

ROHC-Parameters	::= CHOICE {

rOHC					ROHC,

uPlinkOnlyROHC			UplinkOnlyROHC,

choice-Extension		ProtocolIE-SingleContainer { { ROHC-Parameters-ExtIEs} }

}

ROHC-Parameters-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ROHC	::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { ROHC-ExtIEs } } 		OPTIONAL

}

ROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- S

SecurityAlgorithm	::= SEQUENCE {

cipheringAlgorithm				CipheringAlgorithm,

integrityProtectionAlgorithm	IntegrityProtectionAlgorithm	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SecurityAlgorithm-ExtIEs } }	OPTIONAL,

...

}

SecurityAlgorithm-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			IntegrityProtectionIndication,

confidentialityProtectionIndication		ConfidentialityProtectionIndication,

maximumIPdatarate							MaximumIPdatarate							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SecurityIndication-ExtIEs} }	OPTIONAL,

...

}

SecurityIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityInformation ::= SEQUENCE {

securityAlgorithm			SecurityAlgorithm,

uPSecuritykey				UPSecuritykey,

iE-Extensions				ProtocolExtensionContainer { { SecurityInformation-ExtIEs } }	OPTIONAL,

...

}

SecurityInformation-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult			IntegrityProtectionResult,

confidentialityProtectionResult		ConfidentialityProtectionResult,

iE-Extensions						ProtocolExtensionContainer { {SecurityResult-ExtIEs} }	OPTIONAL,

...

}

SecurityResult-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Slice-Support-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF Slice-Support-Item

Slice-Support-Item ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { Slice-Support-Item-ExtIEs } }	OPTIONAL

}

Slice-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST			OCTET STRING (SIZE(1)),

sD			OCTET STRING (SIZE(3)) 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-ExtIEs } }	OPTIONAL,

...

}

SNSSAI-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SDAP-Configuration ::= SEQUENCE {

defaultDRB				DefaultDRB,

sDAP-Header-UL			SDAP-Header-UL,

sDAP-Header-DL			SDAP-Header-DL,

iE-Extensions			ProtocolExtensionContainer { { SDAP-Configuration-ExtIEs } }	OPTIONAL,

...

}

SDAP-Configuration-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SDAP-Header-DL	::=	ENUMERATED	{

present,

absent,

...

}

SDAP-Header-UL	::=	ENUMERATED	{

present,

absent,

...

}

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

#### -- T

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TraceActivation ::= SEQUENCE {

traceID								TraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TraceID ::= OCTET STRING (SIZE(8))

TransportLayerAddress		::= 	BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

T-Reordering	::=	ENUMERATED	{ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750, ms3000, ...}

T-ReorderingTimer ::= SEQUENCE {

t-Reordering				T-Reordering,

iE-Extensions				ProtocolExtensionContainer { { T-ReorderingTimer-ExtIEs } }	OPTIONAL,

...

}

T-ReorderingTimer-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Addresses-Info-To-Add-List	Transport-UP-Layer-Addresses-Info-To-Add-List	OPTIONAL,

transport-UP-Layer-Addresses-Info-To-Remove-List	Transport-UP-Layer-Addresses-Info-To-Remove-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs} }	OPTIONAL,

...

}

Transport-Layer-Address-Info-ExtIEs	E1AP-PROTOCOL-EXTENSION	::=	{

...

}

Transport-UP-Layer-Addresses-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Add-Item

Transport-UP-Layer-Addresses-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Remove-Item

Transport-UP-Layer-Addresses-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- U

UE-Activity	::=	ENUMERATED	{

active,

not-active,

...

}

UE-associatedLogicalE1-ConnectionItem ::= SEQUENCE {

gNB-CU-CP-UE-E1AP-ID		GNB-CU-CP-UE-E1AP-ID	 OPTIONAL,

gNB-CU-UP-UE-E1AP-ID		GNB-CU-UP-UE-E1AP-ID	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-associatedLogicalE1-ConnectionItemExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalE1-ConnectionItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-Configuration	::=	ENUMERATED	{

no-data,

shared,

only,

...

}

ULDataSplitThreshold	::=	ENUMERATED {b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, ...}

UP-Parameters ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF UP-Parameters-Item

UP-Parameters-Item ::= SEQUENCE {

uP-TNL-Information			UP-TNL-Information,

cell-Group-ID				Cell-Group-ID,

iE-Extensions				ProtocolExtensionContainer { { UP-Parameters-Item-ExtIEs } }	OPTIONAL,

...

}

UP-Parameters-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UPSecuritykey	::= SEQUENCE {

encryptionKey				EncryptionKey,

integrityProtectionKey		IntegrityProtectionKey		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UPSecuritykey-ExtIEs } }	OPTIONAL,

...

}

UPSecuritykey-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-TNL-Information		::= 	CHOICE {

gTPTunnel		GTPTunnel,

choice-extension		ProtocolIE-SingleContainer	{{UP-TNL-Information-ExtIEs}}

}

UP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

UplinkOnlyROHC ::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UplinkOnlyROHC-ExtIEs } } 		OPTIONAL

}

UplinkOnlyROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- V

#### -- W

#### -- X

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

#### -- Common definitions

--

-- **************************************************************

E1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= 	ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global			OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..maxProtocolExtensions)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info								ProtocolIE-ID ::= 86

id-QoSMonitoringRequest										ProtocolIE-ID ::= 87

id-ExtendedSliceSupport											ProtocolIE-ID ::= XX

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

#### -- Container definitions

--

-- **************************************************************

E1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

E1AP-PROTOCOL-IES ::= CLASS {

&id					ProtocolIE-ID			UNIQUE,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

E1AP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

E1AP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { E1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer { E1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { E1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				E1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		E1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			E1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, E1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { E1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { E1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					E1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			E1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		E1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { E1AP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { E1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				E1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality		E1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value			E1AP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

End of Text Proposal to TS 38.463