3GPP TSG-RAN WG3 #108-e	R3-203831

1-11 June 2020

Online

Agenda item:		10.2.1.4

Source:	LG Electronics

Title:	(TP for NR_SON_MDT BL CR for TS 38.473): Remaining issues for CU-DU MRO

Document for:		Other

# 1. Text Proposal for TS 38.473

Beginning of Text Proposal to TS 38.473

8.2.z1	Access and Mobility Indication

8.2.z1.1	General

This procedure is initiated by the gNB-CU to send the access and mobility related information, e.g., RLF Report, to gNB-DU.

The procedure uses non-UE-associated signalling.

8.2.z1.2	Successful Operation

Figure 8.2.z1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from the gNB-CU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

End of Text Proposal to TS 38.473