3GPP TSG-RAN WG3 meeting #108-e	R3-203842

Online, 1st – 11th June, 2020

Source: 	Huawei

Title:	Discussion on IAB inactive

Agenda Item:	13.2.1.1

Document for:	Discussion

- Introduction

In last RAN2 109bis-e meeting, RAN2 has discussed the UE capability of IAB-MT, and has the following agreement [1]:

The following features are optional for IAB-MT:

1. PDCP; 1-5: Short SN

3. MAC; 3-3: DRX

4. Measurements; 4-5: ANR

6. Inactive; 6-1: RRC Inactive

From the above agreements, we can see that the IAB-MT can support RRC inactive as an optional feature. However, since IAB node compose an IAB-MT and an IAB-DU, it is not clear about the IAB-DU’s operation when IAB-MT switch from RRC connected to RRC inactive. In this paper, we will discuss more details about IAB inactive.

- Discussion

When an IAB-MT enters inactive state, there are two issues about the collocated IAB-DU need to be discussed. One is how to deal with the air interface served by IAB-DU, and the other is how to deal with the F1 interface on IAB-DU. Next, we will discuss them separately.

How to deal with the air interface served by IAB-DU?

When IAB-MT enters inactive state, the RRC connection between IAB-MT and donor-CU is released, so it is unable to provide continuous services for the connected UEs under the IAB-DU. In this case, the IAB-DU can be operated in two ways:

- Option 1: Switch off IAB-DU.

If IAB-DU is switched off, that means all the cells of IAB-DU are deactivated by donor-CU. Therefore, there is no any UE can camp on the IAB-DU.

This option is easy to implement, and has minimum impact on the standard. However, when IAB-DU is switched off, there will be coverage holes in the network, in addition, this option is unable to flexibly adjust the switch off/on of IAB-DU according to the number of UEs that initiate random access within the coverage.

- Option 2: IAB-DU enters dormant/DTX mode.

If IAB-DU enters dormant/DTX mode, that is to say, the IAB-DU only sends MIB/SIB1/Paging, as well as the SSB with long cycle. In this case, only idle/inactive UE can camp on the IAB-DU.

This option is better than option 1 since there will not be coverage hole from UE’s perspective, but it is relatively more complex, and may has some impact on RAN1. In addition, because IAB-MT enters inactive state, IAB-node can only transmit UE paging message after the IAB-MT perform RRC connection resume. Therefore, how to perform paging for idle/inactive UEs under IAB-DU should be considered. However, since the time is limited for R16, we tend to discuss these issues in future release.

Proposal 1: When IAB-MT enters inactive state, the operation on the air interface of the collocated IAB-DU needs to be discussed, e.g. switch off or enter dormant/DTX mode, in future release.

How to deal with the F1 interface on IAB-DU?

When the IAB-MT enters RRC inactive state, the IAB-DU may release the F1 interface with donor-CU. And then, it need to re-establish the F1 interface again when the IAB-MT resumes the RRC connection, this is workable but may introduce some signalling overhead and latency since all the F1 interface management configuration and UE context will be erased.

However, considering that the IAB-node is fixed, the IAB-node is very likely to connect the same IAB-donor-CU after RRC connection resume. So, in order to make the IAB-DU be ready to serve UEs as soon as possible after IAB-MT’s RRC connection resume, it will be beneficial if the IAB node can maintain F1 interface for IAB-DU even when the IAB-MT in inactive state, but this issue need further study. Considering the R16 will be finished soon, we can study such enhancement in R17 if necessary.

Observation 1: When IAB-MT enters inactive state, the IAB-DU may release the F1 interface, and the F1 interface need to be setup again after the IAB-MT’s RRC connection resume, but with some signalling overhead and latency.

Observation 2: It will be beneficial for IAB-DU to maintain the F1 interface with IAB-donor-CU, for fast node recovery in the case of IAB-MT’s RRC connection resume.

Proposal 2: In R16, the IAB-DU can release the F1 interface if IAB-MT enters RRC inactive, some enhanced solution is worth to be studied in future release.

- Conclusion

This paper mainly discusses more details about IAB inactive, and we provide the following observations as well as proposals:

Observation 1: When IAB-MT enters inactive state, the IAB-DU may release the F1 interface, and the F1 interface need to be setup again after the IAB-MT’s RRC connection resume, but with some signalling overhead and latency.

Observation 2: It will be beneficial for IAB-DU to maintain the F1 interface with IAB-donor-CU, for fast node recovery in the case of IAB-MT’s RRC connection resume.

Proposal 1: When IAB-MT enters inactive state, the operation on the air interface of the collocated IAB-DU needs to be discussed, e.g. switch off or enter dormant/DTX mode, in future release.

Proposal 2: In R16, the IAB-DU can release the F1 interface if IAB-MT enters RRC inactive, some enhanced solution is worthy to be studied in future release.

# References

- Chairman notes of 3GPP TSG-RAN WG2 #109bis-e meeting.