3GPP TSG-RAN WG3 #108-e							                                              R3-203865

1-11 June 2020

Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0368 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.423: AMF region Information IE in Xn setup Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AMF Region information IE be optional in Xn setup procedure for Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedural description relate to AMF region information IE in Xn setup procedure updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Xn interface can not setup between NG-RAN nodes without NG-Flex function and between en-gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Submission to RAN3#108-e |  |  |  |  |  |  |  |  |

Change Start

8.4.1	Xn Setup

8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.

The procedure uses non UE-associated signalling.

8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

The AMF Region Information IE in the XN SETUP REQUEST message shall contain, if available, a complete list of Global AMF Region IDs to which the NG-RAN node1 belongs. The AMF Region Information IE in the XN SETUP RESPONSE message shall contain , if available,a complete list of Global AMF Region IDs to which the NG-RAN node2 belongs.

The List of Served Cells NR IE and the List of Served Cells E-UTRA IE, if contained in the XN SETUP REQUEST message, shall contain a complete list of cells served by NG-RAN node1. The List of Served Cells NR IE and the List of Served Cells E-UTRA IE, if contained in the XN SETUP RESPONSE message, shall contain a complete list of cells served by NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the XN SETUP REQUEST message the SUL Information IE and the Supported SUL band List IE for each served cell where supplementary uplink is configured.

If Supplementary Uplink is configured at the NG-RAN node2, the candidate NG-RAN node2 shall include in the XN SETUP RESPONSE message the SUL Information IE and the Supported SUL band List IE for each served cell where supplementary uplink is configured.

If the NG-RAN node1 is an ng-eNB, it may include the Protected E-UTRA Resource Indication IE into the XN SETUP REQUEST. If the XN SETUP REQUEST sent by an ng-eNB contains the Protected E-UTRA Resource Indication IE, the receiving gNB should take this into account for cell-level resource coordination with the ng-eNB. The gNB shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same ng-eNB.

The protected resource pattern indicated in the Protected E-UTRA Resource Indication IE is not valid in subframes indicated by the Reserved Subframes IE, as well as in the non-control region of the MBSFN subframes i.e. it is valid only in the control region therein. The size of the control region of MBSFN subframes is indicated in the Protected E-UTRA Resource Indication IE.

In case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the XN SETUP REQUEST message and the XN SETUP REQUEST ACKNOWLEDGE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

End of Change