3GPP TSG-RAN WG3 Meeting #108-e	R3-203898

E-meeting, Jun 1st – Jun 12th, 2020

Agenda item:	8.1

Source:	Qualcomm Incorporated

Title:	(TP for TS 38.423) Support of Inter-RAT handover for Fast MCG Recovery

WID/SID:	 LTE_NR_DC_CA_enh-Core – Release 16

Document for:	Discussion and Decision

# Discussion

This paper discusses the changes in the XnAP specification TS 38.423 required to support inter-RAT handover via SRB3 for the Fast MCG Recovery procedure, as requested in the LS from RAN2 to RAN3 [1].

As in our TP paper [2] which discusses the changes required for X2AP TS 36.423, we have the following two proposals. In the Annex, the text changes to TS 38.423 are indicated.

Proposal 1. In order to support inter-RAT handover via SRB3 for the Fast MCG Recovery procedure, the following change is required to the RRC Transfer message definition:

Indicate in the RRC container that carries the network response RRC message from M-NG-RAN node to S-NG-RAN node that MobilityFromNRCommand message can also be included.

Proposal 2. RAN3 agrees on the changes proposed in the Annex.

# Conclusions

Based on the above discussions, we recommend that RAN3 discuss the following proposals:

Proposal 1. In order to support inter-RAT handover via SRB3 for the Fast MCG Recovery procedure, the following change is required to the RRC Transfer message definition:

Indicate in the RRC container that carries the network response RRC message from M-NG-RAN node to S-NG-RAN node that MobilityFromNRCommand or MobilityFromEUTRACommand message can also be included.

Proposal 2. RAN3 agrees on the changes proposed in the Annex.

# References

[1] R2-2004276, LS to RAN3 on Support of inter-RAT HO for fast MCG recovery.

[2] R3-xxxxxxx, (TP for TS 36.423) Support of Inter-RAT handover for Fast MCG Recovery.

# Annex: TP for TS 38.423

------------------------------ Start of change, Section 9.1.2.20 ---------------------------------------------------------------------

#### 9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message or from the S-NG-RAN-NODE to the M-NG-RAN-NODE to report the DL RRC message delivery status.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Split SRB |  | 0..1 |  |  | YES | reject |
| >RRC Container | O |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10] or TS 36.331 [14] and ciphered with the key of the M-NG-RAN node | – |  |
| >SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type to be used | – |  |
| >Delivery Status | O |  | 9.2.3.45 | DL RRC delivery status of split SRB | – |  |
| UE Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | For NGEN-DC and NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MeasurementReport message or the FailureInformation message. For NE-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the MeasurementReport message. | – |  |
| Fast MCG Recovery via SRB3 from SN to MN |  | 0..1 |  |  | YES | ignore |
| >RRC Container | O |  | OCTET STRING | For NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MCGFailureInformation, message. For NGEN-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the MCGFailureInformation message. | – |  |
| Fast MCG Recovery via SRB3 from MN to SN |  | 0..1 |  |  | YES | ignore |
| >RRC Container | O |  | OCTET STRING | For NR-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the RRCReconfiguration or RRCRelease or MobilityFromNRCommand message. For NGEN-DC, includes the DL-DCCH-Message as defined in subclause 6.2.1 of TS 36.331 [14] containing the RRCConnectionReconfiguration or RRCConnectionRelease or MobilityFromEUTRACommand message. | – |  |

------------------------------ End of change, Section 9.1.2.20 ----------------------------------------------------------------------